3GPP TSG SA WG 1 Meeting #99e 	S1-222217r4

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Source:	Huawei

pCR Title:	Pseudo-CR on Use case of data transfer for IoT devices in remote areas

Draft Spec:	3GPP TR 22.865

Agenda item:	7.8

Document for:	Approval

Contact:	Alice Li (alice.li1@huawei.com)

Abstract: This document proposes a new use case about data transfer to/from remote sites.

1. Introduction

Data transfer at remote sites is a very common requirement. Research institutions can obtain data from remote sites for scientific research. Government agencies can obtain data from remote sites for disaster mitigation and avoidance. Commercial companies can obtain data from remote sites for proper resource allocation. Data transmission at many remote sites is delay-insensitive, and satellite coverage does not always ensure that satellites connect to both the serving link and the feeder link. In the past 30 years, many scholars have devoted themselves to studying the data transmission problem of remote sites, and developed the store and forward mechanisms to solve the problem.

2. Reason for Change

To provide a new use case of data transfer for IoT devices in remote areas.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Animal Tracking: https://www.movebank.org/cms/movebank-content/what-is-animal-tracking

[2]	13 Applications of remote sensing in Disaster management: https://grindgis.com/remote-sensing/13-applications-of-remote-sensing-in-disaster-management

[3]	Havlicek, J. P., Mckeeman, J. C., & Remaklus, P. W. (1995). Networks of low-earth orbit store-and-forward satellites. IEEE Transactions on Aerospace and Electronic Systems, 31(2), 543-554.

[4]  	Antonini, M., De Luise, A., Ruggieri, M., & Teotino, D. (2005). Satellite data collection & forwarding systems. IEEE Aerospace and Electronic Systems Magazine, 20(9), 25-29.

[5]  	Abbasi-Moghadam, D., Hotkani, S. M. H. N., & Abolghasemi, M. (2016). Store and forward communication payload design for LEO satellite systems. Majlesi Journal of Electrical Engineering, 10(3).

* * * Second Change * * * *

## 5.1	Use case of data transfer for IoT devices in remote areas

### 5.1.1	Description

Data transfer at remote sites is a very common requirement. Research institutions can obtain data from remote sites for scientific research, e.g. animal tracking [1]. Government agencies can obtain data from remote sites for disaster mitigation and avoidance, e.g. via remote sensing [2]. Commercial companies can obtain data from remote sites for proper resource allocation. Data transmission at many remote sites is delay-insensitive, and satellite coverage does not always ensure that satellites connect to both the serving link and the feeder link. In the past 30 years, many scholars have devoted themselves to studying the data transmission problem of remote sites, and developed the store and forward mechanisms to solve the problem [3][4][5].

In remote areas, there is no terrestrial network for various reasons, e.g. it is difficult to build and maintain communication towers. As a result this makes it challenging to collect information for environmental protection purposes in these areas. For example, sensors installed on animals need to be monitored regularly. In this scenario, the sensors installed on the animals send the status information, e.g. the movements, physiology and surrounding environment of the animals, to the satellite; and the satellite stores the received status information of the animals, and forwards the information to the scientific centre when a feeder link becomes available.

### 5.1.2	Pre-conditions

There is no terrestrial network in remote area #1 and remote area #2, and the satellite covering the remote areas #1 and #2 does not have a constant active feeder link to the ground segment. EA Science Center has installed sensors (IoT devices) on the animals to collect information for environmental protection purposes in these remote areas. Satelles, the satellite communication operator, has launched the store and forward (S&F) operation as part of Core Network functionality onboard the satellites to support the data transferring for the remote areas. EA Science Center has signed contract with Satelles to allow sensors installed on animals to send the status information (e.g. the movements, physiology and surrounding environment of the animals) to the Science Center via satellite.

The CN onboard satellite and the IoT devices are properly configured with sufficient information, e.g. credential/certificate that is needed for the devices to verify the authenticity of the satellite.

### 5.1.3	Service Flows

Figure 5.1.3-1: Animal tracking in the remote areas

1. The IoT devices are installed on animals and powered on, they are registered with the 5G network for the S&F operation. The CN onboard satellite with the store and forward function enables the IoT devices to transfer data to the network, even when the feeder link to the ground is not available. A secured connection between an IoT device and the CN onboard the satellite is established to protect the data security and privacy.

2. The IoT devices send sensor status information to the satellite, the CN onboard satellite stores the sensor status information received from the IoT devices.

3. When the satellite has the feeder link available to the ground segment, the CN onboard satellite forwards the sensor status information, as well as other necessary information, to the ground core network. The ground core network verifies the IoT devices based on the information received; if it is allowed, the ground core network forwards the sensor status information to its destination data network.

4. The ground core network sends the result of the operation to the CN onboard satellite (the same satellite or a different one that will pass through the remote area).

5. When the satellite (or next satellite) passes through the remote area, the satellite pages the UE, and based on the result received from ground core network, the CN onboard satellite sends result of the operation to the IoT devices.

6. If an IoT device needs to update the sensor status information, it can send it to the CN onboard satellite when it is connected to the satellite. The CN onboard satellite stores it and forwards the sensor status information to the ground core network when feeder link becomes available.

### 5.1.4	Post-conditions

After the scientific centre receives the sensor status information, the scientists can analyse the sensor status information, and track the status of the animals.

### 5.1.5	Existing features partly or fully covering the use case functionality

None.

### 5.1.6	Potential New Requirements needed to support the use case

[PR 5.1.6-1] The 5G system Core Network function onboard the satellite shall support mechanisms to store user data, received from UEs via satellite access, connected via a on the satellite and forward it to the ground 5G core network when feeder link between the satellite and the ground segment is available, and vice versa.

[PR 5.1.6-2] The 5G system shall support mechanisms for a user to securely register one or a group of a UE to use the store and forward operation in the locations where air interface between the satellite to the UE and/or feeder link between the satellite and the ground segment are not constantly available.

NOTE: 	The user can be a third party, which is typically a business customer having service level agreement with the operator and interacting with the 5G network via an application server.

Editor’s Note: the above requirements are FFS.

[PR 5.1.6-3] The 5G system shall support the mechanisms to authenticate and authorize a UE for the store and forward operation.

[PR 5.1.6-4] The 5G system shall be able to limit the total amount of the stored data from a UE when using the store and forward operation.

[PR 5.1.6-5] The 5G system shall be able to collect charging information per UE for use of the store and forward operation (e.g., data volumn, duration, involved satellites).

[PR 5.1.6-6] The 5G system shall be able to collect charging information per application for use of the store and forward operation (e.g., number of UEs, data volume, duration, involved satellites).

* * * End of Change * * * *