3GPP TSG-SA WG1 Meeting #99e 	S1-222246r1

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Title:	FS_DualSteer Use Case on - UE on Vehicle connecting to NTN+TN access networks (PLMN/NPN)

Agenda Item:	7.11

Source:	Lockheed Martin,[Nokia,Huawei,Futurewei]

Contact:	Prasad Chandratre (prasad.chandratre<at>lmco.com)

Abstract: This paper proposes new use cases to be captured in TR 22.841 v.0.0.0.

Note: NTN refers to LTE/NR-based satellite radio access technology, constellations based on any of these orbits (GEO/MEO/LEO), as well as - HAPS

# 5.3 FS_DualSteer Use Case - UE on Vehicle connecting to NTN+TN access networks (PLMN/NPN)

## 5.3.1	Description

In the service areas where multiple 3GPP access networks & services are available, including Non-Public network as one of the access paths, we can utilize the FS_DUALSTEER  feature for extending the benefits of dual access paths to the UE on a vehicle / on a UAV.  In this use case, a 4G/5G dual radio capable UE equipped with 3GPP SIM card connects to two core access networks paths  TN + NTN. There will be a multi-Access PDU session possible using FS_DUALSTEER providing benefits like - network aggregation, fault tolerance and capacity. This is possible by using multi-path 3GPP access and Upper Layer Traffic splitting/steering/switching network feature.

When the vehicle UE is in motion / travelling, it can benefit further by having dual access 4G/5G as 5GC can provide dualsteer functionality service

TN / Terrestrial Network 4G/5G service is provided by the radio tower on the ground (PLMN1or NPN)

NTN radio network service is provided by satellite/HAPS.(PLMN2 or NPN)

Multi-Access path combination for Vehicle/UE: 5GCs of TN+NTN, across PLMN1 or NPN  and PLMN2 or NPN

Figure 5.3.1: Illustration of the Vehicle/UE using NTN+TN (with PLMN/NPN) access paths

* Vehicle/UE is only for the illustration purpose, any generic UE can be used

## 5.3.2	Pre-conditions

- One or both access network(s) is/are NPN (Private 4G/5G)

- NTN 4G/5G service – Satellite constellation service area

- Overlapping 4G/5G TN+NTN radio service coverage area.

- 5G UE with dual radio and has PLMN1/NPN or PLMN2/NPN subscription

- Both PLMN1 and PLMN2 are managed by same service provider  OR MNO agreement is in place

- 4G/5G Core - MNO control/policy, to provide flexible user plane traffic aggregation, steering and switching

- General multi-path traffic routing policy: Secure FTP File Transfers / Video Streaming and Web Browsing can use concurrent data connections, one on each 3GPP access path, and can also be aggregated over both paths

## 5.3.3	Service Flows

  - Scope - Secure FTP file transfers + Web / HTTP service , both are concurrent data connections, one on each 3GPP access path core networks

  - UE registers on TN 4G/5G network – PLMN1/NPN, APN1/DNN1 – A secure connection is setup for file transfers

  - UE Capability confirms to the network that UE has dual radios and FS_DUALSTEER capability

  - Note – UE is also capable of registering on NTN 4G/5G network – PLMN2/NPN

  - The UE begins another data session in addition to previous, e.g web browsing session

  - Based on the traffic characteristics, Network will be able to provide applicable dual access traffic in 5GCs of TN and NTN - splitting both data sessions based on the policies created in 5GC. Both applications will be served by concurrent traffic flows in two 5GCs

  - The UE in motion encounters degradation on TN service (PLMN1/NPN) and camps on NTN (PLMN2/NPN)

  - The 5G System continue to provide dual access on the NTN+TN 5GCs using dualsteer

## 5.3.4	Post-conditions

UE can have two concurrent data session using two 3GPP core networks.

## 5.3.5	Existing features partly or fully covering the use case functionality

See ATSSS,  3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900, including the potential applicability of existing multi-NW requirements (e.g. from TS 22.261 sec. 6.1, 6.3, 6.18, 6.25.2, 6.41). NPN concepts are covered by TS 23.501

Stage-2&3 include a feature called ATSSS (Access Traffic Steering, Switching, Splitting), e.g., ref to TS 25.301 sec. 5.3.2, which supports functionalities similar to those described in this use case, but limited to 3GPP access plus non-3GPP dual access.

Existing service requirements, e.g., from TS 22.261 sec. 6.3, capture some general multi-RAT connectivity requirements, which do not fully cover or satisfy the specific target use case and functionalities

## 5.3.6	Potential New Requirements needed to support the use case

[PR 5.2.x-001] The 5G system shall be able to support the Core network of PLMN / NPN and the UE functionality to provide two 3GPP networks access paths (NTN and TN), based on MNO traffic routing policies.

[PR 5.2.x-001] The 5G System shall be able to handle any degradation/interruption on one of the radio access paths by continuing to steering/split/switch traffic irrespective of which RAN is active (TN or NTN)