3GPP TSG-SA WG1 Meeting #99e 	S1-222246r2

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Title:	FS_DualSteer Use Case on - UE on Vehicle connecting to NTN+TN access networks (PLMN/NPN)

Agenda Item:	7.11

Source:	Lockheed Martin,[Nokia,Huawei,Futurewei, Qualcomm]

Contact:	Prasad Chandratre (prasad.chandratre<at>lmco.com)

Abstract: This paper proposes new use case to be captured in TR 22.841 v.0.0.0.

Note: NTN refers to satellite radio access technology, constellations based on any of these orbits (GEO/MEO/LEO), as well as - HAPS

# 5.3 FS_DualSteer Use Case - UE on Vehicle connecting to NTN+TN access networks (PLMN/NPN)

## 5.3.1	Description

In the geographic areas where multiple 3GPP access networks & services are available, including Non-Public network as one of the access paths, we can utilize the DUALSTEER  feature for extending the benefits of dual access paths to the UE on a vehicle.  In this use case, a dual radio capable UE equipped with 3GPP SIM card connects to5G System / network and can get served via two data access paths – one for TN  and another for the NTN . There will be a multi-Access PDU session possible using DUALSTEER providing benefits like - network aggregation, fault tolerance and capacity. This is possible by using multi-path 3GPP access and Upper Layer (above Radio layers)Traffic splitting/steering/switching network feature.

As the vehicle UE is in motion / travelling, it can benefit further by having at least one of the radio access path active, most of the time (either TN or NTN radio coverage), the data call should continue– as 5GC can provide dualsteer functionality.

3GPP - PNI-NPN provides private 5G access and has PLMN integrated, which can be used in certain geographic areas along with standard PLMN based network to achieve the benefits of dualsteer

Figure 5.3.1: Illustration of the Vehicle/UE using NTN+TN (with PLMN/NPN) access paths

* Vehicle/UE is only for the illustration purpose, any generic UE can be used

TN / Terrestrial Network service is provided by the radio tower on the ground (it can be either PLMN1or NPN)

NTN radio network service is provided by satellite/HAPS.( either PLMN2 or NPN)

The 5GC is common across both the TN and NTN networks

Multi-Access path combinations for Vehicle/UE:  as below:

| TN | NTN |
| --- | --- |
| PLMN1 | NPN |
| NPN | PLMN |

## 5.3.2	Pre-conditions

- One of the access networks is NPN, type is – PNI-NPN (not SNPN)

- Overlapping TN+NTN radio service coverage area.

- 5G UE with dual radio, NPN and dual access subscription

- Relationship of the TN and NTN networks :  Scenario 1. both managed by same service provider  OR  Scenario 2. Multiple service providers with shared (MOCN) PLMN arrangement. If needed a roaming agreement can be considered

- Dualsteer specific multi-path traffic routing policy: based on traffic characteristics, which setup access path on each TN and NTN.  e.g. for Secure FTP File Transfers TN, Video Streaming and Web Browsing - NTN.

## 5.3.3	Service Flows

  - Scope – Multiple data applications requested by UE - Secure FTP File Transfers , Video Streaming and Web Browsing  - concurrent data connections, , steered by 5G System

Note: two or more data application on the UE are chosen for illustrating the triggering of “dualsteer” policy in the 5GC. The “dualsteer” policy can also be based on certain data volume based traffic flow template (even for a single data application).

  - UE registers on TN network – which is one of the combination as listed in the table in section 5.3.1– A secure data connection is setup for file transfers

  - Note – UE is also capable of registering on NTN 4G/5G network

  - UE Capability confirms to the network that UE has dual radios and DUALSTEER subscription

  - The UE begins another data session in addition to previous, e.g web browsing session

  - Based on the traffic characteristics, Network will be able to provide applicable dual access traffic paths in 5GC- splitting both data sessions based on the user plane policies created in 5GC. Both applications will be served by concurrent traffic flows in 5GC

  - During the active data call, if the UE in motion encounters degradation on either RAN network, it will be able to continue data application use on another radio connection

  - Once the radio conditions improve again, the 5G System continue to provide dual access path data call

## 5.3.4	Post-conditions

UE in motion gets reliable data connection and a better service as dualsteer provides two access paths using combination of PLMN and NPN.

## 5.3.5	Existing features partly or fully covering the use case functionality

See ATSSS,  3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900, including the potential applicability of existing multi-NW requirements (e.g. from TS 22.261 sec. 6.1, 6.3, 6.18, 6.25.2, 6.41). NPN concepts are covered by TS 23.501

Stage-2&3 include a feature called ATSSS (Access Traffic Steering, Switching, Splitting), e.g., ref to TS 25.301 sec. 5.3.2, which supports functionalities similar to those described in this use case, but limited to 3GPP access plus non-3GPP dual access.

Existing service requirements, e.g., from TS 22.261 sec. 6.3, capture some general multi-RAT connectivity requirements, which do not fully cover or satisfy the specific target use case and functionalities

## 5.3.6	Potential New Requirements needed to support the use case

[PR 5.3.x-001] For certain data/traffic conditions, the 5G system shall be able to support the Core network of PLMN and/ or NPN and the UE with two 3GPP networks access paths (NTN and TN), based on MNO traffic routing policies.

[PR 5.3.x-002] The 5G System shall be able to handle any degradation/interruption on one of the radio access paths by continuing to steering/split/switch traffic using both TN or NTN