3GPP TSG-SA WG1 Meeting #99e 	S1-222247r1

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

# Title:	FS_DUAL_STEER use case on - Vehicle/UE connecting to NTN+NTN access networks

Agenda Item:	7.11

Source:	Lockheed Martin,[Nokia,Huawei,Futurewei]

Contact:	Prasad Chandratre (prasad.chandratre<at>lmco.com)

Abstract: This paper proposes new use cases to be captured in TR 22.841 v.0.0.0.

Note: NTN refers to LTE/NR-based satellite radio access technology, constellations based on any of these orbits (GEO/MEO/LEO), as well as - HAPS

# 5.2 Use Case - Vehicle/UE connecting to NTN+NTN access networks

## 5.2.1	Description

In the service areas where multiple 3GPP access networks & services are available, we can utilize the FS_DUAL_STEER  feature for extending the benefits of dual access paths to the UE on a vehicle / on a UAV.  In this use case, a 4G/5G dual radio capable UE equipped with 3GPP SIM card connects to two access networks paths - TN + NTN. There will be a multi-Access PDU session possible using FS_DUAL_STEER providing benefits like - network aggregation, fault tolerance and capacity. This is possible by using multi-path 3GPP access and Upper Layer Traffic splitting/steering/switching network feature.

When the vehicle/ UAV UE is in motion / travelling, it can benefit further by having dual access 4G/5G service 4G/5G service and also consistently have at least one access path active, most of the time, when a data call is active – as 5GC can provide dualsteer functionality.

NTN radio network service is provided by LEO satellite constellation.(PLMN1)

NTN radio network service is provided by GEO satellite constellation.(PLMN2)

NTN radio network service is provided by HAPS.(PLMN3)

Multi-Access path combination for Vehicle/UE: NTN+NTN, across either combination of any two PLMNs

Figure 5.2.1: Illustration of the UE on a Vehicle using NTN+NTN access paths

* Vehicle/UE is only for the illustration purpose, any generic UE can be used

## 5.2.2	Pre-conditions

- Overlapping PLMN1-NTN and PLMN2-NTN coverage area.

- 5G UE with NTN) capability

- PLMN1, PLMN2 and PLMN3 are managed by same service provider or have MNO agreement

- MNO control/policy provides flexible user plane traffic aggregation, steering and switching

- General multi-path traffic routing policy: Secure FTP File Transfers / Video Streaming and Web Browsing can use concurrent data connections, one on each 3GPP access path, and can also be aggregated over both paths.

## 5.2.3	Service Flows

  - Scope - Secure FTP file transfers + Web Browsing , both concurrent data sessions, one on each 3GPP access path, steered by 5G System

  - UE registers on PLMN1 NTN network. A secure connection is setup for file transfers (APN1/DNN1)

  - UE confirms that UE is capable of dual 3GPP access connectivity

  - Note – UE is also capable of registering on NTN 4G/5G network – PLMN2

  - UE opens another data session , a web browsing session

  - Based on the traffic characteristics, Network will be able to provide applicable dual access traffic in 5GCs of TN and NTN - splitting both data sessions based on the policies created in 5GC. Both applications will be served by concurrent traffic flows in two 5GCs

  - The UE in motion encounters degradation on NTN service (PLMN1) and camps on NTN (PLMN2)

  - The 5G System continue to provide dual access on the NTN+TN 5GCs using dualsteer

## 5.2.4	Post-conditions

UE can have two concurrent data session using two 3GPP core networks.

## 5.2.5	Existing features partly or fully covering the use case functionality

See ATSSS,  3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900 , including the potential applicability of existing multi-NW requirements (e.g. from TS 22.261 sec. 6.1, 6.3, 6.18, 6.41).

Stage-2&3 include a feature called ATSSS (Access Traffic Steering, Switching, Splitting), e.g., ref to TS 25.301 sec. 5.3.2, which supports functionalities similar to those described in this use case, but limited to 3GPP access plus non-3GPP dual access.

Existing service requirements, e.g., from TS 22.261 sec. 6.3, capture some general multi-RAT connectivity requirements, which do not fully cover or satisfy the specific target use case and functionalities

## 5.2.6	Potential New Requirements needed to support the use case

Extra Territorial requirements due to multiple NTN service should be accounted for.

[PR 5.2.x-001] The 5G system shall be able to support the Core network and the UE functionality to provide two 3GPP networks access paths (NTN and TN), based on MNO traffic routing policies.

[PR 5.2.x-002] The 5G System shall be able to handle any degradation/interruption on one of the radio access paths by continuing to steering/split/switch traffic irrespective of which RAN is active (TN or NTN)

[PR 5.2.x-003]  Extra Territorial requirements due to multiple NTN service should be accounted for dualsteer.