3GPP TSG-SA WG1 Meeting #99e 	S1-222247r2

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

# Title:	FS_DUAL_STEER use case on - Vehicle/UE connecting to NTN+NTN access networks

Agenda Item:	7.11

Source:	Lockheed Martin,[Nokia,Huawei,Futurewei,Qualcomm]

Contact:	Prasad Chandratre (prasad.chandratre<at>lmco.com)

Abstract: This paper proposes new use case to be captured in TR 22.841 v.0.0.0.

Note: NTN refers to satellite radio access technology, constellations based on any of these orbits (GEO/MEO/LEO), as well as - HAPS

# 5.2 Use Case - Vehicle/UE connecting to NTN+NTN access networks

## 5.2.1	Description

In the geographic areas where multiple NTN 3GPP access networks & services are available, we can utilize the DUALSTEER  feature and extend the benefits of dual access paths to the UE on a vehicle.

In this use case, a  dual radio capable UE equipped with 3GPP SIM card connects to 5G System /network and can get served by network with two data access paths – one on eachNTN. There will be a multi-Access PDU session possible using DUALSTEER providing benefits like - network aggregation, fault tolerance and capacity. This is possible by using multi-path 3GPP access and Upper Layer (above Radio layers) Traffic splitting/steering/switching network capability.

Since the vehicle UE is in motion / travelling, it can benefit further by having at least one of the radio access path (either one of the NTN) active most of the time, when a data call remains active – as 5GC can provide dualsteer functionality when possible, again

Figure 5.2.1: Illustration of the UE on a Vehicle using NTN+NTN access paths

* Vehicle/UE is only for the illustration purpose, any generic UE can be used

NTN radio network service provided by LEO satellite constellation.(PLMN1)

NTN radio network service provided by GEO satellite constellation.(PLMN2)

NTN radio network service provided by HAPS.(PLMN3)

Multi-Access path combination for Vehicle/UE: NTN+NTN, across either combination of any two PLMNs

## 5.2.2	Pre-conditions

- Overlapping PLMN1-NTN and PLMN2-NTN coverage area.

- 5G UE with  dual radio and dual access subscription

- Relationship of the NTN networks – Scenario 1.

- PLMN1, PLMN2 and PLMN3 are managed by same service provider or Scenario 2. Multiple service providers with MNO agreement. If needed can use roaming agreement.

- Dualsteer specific multi-path traffic routing policy: Secure FTP File Transfers / Video Streaming and Web Browsing can use concurrent data connections, one for each NTN 3GPP access path.

## 5.2.3	Service Flows

  - Scope - Multiple data applications requested by UE - Secure FTP File Transfers , Video Streaming and Web Browsing  - concurrent data connections, , steered by 5G System

Note: two or more data application on the UE are chosen for illustrating the triggering of “dualsteer” policy in the 5GC. The “dualsteer” policy can also be based on certain data volume based traffic flow template (even for a single data application).

  - UE registers on NTN with PLMN1. A secure data connection is setup for file transfers

  - UE capability information confirms that UE is capable of dual 3GPP access connectivity

  - UE begins another data session, e.g. a web browsing session

  - Based on the traffic characteristics, Network will be able to setup and provide applicable dual access traffic in 5GC - splitting both data sessions based on the policies created in 5GC. Both applications will be served by concurrent traffic flows in two 5GC

  - During the active data call, if the UE in motion encounters degradation on either RAN network, it will be able to continue data application use on another radio connection

  - Once the radio conditions improve again, the 5G System continue to provide dual access path data call

## 5.2.4	Post-conditions

UE in motion gets reliable data connection and a better service as dualsteer provides two access paths

## 5.2.5	Existing features partly or fully covering the use case functionality

See ATSSS,  3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900 , including the potential applicability of existing multi-NW requirements (e.g. from TS 22.261 sec. 6.1, 6.3, 6.18, 6.41).

Stage-2&3 include a feature called ATSSS (Access Traffic Steering, Switching, Splitting), e.g., ref to TS 25.301 sec. 5.3.2, which supports functionalities similar to those described in this use case, but limited to 3GPP access plus non-3GPP dual access.

Existing service requirements, e.g., from TS 22.261 sec. 6.3, capture some general multi-RAT connectivity requirements, which do not fully cover or satisfy the specific target use case and functionalities

## 5.2.6	Potential New Requirements needed to support the use case

Extra Territorial requirements due to multiple NTN service should be accounted for.

[PR 5.2.x-001] For certain data / traffic condition, the 5G system shall be able to support the 5GC and UE with two 3GPP networks access paths based on MNO traffic routing policies

[PR 5.2.x-002] The 5G System shall be able to handle any degradation/interruption on one of the radio access paths by continuing to steer/split/switch traffic using both NTN networks

[PR 5.2.x-003]  Extra Territorial requirements due to multiple NTN service should be accounted for dualsteer.