3GPP TSG-SA WG1 Meeting #99e 	S1-222247r4

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

# Title:	FS_DUAL_STEER use case on - Vehicle/UE connecting to NTN+NTN access networks

Agenda Item:	7.11

Source:	Lockheed Martin,[Nokia,Futurewei,Qualcomm, Charter Communications]

Contact:	Prasad Chandratre (prasad.chandratre<at>lmco.com)

Abstract: This paper proposes new use case to be captured in TR 22.841 v.0.0.0.

Note: NTN refers to satellite radio access technology, constellations based on any of these orbits (GEO/MEO/LEO), as well as - HAPS

# 5.2 Use Case - Vehicle/UE connecting to NTN+NTN access networks

## 5.2.1	Description

In the geographic areas where multiple NTN 3GPP access networks & services are available, we can utilize the DUALSTEER  feature and extend the benefits of dual access paths to the UE on a vehicle.

In this use case, a  dual radio capable UE equipped with 3GPP SIM card connects to 5G System /network and can get served by network with two data access paths – one for each NTN. There will be a multi-Access PDU session possible using DUALSTEER providing benefits like - network aggregation, fault tolerance and capacity. This is possible by using multi-path 3GPP access and Upper Layer (above Radio layers) Traffic splitting/steering/switching network capability.

Since the vehicle UE is in motion / travelling, it can benefit further by having at least one of the radio access path (either one of the NTN) active most of the time, when a data call remains active – as 5GC can provide dualsteer functionality when possible, again

Figure 5.2.1: Illustration of the UE on a Vehicle using NTN+NTN access paths

* Vehicle/UE is only for the illustration purpose, any generic UE can be used

NTN radio network service provided by LEO satellite constellation.(PLMN1)

NTN radio network service provided by GEO satellite constellation.(PLMN3)

NTN radio network service provided by HAPS.(PLMN1)

Multi-Access path combination for Vehicle/UE: NTN+NTN, across either combination of any two PLMNs

## 5.2.2	Pre-conditions

- Overlapping PLMN1-NTN and PLMN2-NTN coverage area.

- 5G UE with  dual radio and dualsteer feature subscription with PLMN1 network operator

- Relationship of the NTN networks – Scenario 1.

- PLMN1, PLMN2 and PLMN3 are managed by same Network Operator or Scenario 2. Multiple Network Operartors with an agreement to provide dualsteer service. If needed can use roaming agreement.

- Dualsteer specific multi-path traffic routing policy: Secure FTP File Transfers / Video Streaming and Web Browsing can use concurrent data connections, one for each NTN 3GPP access path.

## 5.2.3	Service Flows

  - Scope - Multiple data applications requested by UE - Secure FTP File Transfers , Video Streaming and Web Browsing  - concurrent data connections, steered by 5G System

Note: two or more data applications on the UE are chosen for illustrating the triggering of “dualsteer” policy in the 5GC. The “dualsteer” policy can be triggered for a single UE application, to allow use of dual data access paths. . The “dualsteer” is intended  for a single data application

  - UE registers on NTN with PLMN1. A secure data connection is setup for large file transfer

  - UE capability information confirms that UE is capable of dualsteer feature

  - UE begins another data session, e.g. a web browsing session

  - Based on the traffic characteristics, Network will be able to setup and provide applicable dual access traffic – by splitting large file transfer based on the policies, using two access paths across PLMN1 and PLMN2 (or PLMN3) created in 5GC. Second application of the UE continues to be served on PLMN1.

  - During the active data call, if the UE in motion encounters degradation on PLMN2 network, 5G System will steer the data access path to PLMN1 to continue data applications without interruption

  - Once the radio conditions improve again for PLMN2, the 5G System continue to provide dual access path data call again

## 5.2.4	Post-conditions

UE in motion gets reliable data connection and a better service as dualsteer provides two access paths

## 5.2.5	Existing features partly or fully covering the use case functionality

See ATSSS,  3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900 , including the potential applicability of existing multi-NW requirements (e.g. from TS 22.261 sec. 6.1, 6.3, 6.18, 6.41).

Stage-2&3 include a feature called ATSSS (Access Traffic Steering, Switching, Splitting), e.g., ref to TS 25.301 sec. 5.3.2, which supports functionalities similar to those described in this use case, but limited to 3GPP access plus non-3GPP dual access.

Existing service requirements, e.g., from TS 22.261 sec. 6.3, capture some general multi-RAT connectivity requirements, which do not fully cover or satisfy the specific target use case and functionalities

## 5.2.6	Potential New Requirements needed to support the use case

Extra Territorial requirements due to multiple NTN service should be accounted for.

[PR 5.2.x-001] For specific data traffic needs, the 5G system shall be able to support two 3GPP networks access paths based on Network Operator’s traffic routing policies for a UE with single Network Operator subscription

[PR 5.2.x-002] In case of any degradation/interruption on one of the radio access paths the 5G System shall be able to minimize impact by continuing to steer/split/switch traffic using both NTN networks as per Network Operator’s policies2