3GPP TSG SA WG 1 Meeting #99e 	S1-222254r2

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Source:	Hansung University, LGUplus, KT, ETRI

pCR Title:	FS_RAILSS consolidated requirements

Draft Spec:	3GPP TR 22.890 v 0.6.2

Agenda item:	7.1

Document for:	Approval

Contact:	Andrew Min-gyu Han, andyhan@hansung.ac.kr

Abstract: This contribution provides contents for the potential consolidated requirements for RAILSS.

1. Introduction

This pCR provides contents for the potential consolidated requirements for RAILSS.

2. Reason for Change

The potential consolidated requirements are important contents for RAILSS.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.890.

* * * First Change * * * *

X	Potential Consolidated Requirements

X.1 	Introduction

The requirements below refer to a “Railway Smart Station Services”, which is acting as an application a FRMCS and the outer systems.

The potential consolidated requirements are mainly focusing the 5G Network characteristics and the interfaces between FRMCS/MCX Functions and Railway Smart Station Services as the application of FRMCS and outer system of 3GPP.

Figure 10.1-1 Scope of the Potential Consolidated Requirements

X.2	Connectivity

Table 10.2-1 – Connectivity Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR X.2-1 | The FRMCS System shall support the following traffic characteristics of data transfer: Note: This table is intended to be included in Section 6.2 of TS 22.289. | PR 5.2.6-2 | Table 5.2.6-1, intended to be included in Section 6.2 of TS 22.289[9] |
| CPR X.2-2 | The 5G System shall support the proximity connections between the UE, the kiosk and the Mobile Intelligent Assistant. | PR 6.2.6-1 | intended to be included in Sections 6.15, 6.17, 7.12 of TS 22.280[7] |

X.3	Other aspects

Table 10.3-1 – Other Aspects Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR X.3-1 | The 5G System shall be able to provide functions to handle the smart station information for making display 3D or metaverse enabled smart station map and generating path information by a kiosk. | PR 6.2.6-2 | intended to be included in Sections 4.8 and 6.1 of TS 22.281[10] |

* * * 2nd Change * * * *

2	References

The following documents contain provisions, which through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	UIC MG-7900v2.0.0, Future Railway Mobile Communication System – Use cases, Feb. 2020.

[3]	UIC FU-7100v5.0.0, Future Railway Mobile Communication System – User Requirements Specification, Feb. 2020.

[4]		TTA TTAK.KO-06.0507/R1, Requirements for Smart Railway Device - Information Model, Dec. 2020.

[5]	TTA TTAK.KO-06.0508/R1, Requirements for Smart Railway Platform - Information Model, Dec. 2020.

[6]	3GPP TR 22.990. “Study on off-network for rail”.

[7]                        3GPP TS 22.280: " Mission Critical Services Common Requirements (MCCoRe)"

[8]	3GPP TS 22.282: “Mission Critical (MC) data”.

[9]	3GPP TS 22.289: “Mobile communication system for railways”.

[10]	3GPP TS 22.281: “Mission Critical (MC) Video”.

* * * End of Change * * * *