3GPP TSG SA WG 1 Meeting #99e 	S1-222254r7

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Source:	Hansung University, LGUplus, KT, ETRI

pCR Title:	FS_RAILSS consolidated requirements

Draft Spec:	3GPP TR 22.890 v 0.6.2

Agenda item:	7.1

Document for:	Approval

Contact:	Andrew Min-gyu Han, andyhan@hansung.ac.kr

Abstract: This contribution provides contents for the potential consolidated requirements for RAILSS.

1. Introduction

This pCR provides contents for the potential consolidated requirements for RAILSS.

2. Reason for Change

The potential consolidated requirements are important contents for RAILSS.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.890.

* * * First Change * * * *

# 7	Potential Consolidated Requirements

## 7.1 Introduction

The requirements below refer to a "Railway Smart Station Services", which is acting as an application a FRMCS and the outer systems.

The potential consolidated requirements are mainly focusing the 5G Network characteristics and the interfaces between FRMCS/MCX Functions and Railway Smart Station Services as the application of FRMCS and outer system of 3GPP.

Figure 7.1-1 Scope of the Potential Consolidated Requirements

## 7.2	Functional aspects

Table 7.2-1 Functional Aspects Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.2-1 | The MCX service shall support obtaining and conveying MCX UE location information describing the positions of each MCX UE with different location accuracy simultaneously. | PR 6.3.6-1 | intended to be included in clauses 5.11 and 6.12 of TS 22.280 |
| CPR 7.2-2 | The FRMCS shall be able to provide a mechanism to trigger an emergency alert based on a combination of UE location (e.g. the location of the specific platform door / train door) and application-generated trigger (e.g. train door did not close properly due to a blockage) | PR-8.x.6-1 | intended to be included in a new clause of TS 22.280 |

## 7.3 Performance

Table 7.3-1: KPIs for Railway Smart Station Services

| Scenario (Note 5) | End-to-end latency | Reliability (Note 1) | UE speed | UE Relative Speed | User experienced data rate | Payload size (Note 2) | Area traffic density | Overall UE density | Service area dimension (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multiple trains’ stops at the same platform (Korea, urban railway) | ≤10 ms | 99.9999% | ≤100 km/h | ≤50km/h | ≤1Mb/s | Small to large | ≤ 1 Mb/s/km | ≤ 5 (100m) | ≤ 15 km along rail tracks including bad weather conditions (Note 4) |
|  | NOTE 1: Reliability as defined in TS 22.289 sub-clause 3.1. NOTE 2: Small: payload ≤ 256 octets, Medium: payload ≤512 octets; Large: payload 513 -1500 octets. NOTE 3: Estimates of maximum dimensions. NOTE 4: Non-Line-of-Sight (NLOS) between UEs shall be supported NOTE 5: Off-network traffic characteristics are not addressed in this table since it can be covered by TR22.990. |  |  |  |  |  |  |  |  |

Note: This table is intended to be included in clause 6.2 of TS 22.289 [9].

* * * End of Change * * * *