| 3GPP TR 22.865 V0.1.0 (2022-08) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on satellite access - Phase 3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	8

5	Use cases	9

5.1	Use case on store and forward - MO	9

5.1.1	Description	9

5.2.2	Pre-conditions	9

5.1.3	Service Flows	9

5.1.4	Post-conditions	9

5.1.5	Existing features partly or fully covering the use case functionality	9

5.1.6	Potential New Requirements needed to support the use case	10

5.2	Use case on store and forward - MT	10

5.2.1	Description	10

5.2.2	Pre-conditions	10

5.2.3	Service Flows	10

5.2.4	Post-conditions	11

5.2.5	Existing features partly or fully covering the use case functionality	11

5.2.6	Potential New Requirements needed to support the use case	11

5.3	Use case on Store and forward operation with discontinuous feeder link for delay-tolerant IoT - Inter-satellite	11

5.3.1	Description	11

5.3.2	Pre-conditions	13

5.3.3	Service Flows	13

5.3.4	Post-conditions	13

5.3.5	Existing features partly or fully covering the use case functionality	13

5.3.6	Potential New Requirements needed to support the use case	13

5.4	Use case on data transfer for IoT devices in remote areas	14

5.4.1	Description	14

5.4.2	Pre-conditions	14

5.4.3	Service Flows	14

5.4.4	Post-conditions	15

5.4.5	Existing features partly or fully covering the use case functionality	15

5.4.6	Potential New Requirements needed to support the use case	15

5.5 	Use case on Temporary LAN using Satellite Access	15

5.5.1	Description	15

5.5.2	Pre-conditions	16

5.5.3	Service Flows	16

5.5.4	Post-conditions	16

5.5.5	Existing features partly or fully covering use case functionality	17

5.5.6	Potential New Requirements needed to support the use case	17

5.6	Use case on Information Exchange between Ships at sea	17

5.6.1	Description	17

5.6.2	Pre-conditions	17

5.6.3	Service Flows	18

5.6.4	Post-conditions	18

5.6.5	Existing features partly or fully covering the use case functionality	18

5.6.6	Potential New Requirements needed to support the use case	18

5.7	Use case on UAVs using satellite access	19

5.7.1	Description	19

5.7.2	Pre-conditions	19

5.7.3	Service Flows	19

5.7.4	Post-conditions	20

5.7.5	Existing features partly or fully covering use case functionality	20

5.7.6	Potential New Requirements needed to support the use case	21

5.8	Use case on Enhanced Positioning Service using Satellite Access	21

5.8.1	Description	21

5.8.2	Pre-conditions	21

5.8.3	Service Flows	21

5.8.4	Post-conditions	21

5.8.5	Existing features partly or fully covering use case functionality	22

5.8.6	Potential New Requirements needed to support the use case	22

6	Additional considerations	22

7	Consolidated requirements	22

8	Conclusions and recommendations	22

Annex A (informative): Store and forward satellite operation	23

Annex B (informative): Change history	25

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes use cases and aspects related to enhancements of the 5G system over satellite, including:

- Operation with intermittent/temporary satellite connectivity for delay-tolerant communication service

- GNSS independent operation

- Communication between UEs under the same satellite’s coverage

Potential service and regulatory requirements are derived for these use cases and are consolidated in a dedicated chapter.

In addition, this document also includes a gap analysis between existing positioning requirements for terrestrial access and the requirements that can be satisfied via satellite access, and determine potential updates to satellite positioning KPIs.

The report ends with recommendations regarding the continuation of the work.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system ".

[3]	3GPP TR 22.822: "Study on using Satellite Access in 5G".

[4]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks".

[5]	Animal Tracking: https://www.movebank.org/cms/movebank-content/what-is-animal-tracking

[6]	13 Applications of remote sensing in Disaster management: https://grindgis.com/remote-sensing/13-applications-of-remote-sensing-in-disaster-management

[7]	Havlicek, J. P., Mckeeman, J. C., & Remaklus, P. W. (1995). Networks of low-earth orbit store-and-forward satellites. IEEE Transactions on Aerospace and Electronic Systems, 31(2), 543-554.

[8]	Antonini, M., De Luise, A., Ruggieri, M., & Teotino, D. (2005). Satellite data collection & forwarding systems. IEEE Aerospace and Electronic Systems Magazine, 20(9), 25-29.

[9]	Abbasi-Moghadam, D., Hotkani, S. M. H. N., & Abolghasemi, M. (2016). Store and forward communication payload design for LEO satellite systems. Majlesi Journal of Electrical Engineering, 10(3).

[10]	Mohit, K. (2021, September 13). 6 Benefits of Information Exchange in the Maritime Industry. Marine Insight. Retrieved Jun 10, 2022, from https://www.marineinsight.com/marine-safety/6-benefits-of-information-exchange-in-the-maritime-industry/.

[11]	Akdağ, M., Solnør, P., & Johansen, T. A. (2022). Collaborative collision avoidance for Maritime Autonomous Surface Ships: A review. Ocean Engineering, 250, 110920.

[12]	3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

[13]	Gure M , Ozel M E , Yildirim H H , et al. Use of satellite images for forest fires in area determination and monitoring. IEEE, 2009.

[14] 	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP".

[15]	Hamza Benzerrouk , “Iridium Next LEO Satellites as an Alternative PNT in GNSS Denied Environments”, June 17, 2019 (https://insidegnss.com)

[16]	“The human cost of disasters: an overview of the last 20 years (2000-2019)”，https://www.undrr.org/publication/human-cost-disasters-overview-last-20-years-2000-2019

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Store & Forward Satellite operation: in the context of this study, it is an operation mode of a 5G system offering satellite-access where the 5G system can provide some level of service (in storing and forwarding the data) when satellite connectivity is intermittently/temporarily unavailable, e.g. to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

S&F	Store and Forward

# 4	Overview

# 5	Use cases

## 5.1	Use case on store and forward - MO

### 5.1.1	Description

This use case illustrates the realization of a S&F service between a UE with satellite access and an Application Server for a delay-tolerant/non-real-time IoT NTN service in the case of a Mobile Originated message.

A description of store and forward operation is provided in Annex A.

Company TrackingInc is offering a service of remote monitoring of fields and is deploying and tracking many battery-powered IoT type UEs across the globe. All the IoT remote monitoring UEs deployed are including a 5G communication with satellite access. Some of them are deployed in remote area where there is no coverage by MNO and only satellite is possible.

For the satellite access, TrackingInc is using the service of IoTSAT for the 5G IoT connectivity by satellite and IoTSAT is using a LEO constellation which supports S&F operation mode.

All IoT remote monitoring UEs are regularly sending information related to the area they are monitoring to the application server of TrackingInc and some times receive new parameters from the application server. In most of the cases, the messages exchanged are delay-tolerant/non-real-time IoT.

### 5.2.2	Pre-conditions

In the present use case, the IoT remote monitoring UE is in a remote area with no ground stations available for feeder link connectivity and the IoT remote monitoring UE is aware that IoTSAT constellation operates in S&F mode.

### 5.1.3	Service Flows

The IoT remote monitoring UE needs to send a message to the TrackingInc application server. The UE waits for satellite network coverage and sends its message when the satellite is passing by.

The IoT remote monitoring UE and the satellite providing coverage interact over the service link, allowing the UE to transfer the message to the satellite, which has not connectivity to the ground segment and, consequently, has to store locally the received message.

At this point:

- Limitations to the size/amount of data that can be sent from the UE could be enforced.

- Delivery priority for the exchanged data could be established.

- Acknowledgement of the received data by the satellite could be issued.

At a later time, the satellite with the stored message establishes connectivity with the ground network via a feeder link and relays/forwards/downloads the message to the ground network.

The ground network, based on established connectivity configuration and routing, delivers message to the TrackingInc application server.

### 5.1.4	Post-conditions

The message generated by the IoT remote monitoring UE has been delivered successfully to the TrackingInc application server without relying on a continuous end-to-end network connectivity path between them.

### 5.1.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2], clause 6.3.2.3 on satellite access includes the following requirements:

The 5G system shall be able to provide services using satellite access.

The 5G system with satellite access shall be able to support low power MIoT type of communications.

However, it is not sufficient in regards of S&F operation especially for the delivery of delay-tolerant/non-real-time IoT NTN services with NGSO satellites.

### 5.1.6	Potential New Requirements needed to support the use case

[PR 5.1.6-001] The 5G system with satellite access shall be able to support store and forward operation.

[PR 5.1.6-002] The 5G system with satellite access shall be able to inform a UE that store and forward operation is applied.

[PR 5.1.6-003] The 5G system with satellite access supporting store and forward operation shall be able to provide a validity period for the data to be stored.

[PR 5.1.6-004] The 5G system with satellite access supporting store and forward operation shall be able to inform a UE when the maximum system capacity of storage for the UE is reached.

[PR 5.1.6-005] The 5G system with satellite access supporting store and forward operation shall support a mechanism to configure and provision specific required QoS and policies for S&F operation (e.g. delivery priority, acknowledgment policy, maximum allowed transfer delay, storage limits…).

## 5.2	Use case on store and forward - MT

### 5.2.1	Description

This use case illustrates the realization of a S&F service between a UE with satellite access and an Application Server for a delay-tolerant/non-real-time IoT NTN service in the case of a Mobile Terminated message.

A description of store and forward operation is provided in Annex A.

Company TrackingInc is offering a service of remote monitoring of fields and is deploying and tracking many battery-powered IoT type UEs across the globe. All the IoT remote monitoring UEs deployed are including a 5G communication with satellite access. Some of them are deployed in remote area where there is no coverage by MNO and only satellite is possible.

For the satellite access, TrackingInc is using the service of IoTSAT for the 5G IoT connectivity by satellite and IoTSAT is using a LEO constellation which supports S&F operation mode.

All IoT remote monitoring UEs are regularly sending information related to the area they are monitoring to the application server of TrackingInc and some times receive new parameters from the application server. In most of the cases, the messages exchanged are delay-tolerant/non-real-time IoT.

### 5.2.2	Pre-conditions

In the present use case, the IoT remote monitoring UE is in a remote area with no ground stations available for feeder link connectivity and the IoT remote monitoring UE is aware that IoTSAT constellation operates in S&F mode.

### 5.2.3	Service Flows

The TrackingInc application server needs to send new parameters to the IoT remote monitoring UE through a dedicated message.

The TrackingInc application server message is sent by conventional means (e.g. IP routing, tunnels) to the network entry-point (e.g. a SCEF, PDN-GW, SMSC).

At this point:

- Limitations on the amount of data to be transferred to the IoT remote monitoring UE could be enforced.

- Delivery priority could be established.

- Acknowledgement of the received data by the network could be issued.

- End-to-end acknowledgement policy can be established.

The network stores the message until it can be delivered/relayed to a satellite expected to fly over and provide coverage to the destination IoT remote monitoring UE.

When the satellite is connected via the feeder link to the ground network, the message is uploaded into the satellite.

When flying over the area that the IoT remote monitoring UE is located, the satellite with the stored message triggers paging over the service link for the UE to connect to the network.

The stored message is delivered/downloaded from the satellite to the IoT remote monitoring UE. Acknowledgment may be requested/issued. Mechanisms to ensure integrity of the delivered information may be in place.

### 5.2.4	Post-conditions

The message generated by the TrackingInc application server has been delivered successfully to the IoT remote monitoring UE without relying on a continuous end-to-end network connectivity path between them.

### 5.2.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2], clause 6.3.2.3 on satellite access includes the following requirements:

The 5G system shall be able to provide services using satellite access.

The 5G system with satellite access shall be able to support low power MIoT type of communications.

However, it is not sufficient in regards of S&F operation especially for the delivery of delay-tolerant/non-real-time IoT NTN services with NGSO satellites.

### 5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-001] The 5G system with satellite access shall be able to inform an application server whether store and forward operation is applied for communication with a UE.

[PR 5.2.6-002] The 5G system with satellite access shall be able to provide integrity protection and confidentiality for communications between an authorized UE and the network when store and forward operation is applied.

Editor's Note:	it is FFS to clarify how security would apply during S&F operation.

## 5.3	Use case on Store and forward operation with discontinuous feeder link for delay-tolerant IoT - Inter-satellite

### 5.3.1	Description

To expand the market of delay-tolerant IoT devices, store and forward operations are necessary to be developed to sustain the user plane data during the feeder link disconnection between the satellite and the terrestrial gateway. Based on the earlier studies [3][4], there are many use cases can be further improved with such mechanisms.

Regardless of a simple scenario describing a relative static location relationship of a IoT device, a satellite and a terrestrial gateway, the serving satellite may change to another one during the time when the feeder link is unavailable.

As shown in Figure 5.3.1-1, a mobile IoT device may move from the coverage of one satellite to the other (e.g. containers tracing and tracking), or as shown in Figure 5.3.1-2, a NGEO satellite may fly away and the other one will come and turn to serving a static IoT device. Under such circumstances, the serving satellite may forward the stored user plane date to the next serving satellite through Inter-Satellite Links, and the next serving satellite may help forward the data to the gateway.

Meanwhile, if the feeder link of the next satellite is also unavailable, it will continue the store operation until the recovery of its feeder link. In this way, for every single IoT device, there will be only one satellite for its data storage in the overall satellite system. And the mobile operators will be easier to manage and maintain the data rather than dealing with the separate data which is belong to one device but among different satellites.

Significantly, during the period that the feeder link is unavailable, the serving satellite only stores or forwards (Inter-satellite) the data received from an IoT device which is already able to send data to the application server through the mobile network with satellite access. Because of the disconnection separates the two parts of the mobile network temporarily, the part in the serving satellite will not be able to fulfill common communication procedures and it will refuse any access from an unregistered device.

Furthermore, considering the limited data storage in satellite and the large amount of IoT devices, a maximum storage for each IoT device should be pre-configured based on the application data characteristics, user subscriptions and overall performance of satellite communication system.

Figure 5.3.1-1: Serving satellite change during the feeder link disconnection - IoT device moving

Figure 5.3.1-2: Serving satellite change during the feeder link disconnection - satellite moving

### 5.3.2	Pre-conditions

A delay-tolerant device has a subscription with the terrestrial operator TerrA, and it is tagged on one container for tracing and tracking.

TerrA has agreement with the satellite operator SatA for satellite access.

SatA maintains multiple serving satellites for the satellite access of TerrA’s subscribers all over the world, including Adam and Bob.

### 5.3.3	Service Flows

1. The container will be shipped from the Harbour A to the Harbour B across the Pacific. After the cargo ship leaves the Harbour A, the device can send some packets through the satellite access during the shipping time.

2. Due to some reasons, the feeder link between the serving satellite Adam and terrestrial gateway is interrupted temporarily or couldn’t be used for a time.

3. Based on the configuration of store and forward operations, Adam will go on to receive the packets from the device, and store these packets until the feeder link recovers.

4. However, during the period of feeder link is unavailable, the cargo ship approaches the border of coverage of Adam and will head to the coverage of another satellite Bob. So, based on the movement of the cargo ship, the serving satellite will change.

5. During the period of the change, Adam sends the stored packets to Bob through the inter-satellite link and Bob will forwards the packets to the gateway if its feeder link is available.

6. Particularly, Bob will continue storing the packets if its feeder link is also unavailable.

### 5.3.4	Post-conditions

1. Those packets will be finally sent to the application server by the network behind the gateway, e.g. transportation network, core network, internet. And the application will parse some information from the packets.

### 5.3.5	Existing features partly or fully covering the use case functionality

Editor's Note:	it is FFS whether other existing features will be identified.

### 5.3.6	Potential New Requirements needed to support the use case

[P.R.5.3-001] Subject to operator policies, a 5G system with satellite access shall be able to store data received from authorized delay-tolerant IoT devices while the feeder link is unavailable.

[P.R.5.3-002] Subject to operator policies, a 5G system with satellite access shall be able to support forward the stored data received from delay-tolerant IoT devices to an intermediate satellite through Inter-Satellite Links only when the intermediate satellite takes over the communication of these devices.

Editor's Note:	[P.R.5.3-002] is FFS.

[P.R.5.3-003] Subject to operator policies, a 5G system with satellite access shall be able to define the maximum amount of data storage per satellite per delay-tolerant IoT device.

[P.R.5.3-004] The 5G system with satellite access shall be able to authorize the communication of a UE when the satellite access is operating in store and forward mode.

Editor's Note:	The wording and explanation of store and forward (S&F) operation will align with corresponding definitions in the final TR.

Editor's Note:	it is FFS whether other potential new requirements can be identified.

## 5.4	Use case on data transfer for IoT devices in remote areas

### 5.4.1	Description

Data transfer at remote sites is a very common requirement. Research institutions can obtain data from remote sites for scientific research, e.g. animal tracking [5]. Government agencies can obtain data from remote sites for disaster mitigation and avoidance, e.g. via remote sensing [6]. Commercial companies can obtain data from remote sites for proper resource allocation. Data transmission at many remote sites is delay-insensitive, and satellite coverage does not always ensure that satellites connect to both the serving link and the feeder link. In the past 30 years, many scholars have devoted themselves to studying the data transmission problem of remote sites, and developed the store and forward mechanisms to solve the problem [7][8][9].

In remote areas, there is no terrestrial network for various reasons, e.g. it is difficult to build and maintain communication towers. As a result, this makes it challenging to collect information for environmental protection purposes in these areas. For example, sensors installed on animals need to be monitored regularly. In this scenario, the sensors installed on the animals send the status information, e.g. the movements, physiology and surrounding environment of the animals, to the satellite; and the satellite stores the received status information of the animals, and forwards the information to the scientific centre when a feeder link becomes available.

### 5.4.2	Pre-conditions

EA Science Center has installed sensors (IoT devices) on the animals to collect information for environmental protection purposes in these remote areas. Satelles, the satellite communication operator, has launched the store and forward (S&F) operation to support the data transferring for the remote areas. EA Science Center has signed contract with Satelles to allow sensors installed on animals to send the status information (e.g. the movements, physiology and surrounding environment of the animals) to the Science Center via satellite.

The satellite and the IoT devices are properly configured with sufficient information, e.g. credential/certificate that is needed for the devices to verify the authenticity of the satellite.

### 5.4.3	Service Flows

Figure 5.4.3-1: Animal tracking in the remote areas

1. The IoT devices are installed on animals and powered on, they are registered with the 5G network for the S&F operation. The satellite with the store and forward function enables the IoT devices to transfer data to the network, even when the feeder link to the ground is not available. A secured connection between an IoT device and the satellite is established to protect the data security and privacy.

2. The IoT devices send sensor status information to the satellite, the satellite stores the sensor status information received from the IoT devices.

3. When the satellite has the feeder link available to the ground segment, the satellite forwards the sensor status information, as well as other necessary information, to the ground core network. The ground core network verifies the IoT devices based on the information received; if it is allowed, the ground core network forwards the sensor status information to its destination data network.

4. The ground core network sends the result of the operation to the satellite (the same satellite or a different one that will pass through the remote area).

5. When the satellite (or next satellite) passes through the remote area, the satellite pages the UE, and based on the result received from ground core network, the satellite sends result of the operation to the IoT devices.

6. If an IoT device needs to update the sensor status information, it can send it to the satellite when it is connected to the satellite. The satellite stores it and forwards the sensor status information to the ground core network when feeder link becomes available.

### 5.4.4	Post-conditions

After the scientific centre receives the sensor status information, the scientists can analyse the sensor status information, and track the status of the animals.

### 5.4.5	Existing features partly or fully covering the use case functionality

None.

### 5.4.6	Potential New Requirements needed to support the use case

[PR 5.4-001] The 5G system shall support mechanisms to store user data, received from UEs via satellite access, on the satellite and forward it when feeder link between the satellite and the ground segment is available.

[PR 5.4-002] The 5G system shall support mechanisms for a user to securely register a UE to use the store and forward operation in the locations where air interface between the satellite to the UE and/or feeder link between the satellite and the ground segment are not constantly available.

NOTE: 	The user can be a third party, which is typically a business customer having service level agreement with the operator and interacting with the 5G network via an application server.

Editor’s Note: the above requirements are FFS.

[PR 5.4-003] The 5G system shall support the mechanisms to authenticate and authorize a UE for the store and forward operation.

[PR 5.4-004] The 5G system shall be able to limit the total amount of the stored data received from a UE when using the store and forward operation.

[PR 5.4-005] The 5G system shall be able to collect charging information per UE for use of the store and forward operation (e.g., data volume, duration, involved satellites).

[PR 5.4-006] The 5G system shall be able to collect charging information per application for use of the store and forward operation (e.g., number of UEs, data volume, duration, involved satellites).

## 5.5 	Use case on Temporary LAN using Satellite Access

### 5.5.1	Description

Satellite access networks are designed to ensure ubiquitous coverage and availability to any users with communication need. Beyond wide coverage capabilities, satellite connectivity can add flexibility to the network topology by providing an alternative route through satellite when terrestrial link is unavailable. The integration of satellite system, to compensate for the limitation of terrestrial network will benefit more users with interim but necessary service need.

When scientist-explorers move to the “blink spots” of terrestrial access network networks (e.g. deserts, oceans, polar regions, etc.), they need to keep collecting scientific data during the regional expedition and store it in local Scientific Research Station for analytic, retrieval, and synchronization with remote Scientific Data Center. In such condition, a provisional local area network (LAN) using satellite access could be an option to provide a temporary reliable communication service.

Scientific Expedition Team with tens of explorers and vehicles arrives at Antarctica to start scientific expedition activities in area LocArea for a month as Figure 5.5.1-1 depicts.

Figure 5.5.1-1: Temporary LAN using Satellite Access

In Figure 5.5.1-1, The local Scientific Research Stationequipped with a data center LocDC will support the data analytics and research in LocArea. The remote Scientific Data Center RemDC located in area RemArea served by Terrestrial Operator TerrA, will retrieve all the data and research results in a fixed time every day.

### 5.5.2	Pre-conditions

All UEs of the Team capable of satellite access are the subscribers of TerrA in RemArea.

LocArea has no terrestrial network but covered by satellite-enabled NR-RAN (e.g. LEO) of Satellite Operator SatA

SatA has an agreement with TerrA to provide 5G network services with the flexible network configuration of satellite and terrestrial elements in LocArea.

RemDC is connected through 5G core network of TerrA.

It’s assumed that UEs can always find serving satellites from the constellation. The serving satellite change is omitted from the flow.

### 5.5.3	Service Flows

Once the explorers settle down in Scientific Research Station, LocDC is provisioned and authorized to connect to 5G core network of TerrA through satellite or ground gateway regarding the management policy agreed by SatA and TerrA.

All UEs of the Team are enabled the corresponding services in LocArea regarding the device setting and subscription information.

During the activities in LocArea, all UE register to TerrA 5G network through SatA’s satellite access network to upload data to LocDC with the optimal route in time.

LocDC can sync up data to RemDC through 5G network configured by SatA and TerrA with the optimal route in each fixed time slot.

Once leave LocArea, all UEs will autonomously switch the services through available terrestrial access network of TerrA or other operators in service agreement with TerrA (e.g. roaming, shared network, etc).

LocDC is disconnected from 5G network regarding the original provisioning information.

### 5.5.4	Post-conditions

Local satellite access network of SatA will be routed to LocDC for data exchange when LocDC is in service.

All the data is successfully transferred between UEs and LocDC, LocDC and RemDC.

### 5.5.5	Existing features partly or fully covering use case functionality

Regarding TS 22.261 [2], satellite access and satellite connectivity are supported in Rel-18, as

The 5G system shall be able to provide services using satellite access.

UEs supporting satellite access shall support optimized network selection and reselection to PLMNs with satellite access, based on home operator policy.

The 5G system with satellite access shall support the use of satellite links between the radio access network and core network, by enhancing the 3GPP system to handle the latencies introduced by satellite backhaul.

The 5G network can also support multiple wireless backhaul connections (e.g. satellites and/or terrestrial), and efficiently route and/or bundle traffic among them.

### 5.5.6	Potential New Requirements needed to support the use case

[PR 5.5.6-001] Subject to regulatory requirements and operator preferences, the 5G system shall be able to support an efficient communication path and resource utilization for a UE using only satellites access, e.g. to minimize the latencies introduced by satellite links involved.

## 5.6	Use case on Information Exchange between Ships at sea

### 5.6.1	Description

Information exchange in the maritime industry is very important [10] [11]. Information exchange allows for better coordination between ships, which is required regardless of the purpose for which ships are sailing. An effective information exchange system can better coordinate the ships and improve the safety of ships, such as resisting pirate attacks, avoiding accidents at sea, and rescuing.

At sea far from land, there is no terrestrial communication system. Ships can communicate directly with each other at short distance through various types of wireless technologies. At long distances, information can only be exchanged through satellites and then through remote data centres, which affects communication efficiency, especially in emergency situations. In addition, in some areas, the satellite has no available feeder link, which causes the communication interruption even though the communicating ships camp on the same satellite. In this scenario, communication between ships through satellites without going via remote data centres can improve communication efficiency and reduce losses caused by potential maritime accidents.

Satellite broadband can be suited to connecting remote areas which do not have reliable mobile or fixed broadband. There are new broadband satellites systems being developed, which use many satellites in a non-geostationary satellite orbit (NGSO) closer to the Earth than earlier satellites. Typically, the beam footprint size of Low-Earth Orbit (LEO) satellites and Medium-Earth Orbit (MEO) satellites is in the range of 100 – 1000 km [12].

### 5.6.2	Pre-conditions

MinosShipping, the shipping company, has many ships operating all over the world. MinosShipping signs a contract with Delphi, an operator with satellite communication services. Delphi has deployed NGSO satellites, which allows communication between ships via satellite without going through the ground network, that is, devices on a ship can communicate directly with devices on another ship via satellite.

### 5.6.3	Service Flows

Figure 5.6.3-1: communication via the same satellite without going through the ground network

1. Device A on ship #1 register with the 5G network via satellite, and device B on ship #2 (small) also register with the 5G network via satellite. The devices A and B can communicate with each other via the 5G network.

2. When the ship #1 and the ship #2 are under the same satellite coverage, the devices A and B want to communicate with each other. The remote core network authorizes the communication between the devices A and B based on e.g., subscription, and location information. After getting authorized, the data traffic between the devices A and B is routed through the same satellite.

Figure 5.6.3-2: communication via satellites with ISL without going through the ground network

3. Along the long journey, ship #1 and ship #2 move across the coverage of different satellites, i.e. ship #2 moves to the coverage of satellite #2 while ship #1 remains in the coverage of satellite #1. Inter satellite link is available between satellite #1 and satellite #2.

4. During the journey, the communication between devices A and B via satellite(s) continues without interruption.

5. 	The charging information of the traffic data exchanged via the satellites is collected in the satellites and reported to the remote core network.

### 5.6.4	Post-conditions

The ship #1 and the ship #2 can exchange information efficiently without data traffic transferred via the remote core network.

### 5.6.5	Existing features partly or fully covering the use case functionality

None.

### 5.6.6	Potential New Requirements needed to support the use case

[PR 5.6.6-001] The 5G system shall support mechanisms to authorize the communication between UEs via satellites (without going through the ground network) based on e.g., location information and subscription.

[PR 5.6.6-002] The 5G system shall support mechanisms to collect charging information for the traffic data exchanged via satellites without going through the ground network.

[PR 5.6.6-3] Subject to regulatory requirements and operator’s policy, the 5G system shall support communication between UEs via satellites without going through the ground network.

[PR 5.6.6-4] Subject to regulatory requirements and operator’s policy, the 5G system shall maintain service continuity with minimum service interruption of the communication between UEs via satellites without going through the ground network when a UE changes from the coverage of one satellite to another (due to the movement of the UE and/or the satellites).

Editor's Note:	For the requirements above, further clarification may be needed on gap/overlap with ongoing R18 stage-2 work

## 5.7	Use case on UAVs using satellite access

### 5.7.1	Description

In Mediterranean climate regions, forest fires are quite common in summer months due to temperature rise [13]. The early detection and monitoring of forest fire is important for fire suppression quickly and reducing the loss of human and property. Therefore, how to detect the forest fire in real time and accurately is an urgent problem to be solved. Another problem is that due to extremely low population density and complex geography, these regions are often not covered by terrestrial networks.

UAV equipped with satellite access capabilities is a feasible method, mainly through the following steps:

- The UAV collects real-time information (including high-precision three-dimensional surface topographic data, real-time pictures, real-time video, etc.);

- This real-time information is transmitted to the forest fire monitoring centre via the 5G network with satellite access;

- The forest fire monitoring centre regularly monitors the flight status and may adjust the route;

- Adjustment command are sent to the UAV via the 5G network with satellite access.

Considering the dense trees in the forest, the satellite positioning may be inaccurate. This will cause the offset of scheduled route. Forest fire monitoring centre with AI system can optimize the route through real-time information collected by UAV. In addition, the 5G system provides high-precision positioning of the UAV, which has been specified in 3GPP TS 22.261 [2]. 5G system gets the real-time location data of flying UAV, then sends them to a trusted third party (e.g., The forest fire monitoring centre equipped with AI system) for UAVs to assist flying.

### 5.7.2	Pre-conditions

Forest fire monitoring centre has several UAVs to patrol the Forest A.  Each UAV has a 4K camera for collecting real-time pictures.

In Forest A, there is no terrestrial network. So, the Forest fire monitoring centre has signed contract with Sat A, an operator with satellite communication services. Then, these UAVs can send real-time pictures to the forest fire monitoring centre via satellite.

The forest fire monitoring centre supports UTM function, and deploys AI system. It can evaluate these pictures to determine whether a fire is present or whether the UAV's flight route is off-course.

### 5.7.3	Service Flows

1. The UAV A takes a real-time picture with its 4-way 4K full-angle camera;

2. The picture is transmitted to the forest fire monitoring centre via the 5G network with satellite access network. This would require high data rate (e.g., 120Mbit/s) in UL direction.

3. The forest fire monitoring centre uses the AI system to calculate the offset of scheduled route and determine whether there is a fire, according to the received picture and high-precision positioning provided by 5G system with satellite access.

4. If the forest fire monitoring centre decides to adjust the route of UAV A, it will send adjustment commands to the UAV via the 5G network with satellite access network, which requires low delay and high reliability in DL direction.

### 5.7.4	Post-conditions

UAV adjusts its route.

### 5.7.5	Existing features partly or fully covering use case functionality

There are a few position requirements specified in 3GPP TS 22.125 [14], which have been described as:

[R-5.1-009] The 3GPP system should enable an MNO to augment the data sent to a UTM with the following: network-based positioning information of UAV and UAV controller.

NOTE 1: 	This augmentation may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information, such as OTDOA, cell coordinates, synchronization source, etc.

NOTE 2: 	This requirement will not be applied to the case which the UAS and UTM has direct control communication connection without going through MNO, such as OTDOA, cell coordinates, synchronization source, etc.

[R-5.1-012] The 3GPP system shall enable a UAS to update a UTM with the live location information of a UAV and its UAV controller.

[R-5.1-013] The 3GPP network should be able to provide supplement location information of UAV and its controller to a UTM.

NOTE 3: 	This supplement may be trust-based (i.e. the MNO informs the UTM that the UAV position information is trusted) or it may be additional location information based on network information.

There are also a few position requirements specified in 3GPP TS 22.261 [2], which have been described as:

The 5G system shall support mechanisms to determine the UE’s position-related data for period when the UE is outside the coverage of 3GPP RAT-dependent positioning technologies but within the 5G positioning service area (e.g. within the coverage of satellite access).

In 3GPP TS 22.071 [x], the following location service requirements are captured:

The precision of the location shall be network design dependent, i.e., should be an operator’s choice. This precision requirement may vary from one part of a network to another.

About horizontal accuracy:

The LCS service shall provide techniques that allow operators to deploy networks that can provide at least the level of accuracy required by the regional regulatory bodies.

10m-50m: Asset Location, route guidance, navigation

About vertical accuracy:

For Value Added Services, and PLMN Operator Services, the following is applicable:

-	When providing a Location Estimate, the LCS Server may provide the vertical location of a UE in terms of either absolute height/depth or relative height/depth to local ground level. The LCS Server shall allow an LCS Client to specify or negotiate the required vertical accuracy. The LCS Server shall normally attempt to satisfy or approach as closely as possible the requested or negotiated accuracy when other quality of service parameters are not in conflict.

-	The vertical accuracy may range from about three metres (e.g. to resolve within 1 floor of a building) to hundreds of metres.

About location delivery latency:

Location Delivery Latency (Time to First Fix) is set at a maximum of 30 seconds from the time the user initiates an emergency service call to the time it is available at the location information center.

Editor's Note:	it is FFS whether other existing features or requirements will be identified.

### 5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] The 5G system with satellite access shall be able to support suitable positioning mechanisms for UAV services.

Editor's Note:	it is FFS whether other potential new requirements or corresponding KPIs can be identified.

## 5.8	Use case on Enhanced Positioning Service using Satellite Access

### 5.8.1	Description

During natural disasters, the recovery of communication services and the acquisition of the survivors’ location are important to aid the rescue activities. Normally, satellite communication networks and standalone GNSS are utilized to serve communication and positioning independently. With the integration of satellite access in 5G systems, it’s possible to provide both communication and positioning services by 5G system together to address the cases that GNSS cannot provide reliable positioning service (e.g. poor GNSS signal, limited visible satellites). Meanwhile, the positioning performance like accuracy can be improved with the assistance of 5G satellites (e.g. LEO), network information, etc. [15]

Indonesia is famous for its extraordinary natural landscapes attracting millions of tourists around the world. Meanwhile, it is widely recognized as one of the most disaster-prone countries in the world according to data released by the United Nations International Disaster Reduction Agency (UN-ISDR) [16]. Any a disaster (e.g. earthquake, tsunamis) will impact tens of millions of people, who may get support from the sustainable and reliable communication service and positioning service of 5G systems.

### 5.8.2	Pre-conditions

Bali, Indonesia is covered by terrestrial access network of Operator TerrA and satellite access network of Operator SatA, which share the core network of TerrA deployed in Jakarta with mutual agreement. 5G communication service and positioning service have been launched all through Indonesia.

GNSS (e.g. GPS, BeiDou) are allowed to use in Indonesia, but are independent from 5G satellite constellation.

It is assumed that UEs served by TerrA network are capable of 5G satellite access and integrated with different types of GNSS receivers.

### 5.8.3	Service Flows

A tsunami strikes Bali and has destroyed most infrastructures and terrestrial access networks. The core network is still in service without damage.

The awake survivors will initiate an emergency call or send an emergency message to report personal information, injuries, and surrounding conditions to Indonesia Rescue Center for rescue requests through SatA access network. During the interaction, the precise location information (e.g. accurate latitude) of the survivors is requested to report to Rescue Center from UE or/and the network within the requested response time of local regulatory requirements. The location of the survivors and rescue personnel will be sent to the survivors as well for preparation.

All powered-on UEs will autonomously update registration in TerrA’s network using satellite access. The network identifies the areas where the devices are located and authorizes Rescue Center to fetch real-time devices’ location and tracing log during the Rescue UAV or Helicopter searching for the survivors in a coma or not able to report emergency information actively.

### 5.8.4	Post-conditions

The locations of the awake survivors are identified as compliant with regulatory requirements.

The location information of powered-on devices is available in Rescue vehicles.

### 5.8.5	Existing features partly or fully covering use case functionality

Regarding TS 22.261[2] as below, UE using satellite access shall have the capability to offer the location, and 5G system needs to determine the location for service, but not consider the situation that the location cannot be decided by UE alone.

A UE supporting satellite access shall be able to provide or assist in providing its location to the 5G network.

A 5G system with satellite access shall be able to determine a UE's location in order to provide service (e.g. route traffic, support emergency calls) in accordance with the governing national or regional regulatory requirements applicable to that UE.

The legacy requirements for positioning service defined in TS 22.261 [2] are not well adapted to all types of UE with satellite access considering the satellite characteristics (e.g. latency).

The 5G system shall provide 5G positioning services in compliance with regulatory requirements.

The 5G network shall be able to request the UE to provide its position-related-data on request—together with the accuracy of its position—triggered by an event or periodically and to request the UE to stop providing its position-related data periodically.

The 5G System with satellite access shall be able to negotiate the positioning methods according to the operator's policy or the application’s requirements or the user's preferences and shall support mechanisms to allow the network or the UE to trigger this negotiation.

### 5.8.6	Potential New Requirements needed to support the use case

[PR 5.8.6-001] Subject to regulatory requirements and operator policy, the 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access.

[PR 5.8.6-002] The 5G system with satellite access shall be able to negotiate the positioning methods according to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS) and shall support mechanisms to allow the network or UE to trigger the negotiation.

# 6	Additional considerations

# 7	Consolidated requirements

# 8	Conclusions and recommendations

Annex A (informative):
Store and forward satellite operation

The store and forward operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

An example of “S&F satellite operation” is illustrated in Figure A-1, in contrast to what could be considered the current assumption for the “normal/default satellite operation” of a 5G system with satellite access.

As shown in Figure A-1:

- Under “normal/default satellite operation” mode, signalling and data traffic exchange between a UE with satellite access and the remote ground network requires the service and feeder links to be active simultaneously, so that, at the time that the UE interacts over the service link with the satellite, there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network.

-	In contrast, under “S&F satellite operation” mode, the end-to-end exchange of signalling/data traffic is now handled as a combination of two steps not concurrent in time (Step A and B in Figure A-1). In Step A, signalling/data exchange between the UE and the satellite takes place, without the satellite being simultaneously connected to the ground network (i.e. the satellite is able to operate the service link without an active feeder link connection). In Step B, connectivity between the satellite and the ground network is established so that communication between the satellite and the ground network can take place. So, the satellite moves from being connected to the UE in step A to being connected to the ground network in step B.

| “Normal/default Satellite operation” mode |  |
| --- | --- |
| “S&F Satellite operation” mode |  |

Figure A-1: Illustration of “normal/default operation” and “S&F operation” modes in a 5G system with satellite access.

The concept of “S&F” service is widely used in the fields of delay-tolerant networking and disruption-tolerant networking. In 3GPP context, a service that could be assimilated to an S&F service is SMS, for which there is no need to have an end-to-end connectivity between the end-points (e.g. an end-point can be a UE and the other an application server) but only between the end-points and the SMSC which acts as an intermediate node in charge of storing and relying.

The support of S&F satellite operation is especially suited for the delivery of delay-tolerant/non-real-time IoT satellite services with NGSO satellites.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | SA1#99e | S1-222089 |  |  |  | TR skeleton | 0.0.0 |
| 2022-08 | SA1#99e | S1-222326 |  |  |  | Scope for the TR22.865 | 0.1.0 |
| 2022-08 | SA1#99e | S1-222327 |  |  |  | new definitions and abbreviations | 0.1.0 |
| 2022-08 | SA1#99e | S1-222334 |  |  |  | Use case store and forward - MO | 0.1.0 |
| 2022-08 | SA1#99e | S1-222335 |  |  |  | Use case store and forward - MT | 0.1.0 |
| 2022-08 | SA1#99e | S1-222328 |  |  |  | Use case of store and forward operation with discontinuous feeder link for delay-tolerant IoT - Inter-satellite | 0.1.0 |
| 2022-08 | SA1#99e | S1-222333 |  |  |  | Pseudo-CR on Use case of data transfer for IoT devices in remote areas | 0.1.0 |
| 2022-08 | SA1#99e | S1-222329 |  |  |  | Use Case: Temporary LAN using satellite access | 0.1.0 |
| 2022-08 | SA1#99e | S1-222331 |  |  |  | Pseudo-CR on Use case of Information Exchange between ships at sea | 0.1.0 |
| 2022-08 | SA1#99e | S1-222332 |  |  |  | New use case of UAVs using satellite access | 0.1.0 |
| 2022-08 | SA1#99e | S1-222330 |  |  |  | Use Case: Enhanced Positioning Service using Satellite Access | 0.1.0 |
| 2022-08 | SA1#99e | S1-222336 |  |  |  | Description of store and forward operation | 0.1.0 |