TSG-SA Working Group 2 meeting #3	TSGS2(99)141 (107 modified)

Nynäshamn 15-19 March 1999

Agenda item: common TSG-S2 + RAN WG3

Agenda Item: 	23.20, Key Issue: MM and Iu

Source: 	Alcatel

Title: 	Principles of SRNS relocation

Document for:	23.20 Change

# Introduction

The aim of this document is to put in 23.30 and 23.20 some clarifications on the principles to be used for SRNS relocation procedures.

# Discussion

To carry out SRNS relocation, the source  SRNC must launch the SRNS relocation procedure, since it is not the target SRNC but the source SRNC that knows the current services of an user. This is done only when this procedure has the least adverse effect on user traffic,

The SRNC relocation procedures shall ensure that there is only one Serving RNC for an user even if this user has services through more than one (IP or ISDN) domain.

The SRNS relocation procedure is split in 2 phases. In the first one resources are reserved on the new Iu interfaces and (if needed) inside the CN. Only when this first phase has been successfully carried out for all domains on which the user has currently some services, the source SRNC can launch the second phase i.e. hand-over the role of SRNC to the target  SRNC.

# Proposal

It is proposed that the content of section 2 of this Tdoc is included in (new) section 7.3.11.2.0 chapter of 23.20 (on "SRNS relocation principles") and that it is also included in a new section 4.2.i of 23.30 (to be created) on "SRNS relocation"

# References

1	UMTS 23.20, version 1.5.0

[2]	UMTS 23.30 v1.1.0