ETSI SMG 29

Dublin

12-16 April 1999								Tdoc 2-99-473

From:	SMG2 WPA

To:	SMG1, SMG12

CC:	SMG3 WPA

# Proposition of CRs concerning the clarification to the class B mode of operation

SMG2 WPA has considered the definition of the class-B mode of operation specified in TS GSM 02.60 and in TS GSM 03.60.

Indeed different understandings appear within SMG2 WPA, particularly on following issues :

- shall a mobile station in packet transfer mode monitor the circuit switched paging channel ?

- shall a mobile station in packet transfer mode systematically answer to a circuit switched paging message ?

Note : the second question is relevant whatever the answer to the first question  (case of paging coordination).

It was hence largely debated on how the selection between the appropriate service is performed by the MS user.

SMG2 WPA considers that there is no requirement for a MS operating in class B mode of operation to listen to circuit switched paging channel when in packet transfer mode. In case a MS operating in class-B mode of operation receives a paging for circuit-switch call when in packet transfer mode, it may either :

- Ignore the paging message ; or

- Answer the paging thereby aborting the current GPRS data transfer(s) and suspend any GPRS activity until return to idle mode.

The selection between the two possibilities would be made automatically by the MS, upon reception of the circuit-switched paging message, probably based upon a preset user input.

Also SMG2 WPA would like to stress out that they took the above statements as working assumptions.

With this view, SMG2 WPA has drafted the enclosed 02.60 and 03.60 change requests, in order to propose them to SMG1 and SMG12.

SMG2 WPA need a Stage 1 decision in this matter.

SMG1 is kindly asked if it can approve the 02.60 CR. SMG12 is kindly asked if it can approve the 03.60 CR.

In case of disapproval, SMG2 WPA would also ask the respective groups to advise them about their choice.

SMG2 WPA thank by advance SMG1 and SMG12 for their time in considering this matter, and appreciate their help in solving this issue.

ETSI SMG 29	Tdoc 2-99-475

Dublin			 						revision of Tdoc 2-99-262

12-16 april 1999

| CHANGE REQUEST No : |  |  |  |  | Axxx |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 02.60 |  | Version: |  |  | 6.2.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| List SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | SMG2 WPA | Date: | 14/4/99 |
| --- | --- | --- | --- |

| Subject: | Clarification to the definition of class B mode of operation |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| And one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification |  |  | Release 99 |  |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | The description of the class B mode of operation in the current GSM standard allows for different interpretations. The definition needs hence to be clarified. |
| --- | --- |

| Clauses affected: | 5.4.5 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 5.4.5	GPRS MS class mode of operation

The purpose of the definition of the GPRS MS class mode of operation is to enable the different needs of the various market segments to be satisfied by a number of MS types with distinct capabilities (e.g., simultaneous use and number of time-slots) .A means shall be provided to indicate the multi-slot capability and current configuration to the network when necessary.

Three GPRS MS class mode of operation are identified:

The term simultaneous (attach, traffic, etc.) is the requirement to simultaneously support GSM GPRS services and GSM circuit switched services including SMS.

Class A:	Supports simultaneous attach, simultaneous activation, simultaneous monitor, simultaneous invocation and simultaneous traffic. The mobile user can make and/or receive calls on the two services simultaneously subject to the QoS requirements.

A minimum of one time slot shall be available for each type of service (circuit switched and GPRS) when required.

Class B : 	The MS is attached to both GPRS and other GSM services, but the MS can only operate one set of services at a time.
When the MS is both in idle mode and packet idle mode, it shall be able to monitor receive paging channels both for circuit switched and packet switched services.
There is no requirement for the MS to monitor the paging channels if the MS is in packet transfer mode or in dedicated mode. Monitoring of paging channels in packet transfer mode is a matter of MS implementation; such a monitoring should not degrade the established service nor waste radio resources. In case the MS receives a circuit switched paging messages when in packet transfer mode, the MS shall either:
	* ignore the paging messages
	* or answer the paging thereby aborting the current GPRS data transfer(s) and suspending any GPRS activity until return to idle mode.,
the selection between the two possibilities being an MS initial configuration (possibly determined by the MS user)
The selection between the two possibilities is made automatically by the MS upon reception of the circuit switched paging message, probably based upon a preset user input.

Class C:	Supports only non-simultaneous attach. Alternate use only. If both services (GPRS and Circuit Switched) are supported then a Class C MS can make and/or receive calls only from the manually or default selected service, i.e., either GPRS or Circuit Switched service. The status of the service which has not been selected is detached i.e., not reachable. The capability for GPRS-attached class-C MSs to receive and transmit SMS messages is optional.

The network shall support SMS message reception and transmission for GPRS-attached class-C MSs.

An MS may be reconfigured. E.g., a class A MS configured as 1 slot for circuit switched plus 1 slot for GPRS may be reconfigured as a class C configured as 0 slots for circuit switched plus 2 slots for GPRS.

Non-voice only MSs do not have to (but may) support emergency calls.

allocate a subset of the numbers privately shall be possible.

ETSI SMG 29	Tdoc 2-99-263

Dublin

12-16 april 1999

| CHANGE REQUEST No : |  |  |  |  | Axxx |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 03.60 |  | Version: |  |  | 6.3.1 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Alcatel | Date: | 6/4/99 |
| --- | --- | --- | --- |

| Subject: | MS class mode of operation |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | The definition of the class B mode of operation has been refined in GSM TS 02.60 as it allowed for different interpretations. The definition of MS class mode of operation was unnecessarily and incompletely duplicated in GSM TS 03.60. It is proposed to replace it by a reference to GSM TS 02.60. |
| --- | --- |

| Clauses affected: | 5.4.5 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 5.4.5	GPRS Mobile Stations

A GPRS MS can operate in one of three modes of operation. The mode of operation depends on the services that the MS is attached to, i.e., only GPRS or both GPRS and other GSM services, and upon the MS's capabilities to operate GPRS and other GSM services simultaneously.

-

--

-

The three modes of operation are defined in GSM TS 02.60.

NOTE:	Other GSM technical specifications may refer to the MS modes of operation as GPRS class-A MS, GPRS class-B MS, and GPRS class-C MS.