Source: 	TSG SA WG2

Title: 	Liaison statement on Area concept

To: 	3GPP Workshop on Handover and Cell Selection

Currently, there are some discussions within TSG SA WG2 on an area concept with GSM and UMTS cells in the same registration area (location area/routing area). The attached text is included in 23.20 (23.120) and is hereby provided for your information.

# 9.x   GSM and UMts cells in the same registration area

The concept of GSM and UMTS cells in the same registration area was introduced in order to minimize location update signaling when changing between GSM and UMTS systems. Especially, the lack of UMTS coverage, e.g. in-building coverage in urban or suburban areas, can lead mobiles frequently changing between GSM and UMTS systems. In such case, the common registration area concept is anticipated to reduce the signaling compared to GSM and UMTS cells in different registration areas.

Currently GPRS routing area updates cause serious disruption to the user plane traffic. This was acceptable in GPRS only because routing area updates would be rare events which should only occur at the borders of large, well planned geographic areas.

If GSM and UMTS routing areas are overlaid, it can be expected that many mobiles will change routing areas at rates greater than in GPRS. In such cases the (negative) impact on the quality and throughput of the data will be very significant. Placing UMTS and GSM cells in the same RA greatly alleviates this effect.

If the core network does not use the Gs interface then, while the mobile is performing a routing update in the GSM cell the mobile will not be pageable from the MSC. Frequent routing area updates will have a serious effect on mobile terminating call success rates. Note that implementing the Gs interface only really helps in the case that the GSM and UMTS cells use the same SGSN. Therefore, this pageability problem disappears when the UMTS and GSM cells are in the same RA.

Third generation needs to offer higher quality (eg higher MT call success rate) than second generation. Hence the capability to have GSM and UMTS cells in the same Registration Area is needed for at least CS traffic.

### 9.x.1   Open issues

The following open issues, opportunities and challenges have been identified concerning this concept:

- Security.

- In UMTS both MS authentication and network authentication is planned to be implemented. Some solutions have been proposed for MS authentication, ciphering and integrity check during change from GSM to UMTS but not any for network authentication. However, all these issues need to be resolved in order to perform handover between UMTS and GSM (either on the MSC, or, on the SGSN side, or both) so it is a requirement that the issue is solved.

- Network service capabilities.

- The availability of network service capabilities should be indicated to the end user somehow in order to offer confidence in the cellular service. However, any communication that starts in GSM cells needs to be easily transferable to UMTS cells, and vice versa. Usage of a UMTS cell does not guarantee any particular data rate: the data rate will vary with, at least, range, load on the cell and interference levels. Thus merely being camped on a GSM or a UMTS cell will give no real indication of the services available from that PLMN - this is irrespective of whether or not the cells are in the same or different registration areas.

- The most likely source of ‘network service capability’ information will come from the core network. As the mobile contacts the core network whenever it does a registration update, it seems sensible that we allow network operators the option to send “network service capability” information in MM (and GMM) messages sent to the mobile.

- Terminal capabilities

- Terminal capabilities have some affect on the service availability e.g. in a situation where a dualmode terminal makes an attach in UMTS and later on moves into GSM cell. If mobile is PS and CS attached in UMTS, it may need to detach either CS or PS in GSM (e.g. class C mobile).

- Idle mode control

- Within a particular visited network it can be expected that the Core Network will restrict some subscribers to only UMTS cells, restrict others to only GSM cells, and permit some to use both types of cell.

- Mobility Management signalling (MM and GMM) needs to be developed so that the (dual mode) mobiles in idle mode adapt their cell reselection procedures according to the Core Network’s instructions.

- Paging channels

When GSM and UMTS cells are in the same common registration area, the overall paging channel capacity is given by the minimum of the GSM paging channel capacity and the UMTS paging channel capacity. If the capacity of the two channels cannot be configured so that the UMTS paging capacity is larger or equal to that of the GSM channel, then paging capacity will be wasted. Note that UMTS paging channels with larger capacity than GSM paging channels probably do not cause any inefficiency (because under occupancy of the UMTS paging channel probably only leads to less radio energy being transmitted). Hence it is expected that UMTS paging channel capacity need to exceed or equal that offered by the GSM radio interface (in the cases of a GSM combined control channel and the case of a single GSM non-combined control channel). Note that this is relevant for both CS and PS domains.

In any case, in a common registration area GPRS pages are always sent also via UMTS paging channel, due to the nature of GPRS paging (CN paging). This may have some impact on paging channel capacity needed in UMTS.