SA WG2 Meeting #106	S2-144406

17 - 21 November 2014, San Francisco, California, USA	(revision of S2-144135)

Source:	Samsung

Title:	UE support indicator for Solution #1

Document for:	Approval

Agenda Item:	7.11

Work Item / Release:	eCSFB / Rel-13

Abstract of the contribution: This paper proposes that some indication is required to ensure the UE can support this type of CSFB functionality.

1. Introduction

As discussed quite much in SA2# 105 w.r.t Solution 1 in TR 23.772, some impacts were identified in section 5.1.2.  In particular, this paper focus on the following:

-	The Call Control entity UE behavior required for this solution is not specified in the related stage-2 and stage-3 specifications.

Editor’s Note: Further check with CT1 is needed in order to confirm this bullet.

2. Discussion

CSFB, as it was introduced, was meant to define different ways of how the UE can “move” from LTE to 2G/3G RAT so the CS procedure can be started with the correct RAT using native CS procedure.

Consequently, UEs which are now out in the field are only tested and should only be expected to work as according to the current defined stage 2/3 procedure in 3GPP.

CSFB with SRVCC is a new feature that has not been described as a whole in any stage 2 or 3 3GPP specification. UE vendor can’t be held responsible for existing UE which has not been tested again this unspecified feature even though Solution 1 is assuming these existing UE will just work!

It is proposed to add UE requirement to solution 1, i.e, only the UE that has been tested should be allowed to use this CSFB with SRVCC. Otherwise, there is no clear responsible party on why things are not working.

3. Proposal

*** begin texts ***

## 5.1	Solution #1: SRVCC based eCSFB Solution

### 5.1.1	Solution Description

Editor's Note: Describe the solutions. Sub-clause(s) may be added to capture details, procedural flow etc.

During the CSFB procedure, eNB can decide whether or not to initiate SRVCC procedure based on its knowledge of UE’s SRVCC capability indication in FGI[7]. If UE indicates it’s capable of SRVCC in FGI, even there is no QCI=1 bearer in eNB, the eNB will still initiate SRVCC procedure when receiving the S1-AP message with CSFB indicator from MME.

Editor’s Note: It is FFS whether a new indication (e.g., new FGI bit, new NAS indication) is needed or via some other means (e.g., IMEI checking) in order to avoid invoking this CSFB/SRVCC procedure for UE (i.e., legacy UE) which has not been tested for CSFB using SRVCC procedure.

The eCSFB procedures of applying SRVCC feature are illustrated in following subclauses. For simplicity, only the procedures without PS HO are depicted, same changes are applied to the procedures with PS HO.

*** End of change ***