3GPP TSG-SA WG2 Meeting #109 	S2-151987

# Fukuoka, Japan, 25th – 29th May 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.468 | CR | 0063 | rev | 2 | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of specification of Group Call suspension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12, GCSE_LTE-MBMS_CM |  |  |  |  |  |  | Date: |  | 2015-05-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The TSG RAN work item “Group Call eMBMS congestion management for LTE” added a new explicit MBMS bearer suspension signalling mechanism from E-UTRAN to UE. When MBMS congestion is detected, this mechanism allows E-UTRAN to faster trigger a switch to unicast data delivery and thus avoid or at least reduce potential service disruption of Group Calls. The current specification text for MBMS bearer suspension in section 5.4 does not describe how the new mechanism should be used on a system level. As it impacts RAN, UE, and GCS-AS, the SA2 spec is the correct place to describe the overall operation. Furthermore the existing section 5.4 text is misaligned with the new MAC signalling mechanism, in that: - The scenario in clause 5.3.3.3 only covers “break-before-make” but does not cover the “make-before-break” suspension scenario enabled by the new mechanism. - The scenario in clause 5.3.3.3 indicates that the UE stops monitoring MBMS channels and just monitors SIBs and not MBMS channels, which would not allow the UE to know when it should resume reception of DL data via MBMS. This is also misaligned with MCCH mechanism for detecting suspension. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification of section 5.4 to correct the MBMS suspension mechanism and not to refer to section 5.3.3.3, as the UE should not stop listening to MBMS channels. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intended UE behaviour following an MBMS suspension via MAC signalling would be unclear. Also the UE would not be able to know when data delivery via MBMS has resumed, which would restrict the ability to handle group calls effectively in congestion and decongestion scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[3]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[4]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.203: "Policy and charging control architecture".

[7]	3GPP TS 26.346: " Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[8]	3GPP TS 33.246: "3G Security; Security of Multimedia Broadcast/Multicast Service (MBMS)".

[9]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[10]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

-------------------------

NEXT CHANGE

-------------------------

## 5.4	Priority and Pre-emption for Group Communication

A Group Communication Session that requires to be prioritized over other Group Communication Sessions or non-Group Communication Sessions includes both the priority level and the GCS identifier in a service authorization request to the PCRF and to the BM-SC.

The priority level and the GCS identifier are defined at the application layer for priority and pre-emption purposes. The GCS AS provides the priority level and the GCS identifier to the PCRF and the BM-SC. It is mapped by the PCRF and the BM-SC to the ARP priority level, pre-emption capability and pre-emption vulnerability indication under the consideration of the respective EPS network.

In the case of Unicast Delivery, the PCRF translates the service characteristics to PCC policies and forwards its policy decision to the PCEF. The PCEF determines, based on policies provided by the PCRF, whether the PCEF modifies already established bearers or whether the PCEF establishes dedicated bearer(s) with the determined bearer characteristics. If the priority of a particular application Group Communication session changes, the GCS AS provides an update of the service characteristics towards the PCRF via the Rx interface. The PCRF updates the PCC policies and forwards them to the PCEF. The PCEF modifies already established bearers or establishes dedicated bearer(s) accordingly.

The PCRF shall, at the reception of service authorization from the GCS AS including an indication that is a prioritized GC Session and priority level, ensure that the ARP priority level of the default bearer is assigned a prioritized value which is at least as high as the highest priority of all Group Communication Sessions within the same IP-CAN session. The PCRF shall also ensure that the ARP pre-emption capability and pre-emption vulnerability indication of the default bearer satisfies the strongest requirements of all Group Communication Sessions within the same IP-CAN session.

When the PCRF detects that all prioritized Group Communication Sessions within the same IP-CAN session are released, the PCRF shall assign the ARP of the default bearer as appropriate.

In the case of MBMS Delivery, if the priority of an MBMS bearer needs to be changed, based upon a GCS AS decision to change the priority level, the GCS AS performs either:

-	The Modify MBMS Bearer procedure in clause 5.1.2.4; or

-	A new Activate MBMS Bearer procedure and a Deactivate MBMS Bearer procedure replacing the old MBMS bearer service with a new one.

In certain network conditions such as congestion, the bearer used for group communication service may be pre-empted.

-	In the case of Unicast Delivery, the GCS AS is notified by the PCRF of unicast bearer release.

-	In the case of MBMS Delivery, the related MBMS bearer may be 'suspended' by E-UTRAN, and the UE becomes aware in either of the following ways:

- 	 packets are dropped at the eNB. In this case the UE can detect that MBMS delivery is no longer available when the related TMGI is removed from MCCH.

-	The UE receives an explicit indication broadcast from the eNB in the MBMS Scheduling Information (see TS 36.300 [9] and TS 36.321 [10]), where it is informed that transmission for the MBMS bearer is going to be, or has been, suspended.

The procedure used by the UE in these scenarios is depicted in figure 5.4-1

Figure 5.4-1: Switching from MBMS Delivery to Unicast Delivery following bearer suspension (and subsequent resumption of MBMS delivery)

1.	The UE has an ongoing group communication.

2.	The UE is receiving downlink data by MBMS Delivery.

3.	E-UTRAN (e.g. after detecting MBMS congestion) decides to suspend one or more MBMS bearer(s) within MBSFN area(s) (based on e.g. the ARP and/or on the counting results for the corresponding MBMS service(s)), and trigger the migration of impacted UEs to receive DL data via unicast, by either:

a) explicitly informing those UEs that the MBMS bearer has been, or is going to be, suspended by broadcasting an indication within MAC MBMS Scheduling Information (and removing the TMGI from the MCCH), or

b)	removing the TMGI of the MBMS bearer that has been suspended from the MCCH.

4.	The UE detects the suspension of the corresponding MBMS bearer service, but continues to monitor for MBMS Delivery (i.e. because while it is establishing unicast there may still be DL data sent on the MBMS bearer).

5.	Over GC1, the UE notifies the GCS AS of the MBMS service suspension.

6.	The GCS AS decides whether to set up the Unicast Delivery path for the downlink data to this UE.

7.	The UE receives DL data by Unicast Delivery and continues to monitor MBMS channels for resumption of the MBMS bearer.

NOTE:	Between step 2 and step 11 data associated with the suspended MBMS bearer continues to be delivered by the GCS AS on the corresponding multicast transport infrastructure towards the E-UTRAN (e.g. because it is still delivered via eMBMS in non congested MBSFN areas). This also allows a quicker resumption of the MBMS service when congestion is over.

8.	At some point, the RAN determines that it can resume the MBMS bearer within the MBSFN area(s), e.g. when the congestion is over. The decision which of the suspended MBMS bearers to resume may be based on e.g. the ARP and/or on the counting results for the corresponding MBMS service(s).

9.	The MCCH indicates that the TMGI is available.

10.	The UE detects the TMGI on the MCCH and prepares to receive it.

11.	The UE is again receiving downlink data by MBMS Delivery.

12.	The UE notifies the GCS AS that it is receiving the group content via MBMS.

13.	GCS AS stops the delivery via unicast.