3GPP TSG SA2 #11		Tdoc S2-000299

Puerto Vallarta, Mexico, 24 - 28 January 2000

Source: 	3GPP TSG SA WG2

Title:	Liaison statement on removal of support for TCP; PDCP sequence numbers

To:	CN 2

Copy:	RAN 3

1)	TCP

S1 has dropped the requirement for X.25 support from release ’99 GPRS/UMTS packet services. As a consequence, support for TCP is no longer required from the GTP protocol.

It is understood that TCP support was the only reason for the R’99 GTP header to contain two bytes of length indication.

As a result of the removal of TCP, N2 may or may not wish to recode the R’99 GTP header. This is a matter for N2.

However, S2 have seen the problems caused by GTP version negotiation and do not want the removal of TCP to be used as a reason for recoding the GTP header in R’2000 (and thus leading to GTPv2).

2)	PDCP

Following requests from RAN2, S2 have changed the type of sequence numbers that are used during SRNC relocation and 2G-3G handover. This will probably have some impact on the R’99 GTP header definition in 29.060.

See S2-00-0226.