SA WG2 Meeting #110AH	 S2-152794

31 Aug- 3 Sept, Sophia Antipolis, France	(revision of S2-152xxx)

Source:	CATT

Title:	Update to Solution 1&2

Document for:	Discussion and Approval

Agenda Item:	6.6

Work Item / Release:	FS_AE_CIoT/ Rel 13

Abstract of the contribution: For those traffic modes defined in Table 4.3-1 other than the Software update/reconfiguration), using GTP-C message over S5/S8 could save resources, such as bearer related signalling during attach, bearer context maintained after attached. It is reasonable to use GTP-C to convey small amount data over S5/S8 for those traffic modes. For Software update/reconfiguration mode, normal EPS attach or on-demand PDN connections / data bearer establishment could be used.                                                                                                         ---------------------------------------------------------------------------------------------------------------------------------------------

It is proposed that the following changes be made to TR 23.720.

************************************ Start of Change ******************************

6.1	Solution 1: Lightweight CN architecture for Narrowband CIoT

6.1.1	Description

Editor's Note:	Describe the solutions. First Sentence should list solution corresponds to which key issue #. Sub-clause(s) may be added to capture details, procedural flow etc.

6.1.1.1	Introduction

NB CIoT RAT (see TR 45.820 [4]) and LTE eMTC (see TR 36.xxx [5]) include the following properties:

- Ultra low UE power consumption

- Large number of devices per cell

- Applied in narrowband spectrum

- Increased coverage

Considering the typical use cases for CIoT devices, the CIoT system only supports reduced and necessary functionalities compared with the existing MME, SGW and PGW:

- Support for efficient small data procedures

- Simplified NAS signalling

- USIM based security

- Authentication and Authorization;

- NAS signalling security

- Simplified mobility management

- Attach, TAU, Detach

- Tracking Area list management;

- UE Reachability in IDLE state (including paging)

- Simplified Bearer management;

- Only GTP-C messages used for IP (or non-IP) data (no need for DRB establishment)

- Lawful Interception

- Paging for coverage enhancements

- Charging and accounting

********************************************next change*********************************

6.1.1.3	Functional Description of new entities

6.1.1.3.1	C-SGN

C-SGN is a new logical entity and can be implemented to support only the necessary functionality required for CIoT use cases.

Namely support for:

- the necessary only part of MM procedures

- efficient small data procedures

- security procedures required only for efficient small data

- if SMS support is needed, SMS on the PS domain using “GPRS like” procedures defined in clause 6.1.1.4.4 [13]

- paging optimisations for coverage enhancements

- termination of SGi interface for the non-roaming case

- support for S8 interface for the roaming case

- support attach procedures for “SMS only” i.e. attach only for sending and receiving SMS, without an additional bearer for IP (or non IP) data

- support tunneling on SGi for non IP data

- support transfering small data over GTP-C on S5/S8 interface;

Editor’s Note: It is FFS if S5 interface is also needed for the non-roaming.

****************************************** next Change ***************************

6.2	Solution 2: Infrequent small data transmission using pre-established NAS security

6.2.1	Description

The solution is based on the Lightweight CN architecture for Narrowband CIoT. Typically, the PGW is only used for roaming case. For non-roaming case, C-SGN can be the termination of SGi interface and C-SGN can send/receive data on SGi directly.

# Procedure for single MO IP packet (and response):

# Figure 6.2.1-1: CIoT Small data transmission for MO

- The mobile already performs Attach activating a default PDN connection.

The finally selected C-SGN makes sure to establish secure exchange of NAS messages, i.e. to use encryption of NAS signalling messages.

The finally selected C-SGN performs P-GW selection taking into account the UE’s roaming status and possible other information, e.g. specific APNs.

The UE requests the UE's AS to establish an RRC connection. A new NAS message format is used to carry the IP packet in an encrypted IE. The UE can also indicate whether acknowledgment/response to the IP packet is expected or not. There is no need to set up the user plane and AS security.

NOTE:	It is RAN decision how to transfer the small data NAS PDU over the radio interface for CIoT RAT.

The unencrypted part of this new NAS PDU carries the "eKSI and Sequence Number" IEs as usual for encrypted NAS messages. The C-SGN uses this, and the S-TMSI, to identify the security context to decrypt the IP packet.

- The CIoT RAN forwards NAS PDU and the indication on whether ack/response is expected to the C-SGN in the initial UE message.

- The C-SGN decrypts the NAS message, obtaining the IP packet.

The C-SGN retrieves the P-GW IP address and TEID, forms the GTP-C packet and sends it to the P-GW.

The P-GW forwards the IP packet on to the SGi interface.

- If no acknowledgment/response to the IP Packet is expected (based on the subscriber information and the indication from the UE), the C-SGN immediately releases the connection. Otherwise, when a (response) IP packet arrives in the P-GW, the P-GW forms the GTP packet and sends it to the C-SGN.

- The C-SGN encrypts the NAS message with the downlink IP packet and sends the downlink NAS transport message to the CIoT-RAN. C-SGN releases the signalling connection after the timer monitoring the connection expires.

- CIoT-RAN sends the Downlink Information Transfer including the NAS message to UE and also releases the RRC connection after the timer monitoring the connection expires.

# Procedure for single MT IP packet delivery

This uses similar concepts to the MO case described above.

CIoT

RAN

C-SGN

UE

P

-

GW

- GTP packet

3b. DL Information Transfer

(

NAS PDU

)

3a. Downlink NAS Transport

(NAS PDU)

2a. paging

2b. paging

2c. service request

4a. UL Information Transfer

(

NAS PDU

)

4b. Uplink NAS Transport

(NAS PDU)

5. GTP Packet

,

# Figure 6.2.1-2: CIoT Small data transmission for MT

- P-GW enclose the received small data packets into a GTP-C message sends the GTP-C message to the C-SGN.

- If there is no signalling connection with the UE, the C-SGN buffers the received IP packet(s), and pages UE. The UE sends the Service Request message to C-SGN.

- The C-SGN then sends the IP packet in an encrypted IE in a NAS PDU in a Downlink NAS Transport message and the CIoT-RAN sends the NAS PDU onto the UE. There is no need to set up the user plane and AS security.

- The UE might send a packet as an acknowledgement that is sent in an encrypted IE in a NAS PDU in an UL Information Transfer message. The CIoT-RAN forwards the NAS PDU to the C-SGN. After the timer monitoring the connection expires, the C-SGN, UE and CIoT-RAN release the connection locally.

- The C-SGN decrypts the IE, adds the GTP header and forwards it to the P-GW.

**************** End of Changes **************