SA WG2 Meeting #110	S2-152275

6-10 May 2015, Dubrovnik, Croatia	(revision of S2-152xxx)

Source:	Motorola Solutions

Title:	Evaluation and update / removal of editorial notes for Solution 4

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	MBMS_enh/ Rel 13

Abstract of the contribution: Provides some clarifying details to remove the editor's notes and provides text for the evaluation of Solution 4 for Key Issue 1.                                                                                            ---------------------------------------------------------------------------------------------------------------------------------------------

It is proposed that the following changes be made to TR 23.741.

**************** First Change **************

# 2	References

[...]

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[3]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[4]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[5]	3GPP TS 36.444: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M3 Application Protocol (M3AP)".

[x]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”

[y]	3GPP TR 36.868: "Evolved Universal Terrestrial Radio Access (E-UTRA); Study on Group Communications for E-UTRA”

**************** End of First Change **************

**************** Second Change **************

## 6.4	Solution 4: Use of MBSFN Area Ids and/or ECGI lists for requesting MBMS service

### 6.4.1	Description

#### 6.4.1.1	Summary of capabilities

This solution provides answers for Key Issue 1.

In this Solution, the network operator configures the MBSFN Areas in the network and then sets each MBSFN Area as a SAI. Therefore each MBSFN Area is a SAI and the MBSFN Area Id is equal (up to a certain number of bits) to the id of the SAI. For example, the two different MBSFN Areas of SAI1 = ‘0000000010101010’ and  SAI2 = ‘1111111110101010’ will end up having the same MBSFN Area Id =’10101010’ broadcast in SIB13. The same SAIs/MBSFN Area Ids are broadcast in SIB 13 and SIB15 (according to their respective formats).  The SAI= MBSFN Area are used to pre-configure the GCS AS and the BM-SC, and they are further used to do the routing down through the MBMS system.

Solution 4 prescribes, that the Application / GCS AS has the option to explicitly request broadcast of content via MBMS in specific MBSFN areas. The Application/GCS AS specifies the exact area of distribution of the MBMS bearer.

-	When a request for MBMS bearer activation occurs, the GCS AS indicates the MBSFN Area Id(s) where MBMS broadcast has to take place.

-	To request transparent service, the GCS AS may indicate both a MBSFN Area Id and the ECGI list.

-	Pre-configuration of MBSFN Areas (= SAIs,) in the GCS AS is proposed.

-	The UE can report relevant parameters associated to the serving cell (e.g. the ECGI, the list of MBSFN Areas to which the cell belongs) from SIBs. This information can be used in case new cells are deployed after pre-configuration, to avoid the need for reconfiguration in most cases.

#### 6.4.1.2	Procedures

##### 6.4.1.2.1	Procedure using MBSFN Area Id only

Other than changes to the broadcast area parameter and updates provided by the UE on GC1, there are no changes (from Rel-12) to the messages on the MB2-C, SGmb, Sm and M3 interfaces. M1 and M2 interfaces are not affected. The impacted procedures are Activate MBMS Bearer and Modify MBMS Bearer. The impact is the same on both of them, namely the change in the broadcast area parameter carried by the messages. As a prerequisite of this solution, MBSFN areas map one-to-one to service areas, i.e. service areas are the same as MBSFN areas.

BM-SC

GCS AS

MBMS- GW

MME

MCE / eNB

UE

1. M3 Setup (SAI= MBSFN Area Id)

3. Update MBSFN Area (optional)

2. UE reports current cell id and MBSFN Areas Ids = SAIs from SIB to GCS AS via GC1

4. MB2: Activate MBMS Bearer Req (..., bcast area= MBSFN Area Id (=SAI), ...)

5. Session Start Req / Resp (..., bcast area = MBSFN Area Id (=SAI), ...) unchanged

6. MB2: Activate MBMS Bearer Response

7. User Plane Traffic

Figure 6.4.1.2.1-1: MBMS bearer initiation using MBSFN Area Id

1.	In Rel-12, the MCE(s) report(s) the list of SAIs to the MMEs. In this proposal, the list of MBSFN Area Ids served by the MCE(s) is reported instead and treated by MCE as SAIs.

2.	As UEs move in the cell or power on in specific cells and acquire the specific system information, the UE will report to the serving GCS AS via GC1 information from the SIB e.g. the identity of the serving cell (from SIB1) and of the MBSFN areas to which the cell belongs (from SIB13), configuration from SIB2, SAIs from SIB15, if needed.

3.	Optionally, the information reported by the UE can be used by the GCS AS to update the MBSFN Areas pre-configured at the GCS AS, in case a new cell is added.

4.	The GCS AS chooses the entire and exact area of distribution for the MBMS bearer by identifying an MBSFN Area Id and sending an Activate MBMS Bearer Request with the broadcast parameter set to the chosen MBSFN Area Id.

NOTE: In most cases, a GCS AS handles only local areas and the 8-bit MBSFN Area Id that it provides to the BM-SC is sufficient to uniquely identify the desired local area of service. In rare cases when an ambiguity might arise, the GCS AS has the option to use the 16 bit SAI id corresponding to the desired 8-bit MBSFN Area Id. In this case, the BM-SC will identify the desired area by the received SAI id, then will mask the 8 MSBs to obtain the MBSFN Area Id.

5.	The BM-SC starts the sequence of messages with the Session Start Request message, with the MBSFN Area Id = SAI as broadcast area parameter. The messages are routed as in Rel-12, based on same type of pointers used for SAIs, but now corresponding to the MBSFN Area Id.

6.	The BM-SC responds to the GCS AS with a Session Start Response message

7.	The GCS AS starts transmitting user plane traffic on the newly activated MBMS bearer.

The MBMS Bearer Modification procedure works similarly (the GCS AS sends the new MBSFN Area Id to the BM-SC) except that it uses the Session Update procedure of TS 23.246 [2] in lieu of the Session Start procedure.

##### 6.4.1.2.2	Procedures using MBSFN Area Id and ECGI list together for transparent mode

It is the responsibility of the GCS AS to map the cells reported by the UEs on GC1 to a MBSFN Area Id. Both the MBSFN Area Id and the ECGI list are sent as broadcast parameters. The BM-SC checks if all the cells in the ECGI list are also included in the MBSFN Area Id, and potentially, eliminates those that are not from the ECGI list. Then the messages are routed based on MBSFN Area Id. The information flow in Figure 6.4.1.2.1-1 applies, with the “bcast area” parameter set to both MBSFN Area Id and the ECGI list. The MCE decides how to allocate optimal MBMS bearer resources based on MBSFN Area or the cells specified in the ECGI list.

### 6.4.2		Impacts on existing nodes

GCS AS:

-	The GCS AS needs to be configured with the definitions of the MBSFN Areas = SAIs for each MBMS system (i.e. per BM-SC).

BM-SC:

-	The BM-SC needs to be configured with the definitions of the MBSFN Areas = SAIs rather than with just SAIs.

MBMS-GW:

-	None identified.

MME/MCE:

-	The M3 Setup message will carry the MBSFN Areas = SAIs, in lieu of just SAIs. An MBSFN Area derived SAI can be treated just as a SAI, which is covered in Rel-12, thus no functional impact.

eNB:

-	Broadcasts the SAIs = MBSFNs in the SIB.

UE:

-	UE reports location information e.g. ECGI, MBMS SAI from SIBs to GCS AS.

There is no impact to MBMS service continuity for this solution, as the SAIs broadcast in SIB15 and the MBSFN broadcast in SIB13 are used without changes. UEs that are in RRC IDLE state will proceed as described in 5.2.4.1 of TS 36.304 [x], for the selection of the frequency layer and cell reselection. Afterwards, the UEs will temporarily connect to the GCS AS on GC1, to receive an update and further instructions as described in 5.2.3.1 of TR 36.868 [y]. UEs that are in RRC CONNECTED state are supposed to have registered with the network their TMGIs of interest and the preference to prioritize unicast over MBMS. Subsequent to a handover they too will proceed as described in 5.2.3.1 of TR 36.868 [y].

### 6.4.3	Solution evaluation

This solution is optimized for Mission Critical and Public Safety systems. It meets all the Architectural requirements from 5.1.2. The solution preserves the Rel-12 architectural and functional choices: the decision on the MBMS area of service continues to be made by the GCS AS.  The impacts on the MBMS network entities are minimal.

The solution enables Local Control of the application in the sense that the exact MBMS areas of service are selected by the application prior to the activation of the bearer and known exactly afterwards. A choice between multi-cell MBSFN and SC-PTM bearers can be expressed by the application.

When the UE reports its current serving cell, the GCS AS can update its pre-configured data, thus avoiding the need for re-configurations in many cases when new cells are added.

Use of the MBSFN areas allows for the minimum processing in matching the areas to a given list of cells and a minimal amount of routing.

The solution relies on the network operator defining the SAIs in the PLMN the same as the MBSFN Areas, but once that is achieved and the system is (re)configured , the system can be used for non-Public Safety applications.

**************** End of Second Change **************