3GPP TSG-SA2 Meeting #113	S2-160249

# Saint Kitts, Saint Kitts And Nevis, 25th Jan 2016 - 29th Jan 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 2961 | rev | - | Current version: | 13.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of support for DECOR in shared networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TeliaSonera AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DECOR |  |  |  |  |  | Date: |  |  | 2016-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | RAN3 has defined the stage 3 details of how DECOR and MOCN interact. This CR introduces the assumptions made by RAN3 in stage 2 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text is added to Section 4.3.25.3 to align the stage 2 description of DECOR in MOCN shared networks. It is clarified that if Selected PLMN information is not provided by the UE, MOCN rerouting first selects an operator that can serve the UE and after that the operator may initiate DECOR rerouting, if needed. After DECOR rerouting, the serving node in the DCN ends the MOCN rerouting procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Assumptions made in stage 3 descriptions of DECOR are not documented in the specifications and may cause interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR reflects the content of the LS from RAN3 in S2-160045. |  |  |  |  |  |  |  |  |

#### 4.3.25.3	Considerations for Network Sharing

If the network supports the MOCN configuration for network sharing (see TS 23.251 [24]), each network sharing operator has separate CN(s). Mechanisms for selection of serving operator for supporting and non-supporting UEs are defined in TS 23.251 [24]. Each of the sharing operators may deploy one or more DCNs.

If Selected PLMN information is provided by the UE, the RAN selects the CN operator based on this provided information and then DECOR rerouting may, if needed, be initiated within the CN of the selected operator. If Selected PLMN information is not provided by the UE, the network initiates MOCN redirection, including CS/PS coordination, to select a CN operator that can serve the UE. After this, DECOR rerouting is initiated if needed. The serving node in the selected DCN ends the MOCN redirection. The functions for redirecting or maintaining UEs in specific DCNs are configured to work within the CNs of the same operator.