# SA WG2 Meeting #S2-115	S2-162991

# 23 - 27 May 2016, Nanjing, P.R. China

# 3GPP TSG CT WG 1 Meeting 98	TDoc C1-162927

Osaka (Japan), 23-27 May 2016

Title:	LS on APN rate control

Response to:	-

Release:	Rel-13

Work Item:	CIoT-CT

Source:	3GPP CT WG1

To:	3GPP SA WG2

Cc:	3GPP CT WG4

Contact person:	Christian Herrero-Veron

Christian dot Herrero at Huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 have discussed the APN rate control under the CIoT work considering the SA2 CR#2999 to TS 23.401 in S2-162710 and want to provide the following information to SA2:

- CT1 noticed that for the serving PLMN rate control only uplink is applied by the UE, and therefore CT1 consider that for APN rate control also only uplink is applied by the UE so downlink APN control should not be provided to the UE as the UE cannot apply downlink APN control. Hence, CT1 propose to remove the following sentence, quote:

The PDN GW or SCEF may also inform the UE of any downlink APN Rate Control that it intends to enforce in the PDN GW/SCEF.

- CT1 further noticed that the UE context in the CR mentioned above is updated as follows:

| Serving PLMN-Rate-Control | The Serving PLMN-Rate-Control limits the maximum number of NAS Data PDUs per deci hour sent per direction (uplink/downlink) using the Control Plane CIoT EPS Optimisation for a UE. |
| --- | --- |

CT1 believe that downlink is not applied by the UE for serving PLMN rate control as described in the new sub-clause 4.7.7.2 on Serving PLMN rate control of the SA2 CR, so CT1 request to remove that word from the UE context.

- CT1 have decided that the NAS does not enforce APN rate control for user data transported over control plane or user plane.

# 2	Actions

To 3GPP SA WG2:

ACTION: CT1 kindly ask SA2 to consider the information provided above in order to update their CR#2999.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 99	25-29 July 2016	Tenerife, Spain, EU

TSG CT WG1 Meeting 100	17-21 October 2016	Guilin, P.R. of China