SA WG2 Meeting #115	S2-163171

23 - 27 May 2016, Nanjing, P.R. China	(revision of S2-163020, S2-162687, S2-160714)

Source:	Deutsche Telekom AG

Title:	Solution for Key Issue 1 and 3

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_SeDoc / Release 14

Abstract of the contribution: This document proposes potential solution to SeDoC key Issue 1 and 3..

# Discussion

This solution targets key issue 1, i.e. transition to a single service profile in IMS only and key issue 3..

# Proposal

It is proposed to add the following solution to TR 23.719

* * * Start of change 1 * * * *

## 6.X	Solution X: MSC Server enhanced for SeDoC

### 6.X.1	Overview

This solution addresses key issue #1 and #3.

The basic principles of this solution are that 2G/3G CS access is mapped to IMS and PS access uses “native” IMS (VoLTE) but there is a single service domain for both accesses in IMS. The user profile is solely provided by IMS domain.

Figure 6.X.1.1Services for PS access and CS access in HPLMN IMS

For the migration phase the SeDoC enabled network operator is required to provide legacy support for inbound roamers as shown in figure 2.

Figure 6.X.1.2 Services for Inbound roamers provided in VPLMN IMS

Long term target is that the service for inbound roamer be provided by the HPLMN, i.e. the VPLMN IMS-GWs (IBCFs) would relay towards home IMS service infrastructure as shown in figure 3.

Figure 6.X.1.3 Services for Inbound roamers provided in HPLMN IMS.

### 6.X.2  SeDoC Architecture

The basic principles described in previous section require the introduction of a CS access to IMS gateway function (MSC Server enhanced for SeDoC ) and an interworking fucntion (ICS IWF) to the SeDoC architecture. The MSC Server enhanced for SeDoC maps GSM Access procedures to IMS procedures but does not store User Profile Data, i.e. the VLR functionality is extracted and is placed for own users into the HSS and for inbound roamer into an ICS-IWF. By doing this the internal IMS architecture does not need to introduce new interfaces.

Note that a similar architecture was already discussed in 3GPP during the development of TR23.892 [7]. It should be noted that the way the architecture and protocols are specified, the functionality of the gateway can be extracted from an MSC Server and deployed as a separate network element.

For the migration phase the SeDoC enabled network operator is required to provide legacy support for inbound roamers. The architecture proposed in this solution is for the SeDoC enabled visited network to emulate a VLR and allocate to the inbound CS roaming subscriber a temporary IMS identity so that they can also be served by the IMS domain.

Figure 6.X.2.1 SeDoC functional architecture

Note does only show the principle architecture, i.e.it does not show all interfaces.

#### 6.X.2.1 Reference points

##### 6.X.3.2	Reference Point MSC Server enhanced for SeDoC - CSCF (I2’ Reference Point)

The I2’ reference point is based on I2 functionality similar to the procedures of P-CSCF – X-CSCF communication. It is used to route session control signalling between the MSC Server enhanced for SeDoC and the IMS, as defined in TS 23.292 [3] and in TS 23.237 [12].

I2’ is used to support the following features:

-	An MSC Server enhanced for ICS and SRVCC, as defined in TS 23.292 [3], TS 23.216 [13] and TS 23.237 [12].

##### 6.X.3.3	Reference Point MSC Server enhanced for SeDoC - TAS (I3’ Reference Point)

The I3’ is based on I3(Ut) functionality similar to the procedures of P-CSCF – AS communication reference point. It is used between the MSC Server enhanced for ICS and the TAS to interwork CS signalling and communication service setting procedures, as defined in TS 23.292 [3].

##### 6.X.3.4	Reference Point ICS-IWF – CSCF (Cx’ Reference Point)

The Cx’ reference point is based on Cx functionality similar to the procedures of S-CSCF – HSS communication. It supports information transfer between CSCF and HSS.

The main procedures that require information transfer between CSCF and HSS are

1)	Procedures related to Serving CSCF assignment

2)	Procedures related to routing information retrieval from HSS to CSCF

3)	Procedures related to authorisation (e.g., checking of roaming agreement)

4)	Procedures related to authentication: transfer of security parameters of the subscriber between HSS and CSCF

5)	Procedures related to filter control: transfer of filter parameters of the subscriber from HSS to CSCF

Further information on the Cx reference point is provided in TS 23.228 [8].

#### 6.X.2.2  MSC Server enhanced for SeDoC Features

The MSC Server enhanced for SeDoC terminates CS Access and maps it to respective IMS procedures.

At high level the MSC Server enhanced for SeDoC performs the following functionality on the control plane:

- towards the UE

  - terminates the A (GERAN) and Iu-cs interfaces (UTRAN) when the UE accesses via a CS RAT;

- towards the IP Multimedia Subsystem

  - Terminates the I2 interface to the x-CSCF

  - Terminates I3 to the Telephony Application Server

- towards legacy networks

  - terminates the G and E interface towards MSC Servers;

  - terminates ISUP signalling towards PSTN.

On the media plane the MSC Server enhanced for SeDoC controls Mc to the Media gateway.

A graphical representation of the MSC Server enhanced for SeDoC is shown in Figure 1.

#### Main differences to MSC Server enhanced for ICS

The MSC Server enhanced for SeDoC is based on the MSC Server enhanced for ICS, associated MGW as defined in TS 23.292 [3], TS 23.216 [13] and TS 23.237 [12] with the following amendments.

Call control solely done in IMS

Call Control (CC) functionality is essentially required only for the A/Iu-cs side of the MSC Server enhanced for SeDoC. Some local call control i.e.to treat “non UE detected emergency call” (Germany: 110) as emergency call and. route to E-CSCF is required. Remaining Call Control functionaliy is done by IMS in the same way as for PS calls. Therefore the MSC Server enhanced for SeDoC behaves as P-CSCF/ATCF from viewpoint of I/S/E-CSCF, SCC-AS and MSC.

Radio Resource Management (RR) and Mobility Management (MM) remain unchanged.

No VLR functionality

MSC Server enhanced for SeDoC does not include VLR functionality, i.e. there is also no D-Interface to the HLR. User Service Profile data and user service profile (i.e. identity, supplementary services) are handled in the IMS domain only. MSC Server enhanced for SeDoC just need to store user data related to radio access and RR/MM/CC procedures (e.g. IMSI, IMEI, TMSI, ....) .

#### Mapping of CS procedures to IMS procedures

The MSC Server enhanced for SeDoC terminates CS Access and maps it to respective IMS procedures.

##### Call Control procedures

Call Control procedures from the mobile CS user interface (A, Iu) are mapped towards IMS interface, i.e. I2 (SIP) towards IMS core in order to support the mapping of

  - MOC & MTC based on A/Iu to respective IMS procedures

  - SMS-MO & SMS-MT based on A/Iu to respective IMS/IP-SM procedures

##### Supplementary Services Management procedures

Supplementary Services Management procedures from the mobile CS user interface (A, Iu) are mapped towards IMS interfaces, i.e. I3-IF (Ut, XCAP) towards IMS MMTEL AS.

For Inbound roamers the SS Management procedures are mapped as depicted below.

##### Mobility and Authentication procedures

Basic Mobility and Authentication procedures from the mobile CS user interface (A, Iu) are mapped to appropriate procedures towards IMS core (CS network attach (LUP) / IMS registration; Authentication).  For Authentication the MSC Server enhanced for SeDoC processes the user-network signalling received over the CS access  for interworking with IMS SIP and vice versa, especially the authentication vector mapping.. For details refer to solution 5.

#### Mapping of IMS procedures to CS procedures

IMS specific procedures that need to be mapped to appropriate GSM procedures are:

  - early media (uni/bi-directional through-connect of media path)

  - multiple early dialogues (due to forking on remote side (e.g. MSIM scenario)

    - i.e. MSC Server enhanced for SeDoC MSC Server enhanced for SeDoC  serving the originating user needs to map multiple early SIP dialogues to one GSM dialogue towards the GSM device

SRVCC is supported as specified for GSM MSC [13], i.e. via Sv Interface to MME and via I2 Interface to ATCF.  Reverse SRVCC is supported as specified for GSM MSC [13]

CSFB support for Voice and SMS over SGs interface to MME, in case of combined attach to EPC and CS (via SGs), the MSC Server enhanced for SeDoC needs to interact with the IMS core similar as for CS attached voice terminals (e.g. map Location Update to IMS Registration, ....)

#### Procedures which can be handled without IMS interaction

Various MSC-Server procedures which can be handled without IMS interaction remain within the MSC Server enhanced for SeDoC. This includes e.g. handover to/from other MSC Server enhanced for SeDoC or VMSC (original MSC Server enhanced for SeDoC remains anchor after handover).

#### SMS support considerations

In order to properly support SMS a SIP based replacement for MAP based SMS alerting procedures (reachability check) is needed, i.e. to set “not reachable” flag in case of unsuccessful SMS-MT and re-register if user is reachable again.

Upon unsuccessful SMS-MT delivery (no response from terminal) the MSC Server enhanced for SeDoC needs to activate a “Not Reachable” (NR) flag (similar to GSM MSC). In case of “NR flag = TRUE” and receipt of any message from the terminal, the MSC Server enhanced for SeDoC needs to re-register towards IMS-Core.

In case of unsuccessful SMS-MT to an attached user, the IP-SM-GW shall not initiate supervision in packet core network (Sh request to HSS).
After unsuccessful SMS-MT delivery, the IP-SM-GW sets a “Not Reachable” flag. Due to this flag a re-registration (and other messages received from the terminal) will trigger the standard retry mechanisms.

#### Location

MSC Server enhanced for SeDoC provides with PANI header the network provided location information according to cell-id received from radio access network (RAN). This information may be used by applications to differentiate between 2/3G Access via MSC Server enhanced for SeDoC and 4G Access via P-CSCF.

#### CS Data support

CS Data is used analogously to a voice call: A data terminal equipment (modem) establishes a connection to another terminal (modem). The connection setup is controlled by the IMS Core, the data stream can be transported transparently with already available codecs (e.g., “clear mode”/G7.11)

#### 6.X.2.3  MSC Server enhanced for SeDoC Call Flows

#### Editors Note: Originating and terminating call flows to be provided.

### 6.X.3  SeDoCArchitecture for Inbound Roamers

For the migration phase the SeDoC enabled network operator is required to provide legacy support for inbound roamers. The architecture proposed in this solution is for the SeDoC enabled visited network to emulate a VLR and allocate to the inbound CS roaming subscriber a temporary IMS identity so that they can also be served by the IMS domain.

The detailed architecture diagram is depicted in Figure 6.X.3.1:

Figure 6.X.3.1 SeDoC Architecture for inbound roamer support

#### 6.X.3.1 IMS Centralized Services Interworking Function (ICS-IWF) Features

The ICS-IWF is necessary to support roaming sceanrios. Therefore it acts as a

- HSS towards the I-CSCF and S-CSCF in the VPLMN and as a VLR towards the HPLMN HLR and is mainly necessary to support roaming sceanrios. It performs the following functionality:

  - Towards the I-CSCF

    - terminates the Cx’ interface

  - Towards the S-CSCF

    - terminates the Cx’ interface

  - Towards HPLMN HLR for inbound roamer during migration phase

    - Terminates the C interface

The ICS-IWF provides the several roles:

- role as VLR towards HPLMN HLR, i.e. in course of registration the ICS-IWF retrieves the user profile from HPLMN and creates a temporary user profile. The temporary user profile may be updated due to MAP ISD or deleted due to MAP Cancel or IMS de-registration or IMS registration time out.

- role as HSS towards the S-CSCF in VPLMN for local IMS registration.

- role as HLR towards VLR.

- role as AS towards HSS in VPLMN for 3rd party IMS registration to enable terminating session routing to the VPLMN.

#### 6.X.3.2  Supplementary Services Management for Inbound roamers

For Inbound roamers the SS Management procedures are mapped as depicted below.

#### 6.X.3.3  Authentication for Inbound roamers

For details on Authentication and Registration of an Inbound Roamer by the SeDoC architecture please refer to solution X.

UE

CS-GW

CSCF

HSS FE

AS

I3’

Mw’

ISC

Sh

A, IuCS

Cx

MGW

A, IuCS

VLR - FE

Cx’

UDR

Ud

Ud

### 6.X.4 Impacts on existing nodes and functionality

GSM HLR

- obsolete for domestic use (after migration phase),

- no impact for outbound roaming

HSS

- No impact

Legacy GSM MSC, MSC-Server

- replaced by MSC Server enhanced for SeDoC (after migration phase)

CS-MGW

- no impact

I/S/E-CSCF, BGCF

- Support of new use cases:

  - re-registration for SeDoC user without authentication

MGCF

- no impact

MMTEL AS

- Editor’s Note: impact to MMTEL is FFS

### 6.X.5 Solution Evaluation

Editor’s Note: Evaluation of this solution is FFS.

* * * End of change 1 * * *,

* * *Start of change 2 * * * *

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.101: "Service aspects; Service principles ".

[3]	3GPP TS 23.292 “Multimedia Subsystem (IMS) centralized services”.

[4]	3GPP TS 24.292 “IP Multimedia (IM) Core Network (CN) subsystem Centralized Services (ICS)”.

[4]	GSMA PRD IR.64 “IMS Service Centralization and Continuity Guidelines”.

[5]	3GPP TS 24.008 “Mobile radio interface Layer 3 specification”.

[6]	3GPP TR 23.883: “Study on enhancements to IMS Centralized Services (ICS)”.

[7]	3GPP TR 23.892: “ IP Multimedia Subsystem (IMS) centralized services”.

[8]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[9]	3GPP TR 23.002: “Network architecture”.

[10]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[11]	3GPP TS 29.335: "User Data Convergence (UDC); User Data Repository Access Protocol over the Ud interface; Stage 3".

[12]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity".

[13]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[14]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

* * * End of change 2 * * *

* * * Start of change 3 * * * *