3GPP TSG SA2 Meeting #116bis		 	S2- 164314

Sanya, China, 29th August – 2nd September 2016

Agenda item:       6.10.0 Input for TR 23.799 clauses 1-4 and new proposed key issue

Source:	Thales, Fraunhofer IIS, DISH Network, Inmarsat, SES S.A., Hughes Network System Ltd, Eutelsat S.A.

Title:	Integration of satellite and other non-terrestrial components in the Next Generation system architecture

Document for:     Discussion and approval

Work Item / Release:     NextGen/ Rel-14

# 1	Abstract

This contribution proposes the addition of a new key issue in TR 23.799 “Study on Architecture for Next Generation System”. The objective is to address the integration of non-terrestrial (Satellite, High-Altitude platforms) components in the architecture of Next Generation Network.

# 2	Discussion

As 3GPP progresses its work towards the development of the next generation system, this document aims at proposing elements of architecture to seamlessly integrate satellite and other non-terrestrial components as part of the 5G ecosystem to enable:

- Broadband access everywhere;

- Global service continuity especially for selected machine type communications and critical communications;

- Optimised network operations through off-loading.

Such an integration of satellite and other non-terrestrial networks into the Next generation network would ultimately seamlessly contribute to the provision of a true ubiquitous coverage.

The integration of satellite and other non-terrestrial components in next generation system architecture relates to

- the “5G Connectivity Using Satellites” use case of 3GPP TR 22.891 [1]  within 5G-SMARTER

- the “Satellite extension to Terrestrial” deployment scenario of 3GPP TR 38.913 [2]

Moreover, the 3GPP technical reports defining the New Services and Markets Technology Enablers for the Next generation system have identified the added value of integrating satellite and other non-terrestrial components in the next generation system:

| 3GPP documents | Next Gen use case families where satellite and other non-terrestrial networks are expected to play a role |
| --- | --- |
| 3GPP TR 22.862 [4] | “Higher availability” and “Mission critical services” |
| 3GPP TR 22.863 [5] | “Deployment and Coverage” |
| 3GPP TR 22.864 [6] | “Access” To this, one can add “Backhaul” since satellite backhaul solutions already exist for 2G to 4G networks |
| 3GPP TR 22.861 [3] | “Service continuity for Machine Type Communication” |

To achieve the objectives as described in the previous paragraph, the architecture of the next generation system has to be defined with the aim at seamlessly integrating satellite and other non-terrestrial network components.

Non-terrestrial components refer to networks, or segments of networks, based on airborne or spaceborne transmission equipment as defined hereafter:

- Spaceborne platforms: Satellites;

- Airborne platforms: High Altitude Platform (HAPS).

# 3	Conclusions

This contribution emphasizes the need for the next generation system to integrate non-terrestrial components to support a variety of use case families and services for the Next Generation system, as identified in technical reports developed within 3GPP SA and RAN groups.

Based on the above discussion, we propose to add a new key issue “integration of satellite and other non-terrestrial components” in the TR 23.799 and in subsequent related work items.

# 4	References

[1] 	3GPP TR 22.891 V1.3.2 (2016-02), Technical Specification Group Services and System Aspects; Feasibility Study on New Services and Markets Technology Enablers; Stage 1 (Release 14)

[2]	3GPP TR 38.913 V0.4.0 (2016-07), Technical Specification Group Radio Access Network; Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 14)

[3]	3GPP TR 22.861: 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Feasibility Study on New Services and Markets Technology Enablers - Massive Internet of Things; Stage 1 (Release 14)

[4]	3GPP TR 22.862: 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Feasibility Study on New Services and Markets Technology Enablers - Critical Communications; Stage 1 (Release 14)

[5]	3GPP TR 22.863: 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Feasibility Study on New Services and Markets Technology Enablers - Enhanced Mobile Broadband; Stage 1 (Release 14)

[6]	3GPP TR 22.864: 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Feasibility Study on New Services and Markets Technology Enablers - Network Operation; Stage 1 (Release 14)

[7]	3GPP RP-161100: pCR to update TR38.913 to include the “Satellite extension to terrestrial” use case, accepted at RAN plenary#72.

# 5	Proposal for approval

It is proposed to add the following text to the TR 23.799 “Study on Architecture for Next Generation System”.

* * * Start of changes * * * * (all new text)

## 5.22 Key Issue 22: Integration of satellite and other non-terrestrial components”

### 5.22.1 Description

Introduction

Thanks to their inherent wide coverage capability and reliability, satellite networks will support a wide range of services and use case families for the next generation systems, as identified in [4] to [7].

The architecture of the Next Gen system shall support the seamless integration of satellite or other non-terrestrial components through several deployment scenarios to enable:

- Broadband access everywhere especially with service extension in low population density regions with low ARPU (Average Revenue Per User), in underserved areas, on board public and private transport moving platforms (e.g. aircrafts, vessels, trains, bus, etc.) through backhaul or distributed core;

- The Global service continuity especially for machine type communications (MTC), public safety communications, emergency communications and Public warning/emergency alert systems;

- Terrestrial network off-loading which includes the following use cases:

  - a) Cell sites in underserved areas which are connected with low bandwidth backhaul links (e.g. microwave) can be easily upgraded by backhauling the latency insensitive downstream traffic over high throughput satellite links. The large coverage area of satellite beams allows for greater statistical multiplexing gains. Moreover, the genuine satellite broadcast and multicast capabilities enable bandwidth efficient live video streaming and content distribution to edge caches;

  - b) When a satellite link is used for backhaul from a cell site the traffic normally first lands at satellite Gateway (which could be in a different country) before being forwarded to the core network. Traffic destined to and from the Internet could be directly offloaded to the Internet at the satellite gateway rather than being sent back to the core network.

By extension, the Next Generation system architecture shall be defined to also support the integration of non-terrestrial components and technologies at large.

Hence, the Next Generation system will contribute to the development of an Heterogeneous Network, that can provide an access or core network through a combination of different wireless (GSM, UMTS, LTE, NextGen, WiFi, satellite, HAPS) or wireline components (copper, fibre).

The above mentioned deployment scenarios will impact the NextGen System architecture (see figure 4.2.1-1) as follow:

Figure 5.22.1-1: Broadband access everywhere enabled by non-terrestrial backhaul

Note that edge network computing/caching can be implemented in this deployment scenario.

Figure 5.22.1-2: Broadband access everywhere with distributed core enabled by non-terrestrial network

Figure 5.22.1-3: Global service continuity through non-terrestrial RAN

Note that mobile broadcast feature can be implemented in this deployment scenario.

Figure 5.22.1-4: Traffic offloading in underserved areas with multi-link backhaul

Figure 5.22.1-5: Traffic offloading with multi country backhaul network

Note that NGx-represent the Next Generation interfaces

Non-terrestrial components

Non-terrestrial components refer to elements of networks based on transmission equipment embarked on board space borne platforms such as Satellites or airborne platforms such as High Altitude Platform Systems (HAPS).

Table 5.22.1-1 provides the main characteristics of the different types of non-terrestrial networks driving their latency:

Table 5.22.1-1: Typical one way propagation delay in non-terrestrial networks (earth station to Earth stations via the Space borne / airborne platform)

| Space borne / airborne platforms | Max altitude | Max distance between earth station and platform assuming 10° elevation | One way propagation delay |
| --- | --- | --- | --- |
| Geostationary orbit Satellite (GEO) | 35 786 km | 40 588 km | 270.6 ms |
| Highly elliptical orbit Satellite (HEO) | 42 708 km | 47 575 km | 317.2 ms |
| Medium earth orbit Satellite (MEO) | 8 500 km | 12 380 km | 82.5 ms |
| Low earth orbit Satellite (LEO) | 1 400 km | 3 480 km | 23.2 ms |
| High Altitude Platform System (HAPS) | 20 km | 110 km | 0.7 ms |

Geostationary (GEO) satellites orbit the Earth at the same speed as the Earth’s rotation, and thus appear to be stationary, as they are placed into orbit around 35,786 km in the Equatorial plane. Other satellite networks are based on constellations of satellites placed in Medium Earth Orbit (MEO) from 6000 to 25000 km altitude and Low Earth Orbit (LEO) below 1800 km altitude. Highly-Elliptical Orbit (HEO) satellites follow elliptical orbits typically with an inclination of 63.4 degrees to the Equator such as to be synchronized to the rotation of the Earth.

The most relevant satellite networks for the Next Generation system are summarized in the next table 5.22.1-2:

Table 5.22.1-2: Satellite and other non-terrestrial networks use cases for the Next Generation system

| Use case | Targeted terminals | Typical Frequency bands | System Bandwidth (DL + UL) | Data rate capability |
| --- | --- | --- | --- | --- |
| Radio Access network | Devices equiped with quasi- omni directional antenna and battery activated | Below 6 GHz (e.g. ~1.5, ~2, ~2.5, 3.5 GHz) | Up to 2x 10 MHz | Up to hundreds of kbps |
| Backhaul | Earth stations fixed or mounted on moving platforms | Above 6 GHz (e.g. ~10, ~20, ~30, ~40, ~50 GHz) | Up to 2x 1000 MHz, depending on frequency | Up to hundreds of Mbps |
| Traffic off loading | Earth stations fixed or mounted on moving platforms | Above 6 GHz (e.g. ~10, ~20, ~30, ~40, ~50 GHz) | Up to 2x 1000 MHz, depending on frequency | Up to few Gbps |

The propagation channel can be modelled mainly with

- a Rician Log-Normal model with 2 or 3 states (representing the Line of Sight and shadowed conditions) for Sub 6 GHz mobile satellite systems serving user equipment with quasi-omni directional antenna;

- a combined Gaussian model with rain fading also subject to transponder non linearities, for satellite systems operating in frequency bands above 6 GHz and addressing fixed directive antennas or directive antennas mounted on moving platforms.

Architectural challenges related to the integration of non-terrestrial networks

The following key objectives have to be addressed:

- The seamless integration of satellite and other non-terrestrial components into the Next Gen system architecture.

- The support of the Next Gen system for non-terrestrial network comprising 3GPP and non-3GPP waveforms and protocols.

Topics to be addressed when defining the Next Generation system architecture

- Flexibility of the end to end network/transport/application protocols (UE – CN) (control and user plane) to accommodate high latency;

- Define the flexibility of the interface between the Core Network and the non-terrestrial components;

- Integration of non-terrestrial link(s) into link aggregation scheme, so as to allow the aggregation of non-terrestrial and terrestrial links at the same time in the User Equipment, for backhaul or distributed core;

- Partitioning of next generation network functions including space-borne or airborne on-board processing capabilities with clear interfacing requirements;

- Power dynamic and control, Doppler and timing ranges adequately defined so as to uniquely integrate terrestrial and non-terrestrial components in a single air interface;

- Unified radio air interface frequency channelling and duplexing definition for terrestrial and non-terrestrial networks to enable the implementation of efficient solutions in accessing to the spectrum;

- Flexibility of the radio interface protocols (UE – RAN) (control and user plane) to accommodate high latency;

### 5.22.2 Work Tasks

Table 5.22.2-1: Work Tasks for Integration of non-terrestrial networks

| Work Task ID | Work Task(s) | Work Task Description |
| --- | --- | --- |
| NTNI_WT_#1 | Non-terrestrial networks integration | Support of NextGen radio interface protocols and potentially waveforms in non-terrestrial networks Support of user and control planes of the NG2 and NG3 interfaces over non-terrestrial network (e.g. backhaul) Support of management plane to interface with the non-terrestrial networks to configure and manage the ground segment and non-terrestrial networks components connectivity Support of simultaneous link aggregation of non-terrestrial and terrestrial links in the User Equipment, for backhaul or distributed core Support mobility between terrestrial and non-terrestrial radio access networks Support traffic routing between terrestrial and non-terrestrial components for RAN & CN. |

* * * End of Changes * * * *