SA WG2 Meeting #116bis	S2-164391

29 August – 02 September 2016, Sanya, China

Source:	China Mobile

Title:	Function description of the node-level tunneling UP model

Document for:	Approval

Agenda Item:	6.10.4

Work Item / Release:	FS_NextGen/R-14

Abstract of the contribution: This contribution is about how UE access to AN node-level tunnel and how the data traffic is transferred between AN node and UP function.

1. Discussion

This proposal describes the function and the procedures of the network support Node-level tunnel used for UP protocol model.

The solution is that: When a UE attaches to the network or sets up a PDU session, the CP authorizes the UE type (e.g., a type of fixed wireless UE) and identification related information to check whether AN node level tunnel applies. If so, the CP will select an existing tunnel for the PDU session on information e.g., AN ID, UE type. The tunnel end point information (e.g., IP addresses, tunnel IDs) may also be provided by AN to the CP.

2. Proposal

It is proposed to add following text to the TR 23.799 v0.7.0.

* * * First changes * * * *

6.4.11	Solution 4.11:  UP protocol model - Per Node-level tunnel

6.4.11.1	Architecture description

This solution addresses the "UP protocol model" of the SM_WT_#1 SM Model.

In this option there is a common tunnel for all traffic between each pair of NFs e.g. between a RAN node and a UP function in the CN or between two UP functions in the CN.

This solution has the following additional properties:

-	There is no identification of the PDU Session within the outer IP header or the encapsulation header. Instead the endpoint needs to use information in the end-user PDU to identify the session, e.g. the UE IP address in case of PDU type IP.

-	For PDU type IP, the PDU session traffic is identified based on UE IP address. This requires that UE IP addresses are unique to allow unambiguous traffic identification.

-	The encapsulation header may or may not be needed, e.g. to carry an identifier for QoS purposes.

-	In case a node/function supports multiple IP addresses there may be a need to signal the tunnel endpoint addresses in order to direct the traffic to the right IP address of the node/function due to e.g. load balancing.

Editor's note: It is FFS if multiple IP addresses per node/function need to be supported in certain use cases like fixed deployments.

End-user payload "layer" decoupled from the transport layer, allowing different technologies in the transport layer.

Editor's note: It is FFS if Ethernet and non-IP PDU types can be supported using this solution and how the PDU Session can be identified in that case.

Editor's notes: It is FFS whether Ethernet and non-IP PDU types would need to be supported for certain use cases like fixed deployments.

Editor's note: The detailed QoS mechanism is per the progress of Key Issue#3: QoS framework.

For one AN node, there may be multiple tunnels connecting to different User Plane GWs.

Editor's note: The need for signaling to support mobility from one access node to another, for scenarios where UE mobility is needed, is FFS. It is FFS is this is applicable to Fixed Wireless Access deployments.

Figure 6.4.11.1-1: One tunnel per destination

A scenario where this solution may apply is when "a fixed wireless terminal" connects to the network, e.g., a IoT UE, or a CPE UE providing fixed-network comparable bandwidth as the access service for the "last one mile".   Such fixed wireless terminals need almost no movement or may also not be allowed (e.g., per-subscription) to move.

Figure 6.4.11.1-2: Scenario with fixed wireless and mobile terminals.

The fixed-UE scenarios are characterized by the large number of connections (e.g., IOT case) and the heavy UP traffics (e.g., CPE case). To simplify the tunnel, an "aggregated" node-level tunnel between the NextGen Access node and the UP Functions could be used.

6.4.11.2	Function description

Editor's note: This clause will contain function descriptions and the interactions among the network functions.

This section describes how a fixed wireless UE access to the network through an AN node-level tunnel.

When a UE attaches to the network or sets up a PDU session, the CP authorizes the UE type (e.g., a type of fixed wireless UE) and identification related information to check whether AN node level tunnel applies. If so, the CP will select an existing tunnel for the PDU session on information e.g., AN ID, UE type. The tunnel end point information (e.g., IP addresses, tunnel IDs) may also be provided by AN to the CP.

For the downlink traffic, the AN node identifies the UE’s IP address to differentiate to which UE the downlink data to be sent. If there are multiple node-level tunnel on the AN node, the AN node shall use the tunnel ID together with UE IP address to identifies the correct downlink path used for sending packet to the correct UE.

Figure 6.4.11.2-X Illustration of a UE attach to the network by an AN node-level tunnel

1.	The UE sends Attach Request to the AN node with the UE type.

2.	The AN node may recognize the UE type and incorporate the node-level information (i.e., tunnel end point IP addresses and tunnel IDs) together with the Attach Request message send to CP-AU.

3. 	The CP-AU verifies the PDU type and User subscription data such as UE type to authenticate the UE.

4.	The CP-AU sends Create Session Request message to the CP-SM.

5.	The CP-SM get the UP function based on the information such as PDU type, AN-ID or based on the tunnel information provided by AN. CP-SM assigns UE IP address corresponding to the UP function. CP-SM then request the AN to setup resources for the session.

6.	The CP-SM function setup the user plane with the UP function, i.e., notifies the assigned UE IP address, indicate the tunnel used to the AN and the corresponding traffic handling policy for this Session.

7.	The CP-SM sends Create Session Response to CP-AU. The message will contains the UE IP address.

8.	The CP-AU send Attach complete to the UE.

6.4.11.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * End of the changes * * * *