SA WG2 Meeting #116BIS	S2-164423

29 August - 2 September 2016, Sanya, P.R China	(revision of S2-16xxxx)

Source:	LG Electronics

Title:	Migration solution with Evolved E-UTRAN to operator with an EPC and a NextGen Core simultaneously

Document for:	Approval

Agenda Item:	6.10.18

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This paper proposes the migration solution with Evolved E-UTRAN to operator with an EPC and a NextGen Core simultaneously

Introduction

This solution addresses Key Issue #18 with the following migration scenarios

- An operator deploys Evolved E-UTRAN partially, that is, the E-UTRAN has not been fully upgraded, so there are Non-evolved E-UTRAN and Evolved E-UTRAN together in the network. In the first phase, only option 3 of Annex J may be operated, then whole NextGen system may be operated in any time.

Figure 1 shows a simplified system architecture for this migration scenario.

Figure 1: Architecture for migration scenario

This solution assumed the follows;

- the EPC and the NextGen Core have access to a common subscriber database (e.g. HSS), like solution 18.2, 18.3 and 18.4.

- Non-evolved E-UTRAN supports only EPC, and it directs the UE to EPC.

- Evolved E-UTRAN supports either EPC or NextGen Core, or both, and it directs the UE either EPC or NextGen Core according to the mode configuration.

- Evolved E-UTAN is operated with one of the following modes, and the mode configuration is informed of the UEs (e.g. via SIB)

  - S1 mode (supporting access to only EPC, it may be similar to non-evolved E-UTRAN operation)

  - NG mode (supporting access to only NextGen Core, it performs only NG RAN operation)

  - S1&NG mode (supporting access to both EPC and NextGen Core)

“S1 mode” Evolved E-UTRAN may not need to broadcast the mode information, and the flexibility of the Evolved E-UTRAN mode depends on the operator’s policy, the local configuration, etc.

Editor’s note:		The feasibility of this assumption should be determined in RAN WG.

- The EPC UE always connects to EPC using EPC NAS.

- The NG UE connects EPC or NextGen Core using EPC NAS or NG NAS according to its capability, RAN mode operation and the following UE preference configuration;

  - EPC preferred

An operator may configure it for some UEs in the environment which the NextGen system has not been fully deployed, or for the management reason at any time.

A UE configured as “EPC preferred” tries to connect to EPC using legacy NAS first, and if it is failed or under “NG mode” Evolved E-UTRAN, it can try to connect to NextGen core.

Editor’s note:		It is FFS whether the UE with option 3 is considered as a NG UE configured “EPC preferred”, and it may be determined in RAN WG.

  - NG preferred

An operator may configure it for some UEs according to their migration strategy or operator policy.

A UE configured as “NG preferred” tries to connect to NextGen Core first, and if it is failed, it can try to connect to EPC.

  - NG Only

An operator may configure it for dongle-type UE supporting Evolve E-UTRA and/or NR.

A UE configured as “NG preferred” always connect to NextGen Core.

- Evolved E-UTFAN determines which core network (i.e. EPC or NextGen Core) a UE should be directed to, when receiving the signal or data from a UE.

Proposal

It is proposed to add the following changes to TR 23.799.

* * * * Start of 1st Change * * * *

All NEW text to TR 23.799

6.18.X	Solution 18.X: Migration solution with Evolved E-UTRAN to operator with an EPC and a NextGen Core simultaneously

This solution addresses Key Issue #18 with the following migration scenarios

- An operator deploys Evolved E-UTRAN partially, that is, the E-UTRAN has not been fully upgraded, so there are Non-evolved E-UTRAN and Evolved E-UTRAN together in the network. In the first phase, only option 3 of Annex J may be operated, then whole NextGen system may be operated in any time.

6.18.X.1	Architecture description

Figure 6.18.X.1-1 shows a simplified system architecture for this migration scenario.

Figure 6.18.X.1-1: Architecture for migration scenario

This solution assumed the follows;

- the EPC and the NextGen Core have access to a common subscriber database (e.g. HSS), like solution 18.2, 18.3 and 18.4.

- Non-evolved E-UTRAN supports only EPC, and it directs the UE to EPC.

- Evolved E-UTRAN supports either EPC or NextGen Core, or both, and it directs the UE either EPC or NextGen Core according to the mode configuration.

- Evolved E-UTAN is operated with one of the following modes, and the mode configuration is informed of the UEs (e.g. via SIB)

  - S1 mode (supporting access to only EPC, it may be similar to non-evolved E-UTRAN operation)

  - NG mode (supporting access to only NextGen Core, it performs only NG RAN operation)

  - S1&NG mode (supporting access to both EPC and NextGen Core)

“S1 mode” Evolved E-UTRAN may not need to broadcast the mode information, and the flexibility of the Evolved E-UTRAN mode depends on the operator’s policy, the local configuration, etc.

Editor’s note:		The feasibility of this assumption should be determined in RAN WG.

- The EPC UE always connects to EPC using EPC NAS.

- The NG UE connects EPC or NextGen Core using EPC NAS or NG NAS according to its capability, RAN mode operation and the following UE preference configuration;

  - EPC preferred

An operator may configure it for some UEs in the environment which the NextGen system has not been fully deployed, or for the management reason at any time.

A UE configured as “EPC preferred” tries to connect to EPC using legacy NAS first, and if it is failed or under “NG mode” Evolved E-UTRAN, it can try to connect to NextGen core.

Editor’s note:		It is FFS whether the UE with option 3 is considered as a NG UE configured “EPC preferred”, and it may be determined in RAN WG.

  - NG preferred

An operator may configure it for some UEs according to their migration strategy or operator policy.

A UE configured as “NG preferred” tries to connect to NextGen Core first, and if it is failed, it can try to connect to EPC.

  - NG Only

An operator may configure it for dongle-type UE supporting Evolve E-UTRA and/or NR.

A UE configured as “NG preferred” always connect to NextGen Core.

- Evolved E-UTFAN determines which core network (i.e. EPC or NextGen Core) a UE should be directed to, when receiving the signal or data from a UE.

6.18.X.2	Function description

Editor's note:	This clause will contain function descriptions and the interactions among the network functions.

6.18.X.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * * End of Changes * * * *