SA WG2 Meeting #116-BIS	S2-164449

29 Aug – 2 Sep 2016, Sanya, CN	(revision of S2-16xxxx)

Source:	ZTE, CATR

Title:	Clarification to solution 3.2

Document for:	Approval

Agenda Item:	6.10.3

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This contribution proposes to clarify that in RRC inactive state, UE performs Area registration to CN when the UE moves outside of the registered area(s) or upon message from RAN node that the UE context is not available.

1.	Discussion

When the UE is in the RRC Inactive state the UE is regarded as in NG CM-CONNECTED mode in both UE and CN this should be clarified clearly. Further when the UE is in the RRC Inactive state the UE still need to perform area registrations when it move outside of the registered areas.

Another thing is when the UE moves to a new RAN node and during the RRC resume procedure if the new RAN node can’t retrieve UE context from the old RAN node, the new RAN node should notify the UE. The UE then moves to NG CM-IDLE mode and it may perform Area registration or Service Request procedure to the CN.

2	Proposal

It is proposed to agree the following text in the TR23.799.

/******************** Start of Changes ***************/

6.3.2	Solution 3.2: Mobility state framework

6.3.2.1	Architecture description

Next Generation (NG) core network should take into consideration the state machines envisioned for RRC protocol within new RAT. It is proposed that the mobility state machine for RRC should have an inactive connected state (in addition to an RRC Connected state and an RRC Idle state). The need for configurability of the RRC inactive connected state is motivated through several factors that require flexibility such as diverse requirements of the 5G use cases, future-proofness and quick time to market requirement for new services.

From the Next Generation core network perspective, UE is considered to be in the NG CM-CONNECTED state when UE is in RRC inactive connected state at the RRC layer. When the UE transitions between RRC connected state and RRC inactive connected state, it is not visible to the core network as no signalling towards the core network is expected due to this transition. Also, core network does not have to page the UE when the UE is in RRC inactive connected state as both control plane and user plane remains established between the RAN and core.

The characteristics of the RRC inactive connected state includes:

-	UE is considered to be in the NG CM-CONNECTED state in UE and CN.

-	Configurable to serve the service(s)  requested by the UE. This implies that the RRC inactive connected state can be configured taking the characteristics and requirements of the application(s)  running in the UE, subscription and UE activity into account. Core network shall provide that related information to RAN.

-	UE based mobility inspired by cell reselection procedure with configuration from network , no network controlled handover is supported.

-	UE performs Area registration to CN when the UE moves outside of the registered area(s).

-	The Access Stratum (AS)  context is stored in RAN and the UE.

-	RRC inactive connected to RRC connected state transition inspired by the Suspend and Resume procedures defined for LTE in Rel-13. Furthermore, no signalling to the CN is needed to perform this transition. The AS Context may need to be transferred between the RAN nodes.

-	U-plane and C-plane connections between RAN and core are kept established.

-	UE reachability will be managed by the RAN, with assistance from core network.

-	UE paging will be managed by the RAN.

-	CN will transit to the NG-CM IDLE state upon RAN request.

-	UE will transit to the NG-CM IDLE state if the RAN notifies the UE that the UE context is not available during the RRC resume procedure. The UE may send Area registration or Service request to the CN.

-	Distributed mobility management – Network follows UE on CN level.

NOTE: Whether a RAN determined area where a UE can move without notifying the network is needed, is for the RAN WGs to determine.

-	No Rx/Tx Data is performed in this state.

-	To support LTE and NR deployments in an efficient way the solution for state transition shall avoid or minimize UE signalling when UE is toggling between NR and Evolved E-UTRA in Inactive state.

Following figure shows how the proposed RRC state machine fits within the NG CM/MM State machine model when using the RRC INACTIVE CONNECTED state:

Figure 6.3.2.1-1: NG RRC/CM/MM with state transition illustrated

State Transitions illustrated:

(1)	UE powers on, performs PLMN/cell selection and camps in a suitable cell.

(2)	UE registers (attaches) with the network and performs connection setup. UE transmits and receives data in RRC connected.

(3)	Inactivity results in transitions to RRC inactive connected.

(4)	MT/MO trigger to transmit data results in transition to connected for Tx/Rx.

(5)	Detach/Power off.

(6) Connection setup/release

NOTE:	This solution has dependency on RAN as the RRC state design decision belongs to RAN WG2. RAN WG2 is expected to define the number of necessary states and their characteristics in order to achieve its requirements.

6.3.2.2	Function description

Editor's note:	This clause will contain function descriptions and the interactions among the network functions.

6.3.2.3	Solution evaluation

Editor's note:	This clause will contain evaluation on the system impacts, e.g. UE, access network and non-access network.

/******************** End of Changes ***************/