SA WG2 Meeting #116	S2-164635

29 August – 2 September 2016, Sanya, P.R. China

Source:	Qualcomm Incorporated

Title:	Evaluation criteria for key issue 1 – network slicing. Minimum set of service functionality for compatibility.

Document for:	Discussion/Approval

Agenda Item:	FS_NexGen /6.10.1

Work Item / Release:	FS_NexGen / Rel-14

Abstract of the contribution: Proposes an evaluation criteria related to roaming for network slicing solutions.

# Introduction

NextGen is studying the support of network slicing to provide specific services via dedicated slices. The support of network slicing as currently being defined is for the NG CN. This means the UE in a Rel. 15 network has access to such slicing solution only when connected to an NG RAN, since only an NG RAN is connected to the NG CN.

For many types of services implemented via slicing, and in particular for the public safety oriented services, it is surely preferable to have the services provided with some consistency or some minimum level of service consistency independently of whether the UE is connected to an NG RAN or another RAN.

Let’s assume that that EPC and the RAN have implemented DÉCOR/eDECOR, and that the operator has deployed NG CN and has upgraded some but not all the E-UTRAN nodes. The UE, e.g. when connecting to E-UTRAN, would need to know whether it is a non-evolved E-UTRAN or an Evolved E-UTRAN, since in the non-evolved case to access a specific service (and the corresponding dedicated CN) will use eDECOR and provide the DCN ID. However, in case of evolved E-UTRAN, it will provide the information defined in the network slicing solution for the network slice solution (e.g. MDD for solution 1.2). It is unclear how the UE knows how to behave.

Moreover, the UE would need to be configured in two different ways to access the same service over EPC and NG CN, assuming the same service is available on both when provided via network slicing in the NG CN.

Providing a minimum set of compatibility between the provisioning of specific services, e.g. Public Safety, when the UE is connected to the NG CN using network slicing and when the UE is connected to the EPC, would simplify the deployment of services and provide a more consistent user experience.

# Proposal

It is proposed to add the following evaluation criteria.

**** Start of changes ****

### 6.1.X	Evaluation Criteria for Solutions to Key Issue 1

#### 6.1.X.1 Minimum Service Compatibility

Solutions for provisioning services via network slicing in NG CN should provide a minimum compatibility with the provisioning of the same service over EPC, in order to provide a more consistent service experience within an operator network. Solutions should be evaluated considering:

-	Compatibility between Rel. 15 network slicing solutions and pre-Rel. 15 solutions for selection and connectivity to dedicated core networks

-	Requirements in terms of different configurations of UE to access the same service over EPC and over NG RAN

**** End of changes ****