SA WG2 Meeting #116	S2-164638

29 August – 2 September 2016, Sanya, P.R. China

Source:	Qualcomm Incorporated

Title:	Update of QoS Solution 2.5

Document for:	Discussion/Approval

Agenda Item:	FS_NexGen /6.10.2

Work Item / Release:	FS_NexGen / Rel-14

Abstract of the contribution: Proposes an update to the QoS solution 2.5 to clarify some FFS and align to the other solutions.

# Introduction

# Proposal

It is proposed to add the following changes to section 7.3.

**** Start of changes ****

### 6.2.5	Solution 2.5: QoS model for flexible traffic differentiation

#### 6.2.5.1	Architecture description

Editor's note: This clause will contain e.g., terminology, overview, architecture description of the solution.

##### 6.2.5.1.1	Functional Architecture

The functional architecture of solution 2.2 is assumed.

##### 6.2.5.1.2	Applications Requirements Input

Other solutions for key issue #2 on QoS defines a set of application requirement inputs provided by an AF to the CP Function for QoS, and define how the QoS authorization based on application requirements takes place.

In addition to what is proposed already, this solution proposes that the AF provides to the CP Functions also an indication of whether different packets within a data flow require differentiated treatment. E.g., for a data flow, the AF may provide two levels of QoS requirements in terms of Service Behavior, Service Requirements, Delay Requirements and Priority for solution 2.1, or in terms of two levels of bitrate for the same IP flow in case of solution 2.2.

##### 6.2.5.1.3	Packet Markings for Traffic Differentiation

A new packet marking that conveys in-band information related to the data flow features (including information describing packet characteristics above layer 4) is introduced in order to enable easier traffic differentiation when traffic cannot be easily detected (e.g. for encrypted traffic) and to enable differentiated treatment of different packets belonging to the same data flow (e.g. packets that have different characteristics at level 5 and above and that require different treatment).

Editor's note: if packetization of data traffic in an application server includes e.g. high priority traffic and low priority traffic in the same packet, it is FFS whether it is possible to provide differentiated treatment for such traffic.

Editor's note: it is FFS to what media types the solution applies.

Upon definition of the QoS Policy or Authorized QoS in solutions 2.1 and 2.2 in collaboration with the AF (i.e. selection of QoS parameters defining the QoS for a data flow according to the QoS solution and the related parameters), the CP Function also generates a Packet Marking Token associated with the IP traffic.

The Packet Marking Token is proposed for media types where the traffic is encrypted and traffic differentiation information at L4 and above are not visible to the NextGen CN entities that perform traffic detection and classification. The proposed packet marking enables identification of the specific traffic in the packet by mapping the Packet Market Token directly to the QoS flow the data packet belongs to. The Packet Marking Token enables mapping a packet to:

-	the corresponding QoS identifier (e.g. FPI, FFI, QoS ID of other solutions) and a PDPI value, if a QoS flow is defined in terms of packets whose QoS treatment differ based on the PDPI

-	the corresponding QoS identifiers (e.g. FPI, FFI, QoS ID of other solutions) of the QoS flow, if an explicit PDPI value is not associated to a data flow but identified directly by the QoS identifier.

In addition, a Packet Set Marking is added by the entity sourcing the data traffic.

The solution is based on:

-	The presence of an AF that supports this packet marking solution and that interacts with the NextGen CN over NG5 to establish the QoS and packet treatment for the traffic

-	The AF marking packets with the proposed Packet Marking Token over the NG6 interface.

Editor's note: it is FFS how the Packet Marking Token and the Packet Set Marking are carried over NG6. E.g. for IPv6, extension headers can be used.

-	The UP-GW that terminates NG6 receives the packet and, based on the Packet Marking Token, identifies the QoS flow the packet corresponds to and the associated QoS policy and packet treatment. The UP-GW marks the packet towards the AN with mechanisms defined in other solutions (e.g. a QoS identifier like an FII or QoS ID, and a PDPI value if supported) based on the associated QoS policy, and with the Packet Set Marking value received over NG6.

-	The marking performed by the UP-GW is carried to the RAN which can enforce QoS as defined in other solutions.

The Packet Marking Token is defined and used as follows:

- Upon definition of the QoS Policy in other solutions in collaboration with the AF (QoS Policy then distributed to the UP function(s) over NG4 and to the AN over NG2), the CP Function also generates downlink Packet Marking Token(s) associated with the data traffic. This may include Packet Marking Tokens for potential future traffic exchanged by the UE and the AF.

-	Each Packet Marking Token contains

-	A Token ID, identifying the token and the corresponding QoS policy

-	A Token Parameter Index (TPI) indicating which fields in a data flow header are used to compute the token hash

-	A Token Hash, calculated using the fields in each packet of the data flow described by the TPI

-	The Packet Marking Token is generated via a cryptographic hash with a key decided by the CP Function and provided to the entities that need to verify the token

-	The Packet Marking Token is appended to each packet of a data flow over NG6. Verification of a Packet Marking Token appended to the packet enables the UP-GW enforcing QoS and performing traffic filtering to identify the QoS policy associated with the packet without the need for TFTs.

Editor's note: security aspects of this solution, e.g. how to securely generate, allocate, verify, and provision tokens to the UE will need to be studied by SA3.

-	In the QoS Policy or Authorized QoS, the CP Function:

-	for each data flow or specific set of packets in a data flow for which a downlink token has been generated, it provides to the UP-GW the Token ID, the information for token verification (e.g. the key used to generate the token), in addition to the remaining QoS information in the QoS policy.

-	for each data flow or specific set of packets in a data flow for which a downlink token has been generated, it provides to the AF the downlink Packet Marking Token .

-	Tokens are generated as follows:

-	if for a data flow the AF provides a single requirement for packet treatment, e.g. flow descriptor/packet filter and the related QoS requirements, the CP Function generates a single downlink (DL) Packet Marking Token .

-	if for a data flow the AF provides an indication that packets within the data flow require different QoS treatment, and the packets differentiation is at a layer above L3 and L4, and provides requirement for each required level of packet treatment, the CP Function generates a separate downlink Packet Marking Token for each different level of treatment. The CP Function indicates to the AF which Packet Marking Token shall be used for each different level of QoS treatment. When packets marked with the different Packet Marking Tokens corresponding to the different levels of treatment reach the UP-GW, the correct QoS policy can be applied by verifying the token and using the token identifier to retrieve the QoS policy.

-	For each packet received containing a Packet Marking Token, the UP-GW retrieves the information corresponding to the Token ID, and computes the hash of the fields in the packet identified by the TPI using the token verification key provided by the CP Function. If the token is verified, the UP-GW does not need to perform any further packet inspection and it applies the QoS policy associated with the token.

Editor's note: whether and how the solution works in the presence of middleboxes on NG6 is FFS and depends on how tokens are transported.

The Packet Set Marking is added in addition to the Packet Marking Token:

-	in order to enable flexible and dynamic QoS for a block of packets in a data flows (e.g. a block of packets for a video streaming) or to enable dynamic modification of a block of packets within a data flow, each block of packets sourced at an application server is marked with Packet Set Marking that distinguishes a block of packets from the following one.

Editor's note: it is FFS if the Packet Set Marking is an additional marking wrt the Packet Marking Token, or if it is part of the Packet Marking Token, and depends on how the Packet Marking Token is encoded.

#### 6.2.5.2	Function description

Editor's note: This clause will contain function descriptions and the interactions among the network functions.

##### 6.2.5.2.1	Dynamic QoS Modification

In order to enable the support of dynamic QoS modification for a data flow, e.g. temporarily, when the UE detects the need to modify dynamically the QoS treatment (e.g. priority) of a block of packets, the UE provides a request to the RAN indicating which data flow the request relates to, providing the Packet Set Marking to identify which specific packets the request relates to, and with an indication of the new requested priority. Though the specific of the indication shall be defined by RAN, from a NextGen system point of view if FPI is used as in solution 2.1, the UE requests an Override FPI value, whereas if FPI is not used as in solution 2.2 the UE may either provide the AN with the deadline for the delivery of the block of packets, or a value of downlink Packet Delay Budget (as defined in TS 23.203 but applied to downlink).

Editor's note: how the UE indicates the modified priority is FFS and shall be defined by AN.

Editor's note: if the AN is in congested status, whether the UE can send this message to the AN is FFS.

In this solution it is also assumed that, when the CP Function provides the QoS Policy of the Authorized QoS to the AN, the CP Function also indicates based on the subscription profile, QoS request from the AF (and which AF is requiring the QoS), and local policies the data flows for which the UE is allowed to request a priority modification.

The AN, upon receiving the request to modify the priority, accepts or rejects such request based on local conditions and policies in addition to the information provided in the QoS Policy or Authorized QoS by the CP Function.

Editor's note: it is FFS if a similar mechanism should be defined for uplink traffic, e.g. when the UE acts as a streaming server.

**** End of changes ****