SA WG2 Meeting #116	S2-164644

29 August – 2 September 2016, Sanya, P.R. China

Source:	Qualcomm Incorporated

Title:	Evaluation criteria for Interworking and migration solutions

Document for:	Discussion/Approval

Agenda Item:	FS_NexGen /6.10.18

Work Item / Release:	FS_NexGen / Rel-14

Abstract of the contribution: Proposes evaluation criteria for solutions to key issue 18.

# Introduction

The following evaluation criteria are derived from the email discussion on open aspects of the interworking and migration solutions for key issue 18 and the following comparison table between the solutions.

|  | 6.18.1 | 6.18.2 | 6.18.3 | 6.18.4 | 6.18.5 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Interfaces between EPC-NG CN | Yes, used independently of RAN node change | “HSS” in common and i/f exists between IWF and NG CN for handover a-la “S1 HO” | “HSS” in common and i/f exists between IWF and NG CN for handover a-la “S1 HO” | No, only “HSS” in common | Yes, for c-plane (mobility, context transfer), i.e. “S1 HO” | Most solutions see an i/f between the two CNs. If the IWF of 6.18.2 and 6.18.3 is implemented in MME and SGW (all or dedicated ones), then 6.18.1, 6.18.2, 6.18.3 and 6.18.5 are aligned |
| Support for IP address preservation | Yes | Yes, UP anchoring in NG CN | Yes, UP anchoring in NG CN | Option available with common UP anchor | With IW functionality in E-UTRAN and UE and common UP anchor | 6.18.1, 6.18.2, 6.18.3 and 6.18.5 seem aligned |
| Support for dual radio UEs | No | No | No | Yes, dual attach with dual radio | No | 6.18.1, 6.18.2, 6.18.3 and 6.18.5 are aligned |
| Security Context | Re-use security context in target system at east for a while | Single security context in NG CN for I1. For I2, a security context is generated in the CN the UE attaches via NCIF. At mobility to evolved E-UTRAN or NR, a target security context is generated and subsequently UE is re-authenticated to “anchor” the security context in NG CN. | As for 6.18.2 | N/A | N/A | Requires further analysis of details. Depending on SA3 design |
| Session anchoring | Anchor point changes | Anchor point remains the same | Anchor point remains the same | May change in loose IW | Unclear if session anchoring changes when moving from EPC to NG CN | Requires further analysis |
| Session continuity (Evolved E-UTRAN to NR) | Via EPC-NG CN interface and anchoring in source system. | In I1, depending on SSC model selected per NG CN mechanisms (can be supported). In I1 and I2, based on anchoring of UP in NG CN. | Based on anchoring of UP in NG CN. | Based on anchoring of UP in CN (common anchor for EPC and 5G CN). If no common anchor, use application level continuity | Based on anchoring of UP in CN | Differences wrt u-plane anchoring always in source system, in common EPC/NGCN anchor, always in NG CN, or no common anchoring |
| Session continuity for E-UTRAN (including option 3) and NR | Via EPC-NG CN interface and anchoring in source system. | Same as above | As 6.18.2 I2 scenario. | Based on dual radio operations (dual attach to EPC and NG CN) and common UP anchor for EPC and NG CN. Otherwise, continuity at service level must be used. Service continuity for IMS anchored in NG core via Dual Radio VCC (DRVCC) upon handover to/from GERAN or UTRAN | Same as above (?) | Differences wrt u-plane anchoring always in source system, in common EPC/NGCN anchor, always in NG CN, or no common anchoring |
| Support for scenarios with no session cont. | Fast session recovery in target system | Session continuity always present for E-UTRAN to/from NG RAN | Session continuity always present for E-UTRAN to/from NG RAN | For loose IW case, continuity at service level must be used. | Session continuity always present for E-UTRAN to/from NG RAN | 6.18.2, 6.18.3 and 6.18.5 are aligned |
| UE Aspects |  | Use EPC NAS or NG NAS depending on the core network by which it is served. | Use EPC NAS or NG NAS depending on the core network by which it is served. | Use EPC NAS or NG NAS depending on the core network by which it is served. | Use EPC NAS or NG NAS depending on the core network by which it is served. | Aligned |
| NAS requirements |  | NG NAS in NG RAN EPC NAS in (non-evolved) E-UTRAN | NG NAS in NG RAN EPC NAS in (non-evolved) E-UTRAN | NG NAS in NG RAN EPC NAS in (non-evolved) E-UTRAN | NG NAS in NG RAN EPC NAS in (non-evolved) E-UTRAN | Aligned |
| Support for Option 3 UE | ? | Support "non-evolved" E-UTRA procedures for backwards compatibility with pre-rel.15 networks Support EPC NAS procedures e.g. for roaming | Support "non-evolved" E-UTRA procedures for backwards compatibility with pre-rel.15 networks Support EPC NAS procedures e.g. for roaming | Support "non-evolved" E-UTRA procedures for backwards compatibility with pre-rel.15 networks Support EPC NAS procedures e.g. for roaming | ? |  |
| Requirements for interworking in eE-UTRAN |  | UE indicates in AS support of 5G CN for e E-UTRAN routing to IWF No need to indicated support of connectivity with IWF by eNB (non-evolved E-UTRAN), since UE will use only EPC NAS | UE indicates in AS support of 5G CN for e E-UTRAN routing to 5G CN eE-UTRAN indicates support of 5G CN in SIB |  | RRC IW in E-UTRAN and in UE | Requires more analysis of details. |

# Proposal

**** Start of changes ****

### 6.18.X Evaluation Criteria

#### 6.18.X.1	Impacts on EPC

Does the solution require modification to EPC nodes functionality?

#### 6.18.X.2	Impacts on NG CN for interfacing/interworking with EPC

Does the solution require the NG CN functions to support EPC specific functionality for the interworking and migration?

#### 6.18.X.3	Impacts on the UE

The solutions are evaluated in terms of:

- Impact on the UE in terms of support of single attach (to either EPC or NG CN) or dual attach (to both EPC and NG CN) over an NG RAN and non-evolved E-UTRAN.

#### 6.18.X.4	Type of interworking supported

Does the solution support tight interworking with session continuity?

#### 6.18.X.4	Impacts of the type of interworking supported

If the solution does not support tight interworking for single radio UEs, what is the expected performance of the solution in terms of delay when moving from RAT or system to the other.

**** End of changes ****