SA WG2 Meeting #116bis	S2-164659

29 August – 02 September 2016, Sanya, China

Source:	China Mobile

Title:	proposal on migration and roaming to TR 23.799

Document for:	Approval

Agenda Item:	6.10.18

Work Item / Release:	FS_NextGen/R-14

Abstract of the contribution: This contribution propose to capture some migration, roaming scenario and UE capability assumptions in the TR 23.799 based on email and offline discussion for SA2#116bis.

1. Proposal

Based on the email converter summary (S2-164614) and the Migration Scenario and Roaming Slides (S2-164658) It is proposed to add following text to the TR 23.799 v0.7.0.

* * * First changes * * * *

5.18	Key Issue 18: Interworking and Migration

5.18.1	Description

This key issue focuses on migration and interworking scenarios.

5.18.1.1 Interworking and migration principles

-	The deployment of NextGen System without interworking between EPC and NG-Core is the long-time final goal after migration (under the assumption the EPC may still last for a long time to provide legacy or roaming UE support).

-	NextGen System’s requirements therefore should not be compromised to improve interworking between EPC and NG-Core or to fit a specific migration path.

-	Any interworking functionality between EPC and NG-Core shall be developed in a manner that

-	Minimizes impacts to EPC as well as to the NextGen core.

-	Does not create any permanent dependencies for NG-Core on interworking to EPC even after the migration is completed.

  - -	To avoid market fragmentation, UE forward compatibility shall be achieved

5.18.1.2 Assumptions on UE capabilities and network

The following UE capabilities are assumed hold for the migration and roaming.

-	Assumption C0: A UE that supports any NG RAN also has LTE/EPC capability.

-	Assumption C1: the NG CP stack can only be used by one RAT at any instance (either NR or eLTE) and cannot be used by both RATs at the same time.

-	Assumption C2: the NG CP stack can be used by either NR or eLTE within a UE based on the network deployment

Editor's note: whether C1 or C2 is true needs to check with RAN TSG.

5.18.1.3 Interworking and migration scenarios within an operator

Any alternative combinations or sequence of deployment and/or transition flows involving options 3, 7, 2/4 would similarly need to be supported. For example, two of the migration paths are illustrated as:

1)	LTE/EPC -> Option 2/Option 4

2)	LTE/EPC -> (Option 3) -> Option 7 -> Option 2/Option 4

NOTE 1:	Option 7 and Option 3 can coexist in the same PLMN at the same stage.

NOTE 2:	the support of Option 7 implies that Option 5 will also be supported..

From SA2 perspective the interworking scenario generated from the migration paths can possibly be simplified as: LTE/EPC and NextGen System.

5.18.1.4 Interworking scenarios on roaming

  - Under the Section 5.18.1.2 Assumption C0, a NextGen UE will get at least LTE/EPC service where LTE/EPC exists. On top of that, it will be able to use any NG RAN in the VPLMN that it supports and that the roaming agreement allows.

  - Under Assumption C2, the roaming scenarios are simplified to the following cases:

1)	EPC in VPLMN, NG Core in HPLMN

2)	NG Core in VPLMN, EPC in HPLMN

5.18.1.5 Forward compatibility

Forward compatibility requirement is based on operators deploying either Option 3 and/or Option 7. Option 7 should be backwards compatible to Option 3. This translates into two main requirements/goals:

a)	UE supporting Option 7 should also support Option 3

b)	Network supporting Option 7 should also provide Option 3 service to UEs supporting Option 3.