SA WG2 Meeting #116bis	S2-164709

29 Aug - 2 Sep 2016, Sanya, China	(revision of S2-16xxxx)

Source:	CATT

Title:	Update to Interim agreements on NextGen RAN and NextGen Core functional allocation

Document for:	Approval

Agenda Item:	6.10.3 and 6.10.8

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: It is proposed to update Interim agreements on NextGen RAN and NextGen Core functional allocation section to include the outcome of email discussion on “MM WT#3 on mobility level”.

Based on the principle that RAN/CN should be allowed evolve independently as much as possible, it is proposed to agree that the NG1 singling terminates in CN.

Proposal

- During the discussion on “Convergence of UE Mobility Level Solutions” and “Summary of email discussion on mobility levels issues”, it is the common understating that 1) the definition of Mobility Level is needed in the NexGen system ; 2) the determination of UE’s mobility level is done by core network, 3) updated the mobility level to the UE is also needed; 4) for some UE scenarios, there is no CN-IDLE state.

- Some solutions to KI#8 think RAN could handle NAS message, while other solutions propose NAS signalling terminates in NexGen Core. Base on the agreed high level principle captured in TR 23.799 section 4.1 .i.e  “ 4	Allow independent evolutions of core network and RAN, and minimize access dependencies.”,  it is proposed that the NG1 NAS signalling terminates in NexGen CN.

  - Note. This conclusion does not preclude other part of solutions that favour RAN handling (in addition to transport) NAS message to be evaluated.

It is proposed to include such agreements into TR 23.799.

* * * Start of changes * * * *

8.1	Interim Agreements on NextGen RAN and NextGen Core functional allocation

The following table documents the current status of agreements on the CN-RAN functional allocation:

Table 8.1-1: Logical function allocation

| Location: Function: | NextGen RAN | NextGen CN | Comments |
| --- | --- | --- | --- |
| Key Issue #1 - Network Slicing |  |  |  |
| CN instance selection when UE attach to a CN network slice | FFS | FFS |  |
| Key Issue #3 - Mobility Management |  |  |  |
| Mobility management control, (Subscription and Policies) |  | X |  |
| Mobility Level Determination/Updated and Update changed Mobility Level to UE |  | X |  |
| Determination of mobility restriction |  | X |  |
| Roaming restrictions execution |  | X |  |
| Mobility restrictions execution, [CN Connected] | X |  |  |
| Mobility restrictions execution, [CN Idle] |  | X | It is expected that the RAN design will enable minimization of CN-initiated paging and UE associated CN/RAN signaling |
| UE registration |  | X |  |
| Area tracking | FFS | X | The need for RAN level area tracking is for RAN WGs to determine. |
| UE unreachability detection |  | X | Assumed to be supported in CN for some UEs in CN Idle state. |
| RAN UE unreachability detection | X |  | Assumed to be supported in RAN for UEs in RAN Inactive state. If RAN inactive state exists. |
| NAS state transitions |  | X |  |
| RRC state transitions | X |  |  |
| Paging initiation and control in RAN Inactive state | X |  | RAN Inactive state is RAN state that corresponds to CN connected state. If RAN inactive state exists. |
| Paging initiation in CN Idle state |  | X |  |
| Access Stratum UE Context storage in RAN Inactive state | X |  | If RAN inactive state exists. |
| Control of connected state mobility | X | X |  |
| UP buffer for UE in CN Idle state |  | X | For those UE have CN Idle state. |
| UP buffer for UE in RAN Inactive state | X |  | If RAN inactive state exists. |
| Key Issue #4 - Session Management |  |  |  |
| PDU Session address allocation |  | X | FFS for non-IP PDU Sessions |
| PDU Session Termination Point |  | X | Note that this refers to the ownership of the specification for the function supporting the termination point. In a NW deployment this function may be deployed on or close to a RAN site. |
| Session Management |  | X |  |
| Termination of UP security | FFS | FFS | FFS |
| Subscription Data Handling (incl. default QoS profile) |  | X |  |
| Key issue #12 |  |  |  |
| Authentication and Key Agreement |  | X |  |
| Key Issue #2 QoS |  |  |  |
| Radio Resource Admission Control | X |  |  |
| Radio Resource management (QoS attributes) | X |  | Packet scheduling with regards to resource utilization and availability (RRM) |
| Max rate control | X | X | Maximum bitrate policing in the CN and RAN in UL and DL. |
| QoS Policy Control |  | X |  |
| Transport marking | X | X | Used for prioritization in the transport network. |
| Charging Data Collection |  | X |  |
| Packet classification of DL packets for QoS differentiation on the Radio | FFS | FFS | Some companies think the QoS classification for QoS differentiation of DL packets is performed in RAN. |
| QoS differentiation and verification for UL packets | FFS | FFS | Some companies think the QoS verification for UL packets is performed in RAN and/or CN. |
| Key issue #8 Next Generation core and access - functional division and interface |  |  |  |
| NAS signalling termination |  | X | NG1 signalling terminates in CN parts. |

* * * End of changes * * * *