SA WG2 Meeting #116-BIS	S2-164903

29 Aug – 2 Sep 2016, Sanya, CN	(revision of S2-164448)

Source:	ZTE, CATR

Title:	Update to solution 2.8

Document for:	Approval

Agenda Item:	6.10.2

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This contribution proposes to adding media type in the application information. The application information provision procedure is also added.

1	Proposal

It is proposed to agree the following text in the TR23.799.

/******************** Start of Changes ***************/

6.2.8	Solution 2.8: Application-based uplink traffic identification

6.2.8.1	Solution Description

This solution addresses the working task#3 and #4 of key issue 2 on a QoS framework and is specific for the uplink traffic identification.

In this solution, the network may provide the UE the QoS policy together with application information when the PDU session is setup. The application information at least includes pair of OSId and OSAppId which can uniquely identify the application installed in the UE. For uplink traffic the UE identifies the application which generates the UL traffic and enforces the QoS Policy associated with the application information.

The User Plane Function in core network performs UL traffic verification by using application detection filters.

6.2.8.2	Function description

Editor's note: This clause will contain function descriptions and the interactions among the network functions.

Figure 6.2.x.2-1 Application information provisioning

1.	A PDU session with a non service specific PDU flow is established. This PDU flow can be used for any service data transmission.

Two options to provision the application information to the UE:

Option 1:

2.	If the network needs to provide QoS control over specific applications running in the UE, the CN-CP conveys the application information together with the associated QoS rules to the UE. The application information includes pairs of OSId and OSAppId which can uniquely identify the application installed in the UE. The UE uses this application information for UL traffic classification.

Option 2:

3.	If the network needs to provide QoS control over specific applications running in the UE, the CN-CP (Policy function) conveys the application information to the UE. The application information includes an application Id and pairs of OSId and OSAppId which can uniquely identify the application installed in the UE.

4.  During the establishment of the service specific PDU flow, the CN-CP(SM function) provides the service specific QoS rules including application Ids over NG1 to the UE. The application Ids used to identify the application which is subject for the service specific QoS rules.

NOTE x:  How the CN-CP conveys the application information to the UE is FFS. It could include e.g OMA-DM, or dedicated NAS signalling.

6.2.8.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

/******************** End of Changes ***************/