SA WG2 Meeting #116-BIS	S2-164921

29 Aug -2 Sep 2016, Sanya, China	(revision of S2-164691)

Source:	Telenor, NEC, Orange, Deutche Telekom AG, Huawei, InterDigital

Title:	Updating KI#9 with Context Awareness

Document for:	Approval

Agenda Item:	6.10.0

Work Item / Release:	NextGen / Rel-14

Abstract of the contribution: This contribution introduces the need for Context Awareness within the NextGen core.

## Introduction

Context information is used in today’s networks for the optimization of Network Functions (NF) within e.g., SGWs or PGWs. However this information is usually handled by the different NFs in a non-consolidated way, i.e. by making use of different reference points and is handled in different semantics.

In NextGen, according to what was discussed in several Key Issues so far, session/network context information is required, not only for optimizing UP NFs behaviour ,but also Control-Plane (CP) NFs. This was captured in the existing Key Issues (KI)s discussions where session/network context has been highlighted as an enabler in KI-2 (QoS framework), KI-3 (mobility management), KI-4 (session management), KI-5 (efficient user plane paths), KI6 (session continuity) and KI-12 (security framework).

The challenge however is in handling the generation of such context from the constituent inputs (e,g, UE, NFs or 3rd party applications) in an efficient way. So the main focus of this contribution is to introduce the study of the efficient generation and retrieval/provisioning of rich session/network context to be utilized by UP/CP NFs, and assist them in decision making.

NOTE: Context consumption/execution by the NF is out-of-scope of this KI.

# Proposed changes to TR 23.799

## 5.9	Key Issue 9: 3GPP architecture impacts to support network capability exposure and context information awareness.

### 5.9.1	Description

The next generation system is expected to accommodate various services and TR 22.861 [4], TR 22.862 [5] and TR 22.863 [6] will continue to define requirements for key service categories, i.e. massive IoT, critical communications, and enhanced mobile broadband, respectively. To allow the 3rd party, UE or Network Functions to access information regarding services provided by the network (e.g. connectivity information, QoS, mobility, etc.) and to dynamically customize the network capability for different diverse use cases within the limits set by the operator, the next generation system should provide suitable access/exchange of network/connectivity information (e.g. via APIs) to the 3rd party, UE and Network Functions.

Additionally, to enable service optimisations, the NextGen system should support the combination, collection and exposure of additional context information from/to Network Functions, UE and 3rd parties.

Solutions for this key issue will study the following aspects (non-exhaustive list):

-	Define network capability and context information exposure framework.

-	Identify information from the UE, external applications, Network Functions, and RAN that can be used, to create session/network context information.

-	Identify the mechanisms and interfaces to expose network capabilities and context information to the 3rd party, UEs and Network Functions.

-	Identify the network information and/or context information that can be provided to 3rd party ISPs/ICPs to the UE, and to Network Functions to enable more customized and efficient service provision:

-	For any identified information, how the operator network to acquire information and to allow the 3rd party, the UE and the Network Functions to access these information.

-	How to create the network slice based on the requirement of the 3rd party or customize network function on-demand.

Editor's note:	Whether the above bullet involves possible work of SA2 is FFS.

-	Support the existing network capability exposure within the next generation system architecture

-	Support APIs for exposure of new network capabilities and context information within the next generation system

NOTE 1:	Exposure of new system capabilities will be studied based on the exposure requirements captured in TR 22.861 [4], TR 22.862 [5], TR 22.863 [6] and TR 22.864 [7].

NOTE 2:	APIs are not necessarily in the scope of 3GPP.

Solutions for this key issue will assume that the next generation system should be able to expose network capabilities to the 3rd party and the UE and enable exchange of information in a secured way.

# End of New Text