SA WG2 Meeting #116	S2-164642

29 August – 2 September 2016, Sanya, P.R. China

Source:	Qualcomm Incorporated

Title:	Update of Consolidated Architecture option 3 in 7.3

Document for:	Discussion/Approval

Agenda Item:	FS_NexGen /6.10.22

Work Item / Release:	FS_NexGen / Rel-14

Abstract of the contribution: Proposes an update to the Consolidated Architecture option 3 in 7.3.

# Introduction

It is suggested to enhance the description of the consolidated architecture 3 to better define the differences between the multiple PDU sessions option and the single PDU sessions option when "local breakout" is needed. In addition, the multiple PDU sessions option is modified to clarify that the solution includes a common UP function that can act as “central” point for e.g. QoS enforcement, LI, and other functionality for multiple PDU sessions for a given UE and for the same data network.

# Proposal

It is proposed to add the following changes to section 7.3.

**** Start of changes ****

## 7.3	Consolidated architecture option 3

### 7.3.1	General

The NextGen network reference architecture should consider the following aspects:

1)	Support 5G use cases and service requirements

2)	Enable Operational agility (enable extreme automation (faster deployments, upgrades, reduce TCO)

3)	Allow independent evolution of different parts of the network (e.g. access and core).

Key Architecture principles:

-	Abstract the transport domain from 3GPP network functions to allow for independent evolution and to enable operators to use different transport technologies (e.g. Ethernet, MPLS, SDN-based transport, etc.). 3GPP network functions should neither mandate nor rule out support for any of these technologies in the transport domain.

Allow scalability of UP and CP functions independently

-	Allow for a flexible deployment of UP separate from the CP, i.e. central location or distributed (remote) location (i.e. with no restriction in the location).

-	Support transmission of different PDU types, e.g. IP, Ethernet

-	Separation of functions including subscription database from functions providing the end user service

-	Separation of Policy function to govern the network behaviour and end user experience

-	Allows for different network configurations in different network slices.

Control Plane:

-	It is important for the UE to trust that certain functionalities are supported in the network thus important to enable multi-vendor interworking between UE and network functions. However, from the UE perspective, it is irrelevant how and where it exists within the network (e.g. which function module or software supports a certain function).

-	It is important to enable multi-vendor interworking between radio and network functions within the core network and between the network functions within the core network. At the same time, it is sufficient if a single interface is exposed towards the radio while abstracting the modular (elementary) functions supported in the core network.

User plane:

-	A generic user-plane function (UP function) is defined, which supports various user-plane operations (incl. forwarding operations to other UP functions/data networks/the control-plane, bitrate enforcement operations, service detection operations, etc.)

NOTE1:	The detailed list of user-plane operations will be based upon the conclusion of key issue 4.

-	The control plane configures the UP functions to provide the traffic handling functionality needed for a session. One or multiple UP functions per session can be activated and configured by the control-plane as needed for a given user-plane scenario.

-	To support low latency services and access to local data networks, user plane functions can be deployed close to the radio. For central data networks, UPFs can be deployed centrally.

Concurrent access to local and centralized services is supported as follows

-	Multiple PDU sessions to both a local UP function (providing access to local data networks) and a central UP function (providing access to central data networks); or

-	A single PDU session, for which the control plane has configured two UP functions: one UP function performing traffic classification and traffic steering towards either the local data network or the central data network, the other UP function providing access to the central data network (as depicted in figure 7.1.3-3).

NOTE 2:	The control plane can also configure multiple UP functions in the single PDU session case for local data network access.

7.3.2	Reference architecture

Figure 7.3.2-1 depicts the non-roaming architecture functional view.

Figure 7.3.2-1: Non-roaming reference architecture

Figure 7.3.2-2 depicts the non-roaming architecture for UEs concurrently accessing a local and a central data network using multiple PDU Sessions. In this option, the UE may be connected to a specific data network both locally and centrally using two separate PDU sessions, established independently by the UE. For multiple PDU Sessions to the same data network for the same UE, it is possible to have a common NG Core UP Function supporting the multiple PDU Sessions.

Editor’s Note: whether a single common NG Core UP function is required for a UE, or multiple are possible for the various Data Networks, is FFS.

Editor’s Note: the need for a common NG Core UP Function depends on the solution for various key issues (e.g. QoS).

Figure 7.3.2-2: Applying Non-roaming reference architecture for concurrent access to local and central data networks (multiple PDU session option)

Figure 7.3.2-3 depicts the non-roaming architecture in case concurrent access to local and central data networks is provided within a single PDU session. Connectivity and routing of the UE data to the local network is decided by the CN based on operator policies.

Figure 7.3.2-3: Applying Non-roaming reference architecture for concurrent access to local and central data networks (single PDU session option)

The two options based on multiple PDU session and single PDU sessions differ on the following aspects:

-	In the multiple PDU sessions case, the UE requests explicitly multiple PDU sessions (e.g. based on service and application requirements, based on indication from the network in SSC mode 3, etc.).;

-	in the single PDU session case, the UE established a PDU session towards a data network. The CN, based on network policies, decides whether to also allow traffic to be routed locally, without the UE specifically requesting the type of connectivity required and without the UE explicitly binding applications or services to either the local or the central connectivity.

Following figure 7.3.2-4 depicts the roaming architecture in case of home routed scenario:

Figure 7.3.2-4: Roaming reference architecture - Home routed scenario

Following figure 7.3.2-5 depicts the roaming architecture in case of local break out scenario:

Figure 7.3.2-5: Roaming reference architecture - local breakout scenario

Editor's note: The choice between NG6* and NG6 depends on whether the PDU Session has a single IP address/prefix or multiple IP address/prefixes. It is FFS whether both types need to be supported. Whether the distinction between NG6 and NG6* is needed is also FFS.

Editor's note: The list of individual NG Core Control network functions (e.g. SMF versus MMF, or access specific versus access independent, etc.) is FFS. The interconnection model for all control plane network functions is FFS.

Editor's note: Need for NG8 from H-CCFs to NG-SDM (i.e. in the home routed scenario) is FFS.

NOTE 3:	Regardless of the number of CCFs, there is only one NAS interface instance between the UE and the CN, terminated at one of the CCFs that implements at least access authentication and mobility management.

NOTE 4:	Criteria to select multi-vendor open (standardized) interfaces should be determined.7.3.3	Network functions and reference points

The 5G Reference Architecture consist of the following functions:

-	NG Subscriber Data Management (NG SDM)

-	NG Policy Control function (NG PCF)

-	NG Core Control functions (NG CCFs)

-	NG Core User plane function (NG UPF)

-	NG RAN

-	NG UE

-	Data network, e.g. operator services, Internet access or 3rd party services.

The following is a high level split of functionality between the control plane and the user plane.

The NG Core Control functions include the following functionality:

-	Termination of RAN CP interface

-	Termination of NAS

-	Access Authentication

-	NAS Ciphering and Integrity protection

-	Mobility management

-	Session Management

-	UE IP address allocation & management (incl optional Authorization)

-	Selection of UP function, including selection of the NG Core UP Function connected via NG3 to the NG (R)AN of figures 7.3.2-2 and 7.3.2-3, and the UP function interfacing the NGCN to the Data Networks

-	Termination of interfaces towards Policy control and Charging functions

-	Policy & Charging rules handling, incl control part of enforcement and QoS

-	Lawful intercept (CP and interface to LI System)

NOTE 5:	Not all of the CCF functions are required to be supported in an instance of CCFs of a network slice

The NG Core User plane function includes the following functionality:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable)

-	External PDU session point of interconnect (e.g. IP).

-	Packet routing & forwarding

-	QoS handling for User plane

-	Packet inspection and Policy rule enforcement

-	Lawful intercept (UP collection)

-	Traffic accounting and reporting

NOTE 6:	Not all of the UPF functions are required to be supported in an instance of user plane function of a network slice.

In the two architectures in Figures 7.3.2-2 and 7.3.2-3, the NG Core UP Function connected via NG3 to the NG (R)AN performs the following additional functionality:

-	act as common QoS handling function for multiple PDU sessions of a UE (e.g. to enforce per-UE or per Data Network QoS parameters)

-	for the single PDU session model in Figure 7.3.2-3, act as UP function UL-CL (uplink classifier) for uplink routing of traffic to a local data network. For downlink, the operator configures the network routing such that only legitimate traffic from the local server may pass via the UP function local IP point of presence.

The NG Policy function includes the following functionality:

-	Supports unified policy framework to govern network behaviour.

-	Provides policy rules to control plane function(s) to enforce them.

Editor's note: The need for an interface between NG Policy Function and SDM is FFS.

The 5G Reference Architecture contain the following reference points:

NG1:	Reference point between the UE and the NG Core Control plane function.

NG2:	Reference point between the RAN and the NG Core Control plane function.

NG3:	Reference point between the RAN and the NG Core User plane function.

NG4:	Reference point between the CP functions and the NG Core User plane function.

NG5:	Reference point between the CP functions and an Application Function.

NG6:	Reference point between the UP functions and a Data Network (DN).

NG6*:	Reference point between a UP function and a local Data Network (when concurrent access to both a local and central data network is provided for one PDU session with a single IP address/prefix).

NOTE 7:	Details of NG6* mechanism are beyond the scope of 3GPP.

NG7:	Reference point between the NG Core Control plane function and the NG Policy Control function.

NG8:	Reference point between the NG Core Control plane function and the Subscriber Data Management.

NG9:	Reference point between two NG Core User plane functions.

NG7r:	Reference point between the V-PCF and the H-PCF.

NG-RC:	Reference point between the V-CCFs and the H-CCFs.

Editor's note: Whether additional reference points between UPFs need to be defined for other user-plane scenarios is FFS.

### 7.3.4	Applicable solutions

Editor's note: List the solutions from chapter 6 that are applicable to the architecture.

### 7.3.5	Evaluation

Editor's note: Evaluation for consolidated architecture option 1.

**** End of changes ****