SA WG2 Meeting #S2-116bis	S2-164962

29 Jul - 2 Sep 2016, Sanya, China

Source:	Nokia, Alcatel-Lucent Shanghai Bell, Verizon, ETRI, AT&T, Deutsche Telekom, Telicom Italia, Oracle

Title:	Solution for “Key issue on Network Function Granularity and Interconnection between them”

Document for:	Approval

Agenda Item:	NextGen/6.10.7

Work Item / Release:	NextGen/ Rel-14

Abstract of the contribution: This contribution propose to add a solution for control plane interconnection model in the Next Generation Network Architecture.

# Introduction

The control plane interconnection model specified for Next Generation Network architecture should enable implementation and delivery of software services with more agility. This also implies that 3GPP should carefully decide how functions are modularized, the functionalities exposed to external entities (including UE, 3rd party application server etc). Furthermore, it is important to carry over the modularity of the functions also in the corresponding external interfaces.

The high level requirements for network function granularity and network function interconnection model was agreed in SA2#113 and reference is S2-160754. Also, the problems faced with the existing architecture were outlined in S2-160094. So, this paper does not re-state the need and importance to re-visit the control plane interconnection model in the context of new architecture.

Requirements from S2-160094:

The solution for the interconnection of the control plane network functions should allow

- Network functions to be able to interact with each other, e.g. for new services and features, while avoiding functional and signalling impact to unrelated network functions for a given interaction

- Mechanism for the exchange of information between network functions that results in agile/rapid deployment of new services, e.g. mechanism that allows reuse of procedures, wherever possible

- Ability to deploy network functions in various network configurations

In this paper, we focus on showing how this should be approached. We propose the following architectural principles:

- It is important for the UE to learn about the functionalities that are supported in the network thus important to enable multi-vendor interworking between UE and network functions.

- It is important to enable multi-vendor interworking between network functions within the core network and within the access network.

- Minimize dependencies between function modules, e.g. decouple mobility management and session management as far as possible

- Enable a new modular UE – Core, RAN-core interface

- Contain access-specific functions in specific adaptation modules

- Remove overlaps in functional scope from network functions, e.g. consolidate session management into a single control plane function

- Consolidate session context state into a single data repository. This allows for control plane functions to be stateless.

The Data Layer (DL) provides the following services to the Core Control Plane Functions:

1)	distributed and real-time storage of structured data (i.e. allowing the control plane function to create, read, update, delete their own data and subscribe to notifications upon data change); In exceptional cases, opaque data can be stored by the network functions.

2)	possibility of data sharing between the Core Network Functions;

3)	data redundancy.

The Data Layer allows:

a)	for the Core Control Plane Functions to be stateless;

b)	for cloud-based RT data storage and flexible deployment of data and Core Control Plane Functions in centralized or distributed clouds;

c)	Scalability and Elasticity – scale-in, scale-out functions can be leveraged for core control plane functions and the data layer. The data layer can adapt itself to the application functions.

d)	Network can store context in the DL for Massive number of IoT devices (e.g. application VNF resources do not have to be used up for such devices when they are in IDLE mode).

c)	data analytics and network optimizations (e.g. minimizing mobility management signaling for stationary devices);

d)	for avoiding data duplication between the Core Control Plane Functions. This also helps avoid signalling incurred due to transfer/synchronization of data between Core Control Plane Functions.

e)	moving from a message centric towards a data centric network architecture, permitting optimized call flows.

f)	robustness and high availability (seamless Core Control Plane Function's failure).

To:

As stated in the annex E (E.1.1) (snippet below), we propose to define the service and functionality of the Data Layer and SDM without having to fix the service requester. This could then be re-used by any service requester (i.e. application) within the network.

“Any functionality or service involves two types of network functions: service-requester and service-provider. If a functionality or service is defined without fixing the service-requester then it could be re-used by any service-requester within the network.”.

# Proposal

It is proposed to add the following Solution on Network function interconnections to the TR 23.799 “Study on Architecture for Next Generation System”.

* * * Start of changes * * * * (all new text)

## 6.7	Solutions for Key Issue 7: Network function granularity and interactions between them

### 6.7.1	Solution 7.x: Control Plane Interconnection model with a Data Layer

This is a solution for key issue #7 and key issue #9.

The main principle of this solution is to store (expose) UE related context data to a data layer. This can include both near real-time storage of structured data and opaque data.

It addresses Key Issue #7 as it impacts how Network functions interact with each other.

It addresses Key Issue #9 as it impacts how and what data can be exposed within MNO domain and to 3rd party.

#### 6.7.x.1	Architecture description

##### 6.7.x.1	Reference model

This solution assumes the following reference model for non-roaming and roaming Data Layer interconnection model.

Figure 6.7.x.1-1: Non-roaming Data Layer interconnection model

Figure 6.7.x.1-2: Roaming Data Layer interconnection model

Figure 6.7.x.1-3: Data Sharing between Core Network Nodes

NOTE: The location(s) of component(s) of the Data Layer is up to deployments to decide.

It should be noted that access to Context Data and Subscriber Data have different characteristics in terms of expected location, frequency of data access etc. While the subscriber data can be placed in a centralized location (e.g. NG SDM/HSS), context data (context data for a certain network function) should be placed close to the network function to reduce latency for data access and to meet the necessary performance criteria.

NG8:	Reference point between the Core Network Functions (e.g. CCF, PCF) and the Subscriber Repository.

NGx:	Reference point between the Core Network Functions (e.g. CCF, PCF) and the Data Layer.

##### 6.7.x.1	Architecture principles

Editor's Note: This clause will contain e.g., terminology, overview, architecture description of the solution.

Following are the architecture principles that are proposed for the control plane interconnection model:

- It is important to enable multi-vendor interworking between network functions within the core network and within the access network.

- Minimize dependencies between function modules, e.g. decouple mobility management and session management as far as possible

- Remove overlaps in functional scope from network functions, e.g. consolidate session management into a single control plane function

- Consolidate session context state into a single data repository. This may allow for control plane functions to be stateless.

#### 6.7.x.2	Function description

##### 6.7.x.2.1	Data Layer (DL)

The Data Layer (DL) consists of session data. It provides the following services:

1)	distributed and near real-time storage of structured data (i.e. allowing the control plane function to create, read, update, delete their own data and subscribe to notifications upon data change); In addition, opaque data can also be stored by the network functions. 	the network functions can store context as a secondary storage and also optionally use it for primary storage. Whether the network function uses the data layer for primary storage for context can be an implementation choice.

2)	possibility of data sharing between same type of Core Network Functions as in Figure 6.7.x.1-1 and different type of Core network functions as in Figure 6.7.x.1-3; In the latter case, it is also assumed that only the network function that stored the data owns the data (e.g. a CCF creates, updates, deletes the own data such as ULI) and other network functions can have access it or subscribe to notifications upon data change (i.e. PCF reads the ULI information).

3)	data redundancy.

j)	the network functions to store context as a secondary storage and also determine when it should be used for primary storage. Whether the network function uses the data layer for primary storage for context can be implementation choice.

##### 6.7.x.2.2	Core Control Plane Function (CCF)

Control plane functions are grouped into procedure sets that handle a number of related, ideally self-contained control-plane procedures. A frontend terminates external protocol interfaces, including message ciphering/integrity protection and performs load balancing towards processing entities. Frontends have a coordination function for collision detection & resolution, sequencing and/or selection of procedures.

Figure 6.7.x.2.2-1: Core Control Plane Function (CCF)

#### A network implementation based on Network Functions only relying on standardised state information and using the data layer as the primary and only storage for their state information could enable the following example scenario illustrated below:

#### At first UE attach RAN selects a CCF in the selected slice based e.g. on load balancing algorithms, For subsequent transactions RAN routes the NAS signalling from the UE to a CCF in the serving network slice based on the NSSAI provided by the UE and according the same logic, e.g based on load balancing algorithms. The stateless CCF in the slice that receives the NAS message will retrieve the user’s Context Data from the Data Layer and will execute the transaction.

#### 6.7.x.3	Example Call flows

#### NOTE: The following example call flows are presented for illustration of the concept of the exposure of UE related data and the scenarios the data layer could be used for.

##### 6.7.x.3.1	 Detach/Attach scenario

Figure 6.7.x.3.1-1 shows the example call flow of a UE re-attaching in the same PLMN to which it was previously attached. As part of the previous Attach scenario, the Core Control Plane Functions (CCF) stored UE related data (including subscription data downloaded from the SDM) in the Data Layer.

During the next Attach, the new CCF serving the UE (CCF in the call flow) accesses the data stored in the Data Layer and proceeds with the Attach procedure without necessarily requiring the signalling towards the NG SDM.

Figure 6.7.x.3.1-1: Detach/Attach

##### 6.7.x.3.2	 CCF Failover

Figure 6.7.x.3.2-1 shows the example call flow of a UE initiating signalling towards the Core Control Plane Function (e.g. Mobility Management or Session Management signalling) after a CCF failure.

The new CCF serving the UE (CCF 2 in the call flow) accesses the data stored in the Data Layer and proceeds with the UE request successfully.

Figure 6.7.x.3.2-1: CCF Failover

##### 6.7.x.3.3	 Successive UE transactions served by different stateless CCFs

Figure 6.7.x.3.2-1 shows the example call flow of a UE initiating consecutive UE transactions over time, involving different CCPFs (e.g. resulting from scale-in or scale-out operations).

The CCF serving the second UE request (CCF 2 in the call flow) accesses the data stored in the Data Layer by the previous CCF (CCF1 in the call flow) and proceeds with the UE request.

Figure 6.7.x.3.3-1: Successive UE transactions served by different stateless CCFs

##### 6.7.x.3.4	 UE location reported to PCF

#### Figure 6.7.x.3.4-1 shows the example call flow for a UE location being reported to PCF, CEF.

User Location (changes) are notified to the PCF/CEF via subscribe/notify procedure. Access to read/subscribe to User Location needs to be controlled by the VPLMN to enable a network entity from a particular PLMN to access the ULI of a particular UE under the control of the CCF and/or the data layer.

Figure 6.7.x.3.3-1: User location Information subscription and notification

##### 6.7.x.3.5	 Context Clean-up scenario

Figure 6.7.x.3.5-1 shows the example call flow of a UE context clean-up triggered by CCF.

Figure 6.7.x.3.5-1: Context Cleanup

#### 6.7.x.4	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * End of Changes * * *