SA WG2 Meeting #116BIS	S2-165241 was 4700

29 August – 2 September 2016, Sanya, China	(revision of S2-16xxxx)

Source:	ZTE, Interdigital, ETRI, Oracle. Samsung, Nokia

Title:	Network Slicing Roaming Support (Based on Solution#2)

Document for:	Approval

Agenda Item:	6.10.1

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: Proposed solution for roaming support based on Network Slicing Solution#2

# 1	Introduction

There was discussion in last SA2#116 meeting from operator who would like to see the architecture proposal for roaming support in Network Slicing.  The intent of this proposal is to present the roaming architecture that is based on Solution#2.

***** First Change *****

### 6.1.x	Solution 1.x: Roaming architecture for Network Slicing based on solution#2

Figure 6.1.x.1-1 depicts the non-roaming architectural concept based on solution#2 for the Core Network Slicing. Figures 6.1.x.2-1, 6.1.x.3-1 and 6.1.x.4-1 depict the roaming architecture scenarios. Among all these architecture, the Core network slice is sharing some network functions with other slices for that serve the same UE, including the NG1 and NG2 terminations, at the CCNF.

#### 6.1.x.1		Basic Non-roaming architecture

The following non-roaming scenario is based on network slicing solution#2 as captured in clause 6.1.2.

Figure 6.1.x.1-1: Core Part of Network Slicing concept (Non-roaming scenario)

Editor’s note: The support of NGcp between the CCNF and the Policy Control Function (PCF) is at the NSI level.  Such functionality is dependent on the outcome of  NextGen policy framework as documented as KI#10 in clause 6.8.10.

#### 6.1.x.2	Home-routed	roaming architecture

Figure 6.1.x.2-1: Core Part of Network Slicing concept (Home Routed roaming scenario)

In all roaming scenarios, it is possible that, the control plane and the user plane of the Core part of the NSI may be split between the visited and the home PLMNs (e.g. SM, SC etc.).  Such design consideration is very similar to today EPC roaming scenario.

The assignment of network function at the HPLMN corresponding to the requested slice type from the VPLMN is the decision of the HPLMN.

Based on the roaming agreement, during NSI selection, the serving CCNF uses the subscriber information it has obtained from the HPLMN Subscriber Repository, which includes the subscribed MDD,  the subscribed DNN and possibly other information (e.g. to support the selection of the network function entities corresponding to the subscribed  MDD and subscribed DNN in HPLMN) to bind the subscribed MDD with a MDD value in the VPLMN.

The serving CCNF replaces the serving PLMN MDD with the corresponding HPLMN subscribed MDD (which was bound to it in NSI selection phase) in signalling it routes to Slice Specific Network functions which may require the Slice Specific Network Functions in the VPLMN to interact with Slice Specific Network Functions in the HPLMN. The Routing of this signalling from VPLMN Slice Specific Network Functions to HPLMN Slice Specific takes into account the subscribed MDD.

If there is no roaming agreement to support Network slicing between a HPLMN and a VPLMN, the VPLMN assigns the UE to an inbound roaming slice and the selection of the HPLMN functions is based on the DNN only.

.

#### 6.1.x.3	Local breakout roaming architecture supporting Home PLMN Application Function

Figure 6.1.x.3-1: Core Part of Network Slicing concept (Local Breakout roaming scenario with Home Operator’s Application Function)

In the above local breakout roaming scenario, the visited PLMN hosts the NSI to serve the UE’s home PLMN’s application function.

#### 6.1.x.4	Local breakout roaming architecture supporting Home PLMN Application Function

Figure 6.1.x.4-1: Core Part of Network Slicing concept (Local Breakout roaming scenario with Visited Operator’s Application)

In the above local breakout roaming scenario, the visited PLMN hosts the NSI to serve for its local application function.

#### 6.1.2.3	Solution evaluation

Editor's note:	This clause will contain evaluation on the system impacts, e.g. UE, access network and non-access network.