SA WG2 Meeting #116BIS	S2-1645129

29 August – 2 September 2016, Sanya, China	(revision of S2-164447)

Source:	ETRI, KDDI, KPN, Nokia,Oracle, Samsung, ZTE,

Title:	Network Slicing Solution#2 Update

Document for:	Approval

Agenda Item:	6.10.1

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: Proposed updates to Solution#2 in clause 6.8.2

# 1	Introduction

Based on some discussions offline since last SA2#116 meeting, the following changes are provided in this revision:

- Proposing new interfaces in NextGen to interconnecting to the CCNF and Slice Specific Core Network Functions

- Adding NFI selector to initiate the Network Function Instances discovery and interconnection as supported by KI#7 when receiving the PDU Session Request

- Adding Policy Control Function support for network slicing policy support

- Re-arranging and clarifying some of the existing descriptions

- Clarifying some of the terminologies – i.e. Tenant/Tenant ID, NFI Selector

- Adding new control flows for PDU session establishment and management

- A companion paper S2-164665 clarifies the procedures for addition or removal of a slice

***** First Change *****

### 6.1.2	Solution 1.2: Network slice selection

This is a solution for key issue 1 on support for network slicing. This solution is focused on supporting the Group-B type of network slicing as described in Annex-D. The UE related NG2 and NG1signalling are handled by a set of Common Control Network Functions (CCNFs) as shown in figures 6.1.2-1 and 6.1.2-2. Figure 6.1.2-1 depicts the non-roaming architectural concept of this solution to support the Core Network Slicing. In this architecture, the Core network part of the Network slice is sharing some network functions with other Core Network part of Network slices which serve the same UE, including the NG1 and NG2 terminations, in the CCNF. In the remainder of this Solution description we may refer to the Core Network part of a Network Slice as Core Network (or CN) Slice, and to the Radio Network part of a Network Slice as RAN Slice.

Figure 6.1.2-1: Core Part of Network Slicing concept (Non-roaming scenario)

In this architecture, once the UE is assigned to CCNF during the attach procedure, as described below in subclause 6.1.2.2.3.1, the signalling connection between the UE and the CCNF is based on the UE Temporary ID, i.e. the RAN part of the network slice routes UE signalling to the CCNF based on the Temporary ID the UE provides in RRC signalling. The CCNF is logically part of all the Network Slice Instances (NSIs) that are being used for the UE. So when a new NSI is added or an existing NSI is removed to/from the set of slices used by a UE, a more optimal CCNF may have to be selected to serve the new set of NSIs used for the UE, thus triggering a CCNF redirection procedure.

Figure 6.1.2-2 shows UE1 and UE3 with 3 slices accessing multiple core network slice instances (NSIs) and therefore  multiple Slice-Specific Core network Functions (SCNF) (see section 6.1.2.1) via CCNF-1;  UE2 is associated with 1 NSI and is assigned to different CCNF-2 after the UEs attach to has occurred. Note that, prior to the UE being assigned to a Serving CCNF during the attach procedure (e.g. due to the processing of the network policy or subscription policy), a UE signalling may be first routed by the RAN to a default CCNF based on RAN Routing policy, if the UE has no Temporary ID yet.  Further details on the CCNF assignment are described in the later sections.

Figure 6.1.2-2: UEs assigned to Core part of NSI after Attach.

#### 6.1.2.1	Architecture description

This solution is based on the following fundamental architecture principles for network slicing:

-	In this proposed architecture, the control plane of the NextGen Core is partitioned into two types of Network Functions (NFs).

-	Common Control Plane Network Functions (CCNFs), and

-	Slice-specific Control Plane Network Functions (SCNFs)

The CCNF is the set of fundamental control plane network functions to support basic functions operation common among the NSIs within a NextGen Core. The CCNF functions are:  Subscriber Authenticator and Authorisator, Mobility Management, Network Slice Instance Selector (NSI Selector), Network Function Instance Selector (NFI Selector) and NAS Routing Function.  In order to support the CCNF operation, CCNF uses the Subscriber Repository to support UE authentication and for UE subscription data retrieval (e.g. operator's policy towards the UE's requested service, QoS and security policy etc.)

-	To support the selection of a particular NSI for a UE, this solution proposes that an MDD (Multi-Dimensional Descriptor) may be configured in the UE. The definition of  the MDD is provided below in clause 6.1.2.1.1. The UE may providethe MDD in RRC and in the NG1 (NAS) signalling during the UE attach. The RRC layer may route NAS signalling to a CCNF based on the assistance of MDD during the attach procedure, if the UE does not provide a valid Temporary ID for the PLMN, if available.  Once the CCNF is selected at attach time, a Temporary ID is provided to the UE for the subsequent UE’s NAS messaging to its serving CCNF. This Temporary ID is included in the RRC signalling to enable such routing of NAS signalling.

- By leveraging also the UE's capability and subscriptions data from the subscriber repository in addition to the MDD, the UE is authorized at attach time to access certain NSIs that correspond to certain MDD values, and the target NSI(s) are selected by the NSI selector of the CCNF. This creates a context in the CCNF for the given UE which defines the binding between the MDD vectors and the corresponding NSIs.  Any PDU session establishment in these NSIs may happen via subsequent SM signalling after attach, or related SM signalling containers could be embedded in the attach request.

Note that, while NSIs are selected, it is possible thethe CCNF that was initially selected by the RAN (e.g. default CCNF) for the UE is not the most optimal based on operator policies, so a CCNF change may happen and the UE is redirected to an optimal Serving CCNF; otherwise, the default CCNF becomes the Serving CCNF if it is considered suitable based on operator policy.  If the UE is not configured with an MDD, the network may provide a default MDD at attach time based on UE subscription information and other criteria as discussed below.

Once the NSIs applicable to the UE are selected and with that the Serving CCNF, the Temporary ID and the Accepted MDD may be used by the UE for the subsequent NAS signalling as necessary for each individual NAS message.

-	It should be possible for the UE or a PLMN to change the set ofNSI(s) a UE is using depending on the type of application and service it requires, or depending on changes to UE's subscription or operator's policy etc. This may depend on factors such as UE capabilities, configuration and authorization.

Editor's note: It is FFS on how to perform the steering to a different NSI.

Editor's note: The non-3GPP access aspects are FFS.

##### 6.1.2.1.1	Network Slice and Network Function selection

In order to perform NSI selection, the NSI selection principle should enable the selection of the appropriate set of network functions to deliver a certain service and/or for a certain use case.

A network slice fulfils at least a couple of purposes for a UE:

1.	It offers a UE particular system behaviours tailored to specific application needs, from the standpoint of specific control plane (e.g. a UE may not need MT procedures, or require optimal behaviour for massive MTC, or Critical communications, etc.) or user plane behaviours (e.g. the UE may need a slice supporting header compression)

2.	It offers a UE access to resources allocated for a specific Service or Application domain, or a Tenant (e.g. minimum level of guaranteed resources or aggregate number of subscribers allowed to access the service at any point in time)

In summary, Tenant represents an organization, agency, application (or application class) or business entity which is entitled to access the service for the use of guaranteed network resources through a predefined Service Level Agreements and Policies with the network operator.

For example, a Tenant of PLMN resource e.g. is a wholesale customer of the PLMN ( e.g.a big company, or an agency which requires a PLMN to provide at least access to a predefined set of resources, or some specific policies in handling its slices subscribers at times of congestion). APLMN may apply tenant specific policies at times of overload.  An example of a tenant may be some public safety agency. The tenancy concep may also be needed to meet application specific requirements. For instance let us consider an enterprise deployment: in the enterprise, we may have a factory floor set of devices and devices that are associated to non factory floor operations. The enterprise may have a policy to allocate at least 60% of resources (whether in the RAN or in the Core) to factory floor operation at times of congestion but not to segregate resources at all times. So the enterprise may set up a tenanted slice so that factory floor devices get at least 60% of resources at times of congestion.

Therefore at least two dimensions can identify a slice and these are the components of a Multi Dimensional Descriptor (MDD) vector:

-	one identifying a tenant  identified by the Tenant ID component of an MDD vector

-	one identifying the network behaviours for the target network service (identified by the Slice Type component of an MDD vector (e.g. eMBB service, CriC, mMTC or other behaviours which may also be operator specific).

An MDD vector can be PLMN specific or standardized. When a MDD vector is PLMN specific, the UE stores the PLMN ID of the PLMN that assigns the MDD vector. The UE shall not use this MDD vector outside the PLMN that assigns it as it identifies a PLMN specific slice, not available elsewhere. Standardized MDD vectors can be used in all PLMNs. Federations of PLMNs may agree on common values of MDD vectors and support the same slice types, but this outside the scope of 3GPP.

This is to say, in summary, that the value of Tenant ID and Slice Type may be standard or operator specific.

The Tenant ID field may have a standard value that indicates to the network that it is to be ignored (so the MDD vector is effectively only based on one dimension, i.e. the Slice type field).

Information such as UE's capability, UE's location (based on the UE's serving access) may also be used to support the UE authorization to access and to select a slice, and therefore to be authorised to use a certain MDD vector in the PLMN.

The MDD is a matrix that indicates in each of its rows a slice the UE may request to access or address. If the NSI to be accessed is only one, the MDD is just a single MDD vector. A row in the MDD is referred to as MDD vector.

For instance, UE1 in Figure 6.1.2.1.1-1 may indicate the following MDD at attach time to request the network to allow it to use slices from the same tenant in parallel.

| Tenant ID=1 | Service Descriptor/Slice type = 1 |
| --- | --- |
| Tenant ID=1 | Service Descriptor/Slice type = m |

However, for example and illustrative purposes only, if the UEs just needed to access the first slice of the two in the MDDs above, to e.g. send UL data for that slice, they both may indicate to the network in the RRC signalling needed to establish the RRC connection the following MDD

| Tenant ID=1 | Service Descriptor/Slice type = 1 |
| --- | --- |

A network may configure the RAN to use the whole MDD or part of it, if available in RRC signalling, to assist the routing of NAS Attach request messages in a RRC connection that was established without the indication of a valid UE temporary ID for the PLMN. This allows the Attach request to be routed to a more suitable CCNF which would otherwise be selected if the network would just apply a load balancing based policy without using the MDD.

For access control purposes, or overload control, and in general for the NextGen system slicing support, the RAN may be configured to act on the whole MDD or just a part of it.

The network may also refer to the MDD alongside other information (e.g. subscription, UE capabilities) available to the network to choose the appropriate Network Slice Instances and network functions which the UE can use.  The Attach accept includes the MDD indicating the MDD vectors of the active NSIs the Ue has been authorised to use.

The UE may be provisioned with MDD values to be used or the UE may obtain these from the network during the attach procedure.

The UE uses the last accepted MDD vectors to address the active set of NSI.

For the case of PLMN-specific values of the MDD fields, they are always provisioned as a (PLMNID, Value) pair and the UE shall not use these when camping on PLMNs different than the one associated to the value, (e.g. if a Slice Type is applicable just to a certain PLMN, it shall be stored in the UE as (PLMN ID, Slice Type).

Figure 6.1.2.1.1-1 depicts the semantics intended by the MDD with UEs that can access a single tenant slices. The case of  UEs accessing multiple tenant NSIs is also possible as long as these NSIs do not require their dedicated CCNFs

Figure 6.1.2.1.1-1: Network with n Tenants and m possible Slice Types (with UEs which can only access a single tenant slices)

In Figure 6.1.2.1.1-1, we have a network that is supporting n tenants and up to m network behaviours (slice types) in both the RAN and the core.

It is assumed to be possible for the RAN to dedicate resources to a slice.

Editor's note: How RAN implements this (RAN slicing or not) is FFS and shall be decided by RAN, including whether certain slice types require specific RATs.

The indication of the MDD (vectors) in the RRC layer during RRC connection establishment allows the RAN to apply Congestion management policies based on MDD (vectors), RAN WGs should decide which exact RRC messages shall include the MDD if available in the UE..

A CCNF that serves the PLMN can also serve multiple tenants as long as all these tenants are hosted by the same PLMN and these tenants do not request their dedicated CCNFs

In the Core Network, receiving the MDD in NAS signalling (and maybe also in RAN-CN signalling) allows the core to handle incoming NAS signalling in accordance to policies specific to both the Tenant ID (from a resource isolation and congestion handling standpoint) and the Slice Type (from a dedicated network behaviour standpoint). It also may allow the core network to understand which  NSI or a set of NSIs that a UE may request to be selected out of a subscribed set of MDD vectors (if available) applicable to the PLMN.

In summary, NSI selection criteria should enable selection of a correct NSI for a certain application/tenant. This is a role of the NSI selector function in the CCNF.

When a PDU Session Request is sent by a UE for a certain NSI, the NFI selector will then trigger the corresponding set of control plane NFIs within the serving core NSI for the network function discovery and interconnection among themselves.

Editor’s note: Further details on NFIs discovery and interconnection are discussed in the separate work task related to KI#7.

#### 6.1.2.2	Function description

##### 6.1.2.2.1	Functions.

CCNF: 	the Common Control Network Functions is a group of functions that are shared across the slices concurrently supported for a UE

SSNF: 	Slice Specific Network Functions is the set of functions of a Network Slice that are not shared with other Network slices concurrently supported for a UE. It includes Slice CP Network functions and Slice UP network functions.

Slice CP Network functions: 	Control plane functions applicable to a SSNF

Slice UP network functions: 	User Plane functions applicable to a SSNF

PCF: 	the Policy Control Function, controls the Slicing Selection policies in CCNF and also Policies related to UE handling inside a SSNF

Subscriber Repository: 	Repository of subscription information

##### 6.1.2.2.1 	Reference points

NGc:	Reference point between the CCNF and the Slice CP Network functions, used to exchange control information

NGr:	it is used by the CCNF to access subscription data

NGcp:	It is used by the CCNF to obtain e.g. UE Mobility polices.

NGs:	It is used by the Slice CP Network functions to access subscription data

NGp:	It is used by the Slice CP Network functions to obtain Policy decisions (e.g. related to QoS or charging aspects for a session PDU flows).

##### 6.1.2.2.1	High level operation description

The binding of a UE to a set of Network Slice Instances happens when the network receives the Attach request. The set of these NSI's that are bound to a UE is known as "Active Set of NSIs". This is a high level description of the steps that are followed:

1.	When a CCNF receives the attach request, it checks the UE has valid credentials and validates its IMSI (in the equivalent of an IMSI attach) or it binds the Temporary ID the UE provided to its IMSI (If the attach uses a Temporary ID). In the process, the network gains access to subscription data for the UE and a (new) Temporary ID may be assigned by the CCNF that has received the UE signalling.

The functions of CCNFs are described as follows:

a)	Subscriber Authenticator and Authorisator: is responsible to authenticate the UE for accessing the NextGen system as well as to maintain the context for UE's identity, capability and service subscription etc. to support Network Slice Instance selection.

b)	Mobility Management: is responsible for supporting the UE registration and MM status in the operator's network, keeping track of UE location, reachability, providing roaming support, and supporting mobility restriction

c)	NSI Selector: is responsible for selecting specific core network slice instance based on the MDD and other information provided by the UE (e.g. UE capabilities) and the subscriber information retrieved from the Subscriber Repository.

-	It is assumed that the NSI Selector is aware of the presence of the NSIs that were provisioned by the Network Management which is responsible to maintain the life cycle of the network slice instances.  In addition, the NSI Selector is programmed with the intelligence to interpret the MDD, the UE capability, UE location and the subscriber information to select the appropriate core part of the NSI. When a specific NSI is selected based on a certain MDD vector value, the binding between NSI and the corresponding MDD vector is included in the UE's context in the CCNF.

d)	NAS Routing function is responsible for relaying NAS message between the UE and the NSI Slice specific part, if applicable (e.g. SM messages)

e)	NFI Selector is responsible for initiating the NextGen Core CP NFIs discovery among themselves as well as interconnecting with each other when the UE’s serving NSI receives the PDU Session Request.

Editor’s note: Further details on NFIs discovery and interconnection are discussed in the separate work task related to KI#7.

2.	If the UE provides no MDD in the Attach Request, default policies in the PLMN apply which could be based e.g. on subscription information. In such case, the CCNFassigns the UE to  Default set of Slice Instances and the related network selected MDD vectors are passed to the UE in the Attach Accept. 

If the UE provides an indication of initially requested MDD, then the NSI selector verifies the UE is allowed to access the corresponding NSI(s).  Upon successful authentication and authorisation, the network returns the same MDD to the UE unless the operator policy has changed for the UE due to UE capabilities, UE's subscription and/or UE's serving RAN type and therefore the NSI selector is not allowing all or part of the MDD vectors to be associated to a NSI.  If happens, the requested MDD will then be revised prior to be included in the Attach Accept sent to the UE for the subsequent use. 

The allowed NSI assignment to the UE can be based on the UE capabilities, assuming that the subscriber may use different types of UEs by swapping the UICC. 

In summary, it is the evaluation of the received combination of IMSI, IMEI-SV, requested MDD, UE capabilities, RAN type as well as the UE's subscription which determines which Accepted MDD is returned to the UE in the Attach Accept message. 

In this step, the UE may be assigned to a different Serving CCNF different than the one the RAN had selected if the resulting set of NSIs is better served by another CCNF, or due to load considerations. In this case, the Temporary ID assigned to the UE in the Attach Accept Message is corresponding to this finally selected Serving CCNF.

3.	The Accepted MDD is also included in the NG2 protocol used to carry the Attach Accept message (and other DL NAS messages) to the UE so that the RAN is aware of the active set of slices for the UE. 

In general in all the cases where the RAN needs awareness of Active Slice set or applicable NSI for a transaction, the MDD or applicable MDD vector should be included in messages that the AS layer can interpret.

4.	The UE, at NAS transactions subsequent to the successful attach, exiting from Idle state, will include the Temporary ID and the MDD in the RRC layer if these eventually result in slice specific resources to be required or other slice-specific outcomes.

5.	The RAN handles RRC messages without a MDD in the RRC connection request using a default behaviour.

6.	When a Temporary ID is present in RRC message, the Routing of NAS messages to the correct CCNF by the RAN is always based on the Temporary ID regardless MDD is present or not. If the Temporary ID is not present, then a default routing is used. This default routing of NAS messages could be assisted (and made more accurate) also if the UE includes the MDD in RRC messages.

##### 6.1.2.2.2	Definitions used in the solution

Temporary ID:  Identifier, similar to a GUTI, which logically associates the UE to a CCNF for the UE in a PLMN on a temporary basis. It is composed of a Routing information (necessary for the RAN to route the NAS messages carried in the RRC layer to the right contact point for the selected Serving CCNF), and a value assigned by the CCNF which is used to uniquely identify the UE context it retains. If the Temporary ID is not indicated to the RAN, or it is not recognised in the RAN, the RAN applies a default routing behaviour.

Multi-Dimensional-Descriptor (MDD): The multi-dimensional descriptor of a slice is typically composed of MDD vectors each comprised of a Tenant ID field that identifies an Application (e.g. a popular OTT service) or a tenant (e.g. a certain company that requires certain minimum resources in the network) and a Slice Type (which may be standard -e.g. eMBB, mMTC, criC - or operator specific). There may be default values for MDD. The MDD is used to support the Slice Instance selection in the core and in the RAN. If an MDD vector is PLMN-specific, it is not used outside the PLMN that assigns it.

Tenant ID:  Identifier of the Tenant which represents an organization, agency or business entity that is entitled to access the service for the use of guaranteed network resources through a predefined Service Level Agreements and Policies with the network operator.

##### 6.1.2.2.3	Example procedures

###### 6.1.2.2.3.1	Initial Access

Figure 6.1.2.2.3.1-1: Initial Access

1.	The UE sends and attach Request including the IMSI if a Temporary ID for the UE is not available.

If the UE requests one or a set of NSIs to be bound to it initially, it can do so by including a Requested MDD in the Attach Request. The Requested MDD may also be included in the RRC layer to further enhance the routing to an initial default/target CCNF of the Attach Request message in the event when the Temporary ID is not available to the UE, or the Temporary ID was not assigned by the same PLMN of the current network that serves the UE. The MDD in the RRC layer may also be included to enable the access to a suitable RAN resources. It is assumed the RAN maintains a MDD-based  table in order to route/forward the attach request message to a proper CCNF when the Temp ID is missing.

If the Temporary ID is available, the UE includes it in the RRC layer message used to establish the RRC connection, so that the RAN can route the message to the CCNF in the core which has generated it

2.	The RAN forwards the Attach Request to the Core based on the routing criteria outlined in step 1.

3.	The CCNF proceeds with the procedures for 3GPP system authentication and authorization for the UE .

4.	If the UE is successfully authenticated, its subscription data is checked and the allowed MDD vectors are decided (which will form the Accepted MDD which will be returned to the UE in the Attach Accept),  alongside the compatibility of the CCNF with these MDD vectors. It the CCNF detects  it is not an appropriate handler for the UEstep 5 is executed immediately without binding of NSIs to MDD vectors occurring in this CCNF.

The CCNF decides the initial set of NSI(s) for the UE based on an evaluation of the Requested MDD, Subscribed MDD (if available), UE capabilities, UE's subscription policy, UE's serving RAN type etc. The following may apply:

- if the UE did not provide the Requested MDD, the network assigns the UE to the default NSI (s)
- if the UE did provide the Requested MDD, the network assigns the UE to the NSI(s) that the UE is authorized for.

- if some Default NSI(s) was missing from the Requested MDD, the UE is still assigned to these NSI(s) and the corresponding MDD vector(s) is added in the  Accepted MDD

The Accepted MDD and is passed to the UE in the Attach Accept message.

If some DNNs are specific to some MDD vectors (e.g. based on subscription data), the mapping of these DNNs to MDD vectors is recorded in the UE context in the CCNF.

The procedure continues to step 8.

5.	If the UE is not suitably handled by the (Default/Target) CCNF where the Attach Request was routed to, this CCNF may redirect the UE to a new Serving CCNF that is more optimal (or less loaded) for the selected slices. Then, the Default/Target CCNF forwards the attach request to the new Serving CCNF with an indication that it is a forwarded attach together with IMSI , the MM context including the NAS key and Accepted MDD (which was determined to evaluate CCNF compatibility in step 4)  in order to indicate that the UE has been authenticated for NSI(s) Assignment as described in step 4.

6.	The Selected Serving CCNF performs the NSI selection as described in step 4 above.

7.	The Selected Serving CCNF binds the UE to the selected NSI(s) by creating the related context associating the Accepted MDD vectors to the NSI(s) for the UE and then sends back the Forwarded Attach Accept message with Temporary ID and the Accepted MDD for subsequent usage by the UE. If there are any DNNs that are specific to any MDD vectors, the mapping is passed to the UE also.

8.	If the steps 5 to 7 were executed, the Serving CCNF sends to the RAN the Attach Accept in a NAS message including the content as described in the message in step 7 above. Otherwise, the Default/Target CCNF, which is now the Serving CCNF, binds the UE to the selected NSI(s)  by creating the related context associating the Accepted MDD vectors to the NSI(s) and then sends Attach Accept message with Temporary ID the Accepted MDD  for subsequent usage by the UE and any DNN to MDD vector mappings. The Serving CCNF includes the Accepted MDD in the NG2 transport.

9.	The RAN forwards the Attach Accept received in step 7 or 8 to the UE

It is possible to include in the Attach Request an SM container so that the PDU sessions are established for all or a subset of the Active NSIs for the UE. Alternately a subsequent SM message exchange could be used to establish the PDU sessions for the NSIs that need these.

Editor's note: How the NSI matching a certain MDD is obtained in steps 4 and 6 is to be decided together with SA5.

6.1.2.2.3.2	Subsequent NAS signalling - PDU Session Request

The following procedure describes how a PDU session can be established with an existing NSI identified by a certain accepted MDD vector.

Figure 6.1.2.2.3.2-1: Subsequent PDU Session Request Slicing Procedures

- The UE initiates the PDU Session Request message to request for the service by including the Temporary ID, the MDD vector of the NSI that the PDU session applies to, the DNN of the PDN connection and the related parameters (e.g. QoS).  The UE includes the routing field of the Temporary ID in the RRC layer message for the RAN to route the NAS Message to the appropriate Serving CCNF. If the DNN is omitted a default or a set of default DNN connections in the NSI are established.

- The RAN routes and forwards the NAS message to the proper Serving CCNF based on the information in step 1.

3-4.  Serving CCNF verifies with Subscriber Repository if the requested DNN is subscribed- for the NSI that is identified by the MDD vector. If the DNN is allowed the NFI Selector forwards the PDU Session REQ to the target NSI.

5.	SM function will then proceed with the PDU Session Establishment procedures accordingly and the operation details should refer to KI#4.

6-8.	Once the step 6. above is complete, the serving NSI completes the PDU session establishment procedure by sending the PDU Session Response to the UE through RAN.

6.1.2.2.3.3	Subsequent NAS signalling for PDU Session Modification

This message flow represents a PDU session modification for a PDU session previously established in a certain NSI.  Other parameters in the NAS message (e.g. the DNN in certain session management messages, or QoS parameterts etc.) beyond MDD vector and the Temporary ID related ones are not discussed here in this procedure.

Figure 6.1.2.2.3.3-1: Subsequent Session Modification Request Slicing Procedures

1. The UE initiates Session Modification Request procedure after the PDU session was established in the target NSI, so it sends a Session Modification Request message including its Temporary ID, the MDD vector of the NSI and the Session ID and the related parameters (e.g QoS). The MDD vector is included since a specific NSI needs to be addressed by the NAS procedure. The UE includes at least the routing field of the Temporary ID in the RRC layer message for the RAN to route the NAS Message to the appropriate Serving CCNF.

2. The RAN routes and forwards the NAS message to the proper Serving CCNF based on the information in step 1.

3. The Serving CCNF forwards the Session Modification Request to the NSI indicated in the MDD vector

4. The PDU Session Modification REQ is processed.  The operation status is captured in the cause to be responded to the UE.

5. The Serving NSI responds to the UE by sending to the RAN a Session Modification Response NAS message with the cause.

6-7.  The CCNF and RAN forward the Session Modification Response message the CCNF has received in step 5 to the UE.

#### 6.1.2.3	Solution evaluation

Editor's note:	This clause will contain evaluation on the system impacts, e.g. UE, access network and non-access network.