SA WG2 Meeting #116bis	S2-165399

11 - 15 Jul 2016, Vienna, AT	(was S2-16xxxx)

Source:	Nokia (Email Session Convener)

Title:	Update to Interim agreements on QoS framework – QoS parameters email session

Document for:	Approval

Agenda Item:	6.10.2

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: Updates to interim agreements are proposed for Key Issue #2 (QoS framework).

# 1	Proposal

Based on the email session summary, it is proposed to update the interim agreements to include the following:

- Some User plane marking values may be a scalar value and may have standardized QoS characteristics that correspond to PER, PDB, Priority. In this case, NG2 signalling includes only an indication that such values are authorized to be used.

- Some User plane marking values may be a scalar value and may point to dynamic QoS parameters signalled over NG2. Dynamic QoS parameters signalled over NG2 may include the following:

  - Maximum Flow Bit Rate

  - Guaranteed Flow Bit Rate or Required Flow Bit Rate.

  - Priority level

  - Delivery characteristics: Packet delay budget.

  - Delivery characteristics: Packet error rate.

  - Allocation and Retention Priority or Network Behaviour (Admission, Retention and Notification).

It is proposed to agree the following modified text in TR 23.799:

####################### START TEXT FOR TR 23.799 ##########################

## 8.3	Interim Agreements on Key Issue #2 QoS Framework

Interim agreements for Key issue #2 QoS framework are as follows:

1)	Support Reflective QoS over RAN under control of the network. The network decides on the QoS to apply, reflects the DL traffic and the UE reflects the DL QoS for associated UL traffic.

Editor’s note: How reflective QoS is supported will be discussed as part of the solutions.

2.	U-plane marking for QoS is carried in encapsulation header on NG3 i.e. without any changes to the e2e packet header.

3a.	 A default QoS rule shall and pre-authorised QoS rules may be provided at PDU Session establishment to UE. using NG1 signalling.

NOTE: In some cases part of the QoS information can be provided as AS information even at PDU Session establishment.

Editor’s Note: The content of the QoS rule is FFS, including a possible change of the term to avoid confusion with PCC/QoS rules.

Editor’s Note: QoS related signalling to the UE for non-3GPP access is FFS.

3b. QoS rules can be (e.g. depending on access capabilities) provided at PDU Session establishment to the RAN using NG2 signalling.

4.	Flow-specific QoS signalling via the C-plane is needed for GBR SDF.

Editor’s Note: Definition of Flow is for FFS.

5. Some User plane marking values may be a scalar value that have standardized QoS characteristics.

6. Some User plane marking values may be a scalar that point to dynamic QoS parameters signalled over NG2.

7. Dynamic QoS parameters may include the following:

  - a. Maximum Flow Bit Rate

  - b. Guaranteed Flow Bit Rate.

  - c. Scheduling Priority level

  - d. Packet Delay Budget

  - e. Packet Error rate.

  - f. Parameter for admission control.

Editor’s note: WHETHER a certain parameter in bullet 7) applies to both bullets or only one of them (bullet 5), or bullet 6) is FFS.

####################### END TEXT FOR TR 23.799 ##########################