SA WG2 Meeting #116bis	S2-165444

29 August – 3 September 2016, China, Sanya	e-mail revision 2 of S2-165065

Source:	BlackBerry UK Ltd

Title:	Solution 2

Document for:	Approval

Agenda Item:	6.14

Work Item / Release:	FS_PS_DATA_OFF / 14

Abstract of the contribution: Use USSD as mechanism to carry 3GPP PS Data Off.

# Discussion

Any solution deployed should have minimum impact on the VPLMN and be capable of being supported across 2G, 3G and LTE networks.  USSD provides a mechanism by which specific *# codes have been allocated to always be routed to the HPLMN.  No processing is required in the VPLMN.  USSD is also a service that has been used for numerous years for service like prepaid, thus there is already working procedures how to reconcile USSD traffic between operators without impact the end user.

# Proposal

Use USSD as a mechanism to provide an indication to the HSS that 3GPP PS Data Off has been activated or deactivated, and then use standard filter criteria updating mechanisms to route traffic to an application server.

* * * Start of changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.011: "Service accessibility".

[k]	3GPP TS 23.221: "Architectural requirements".

[w]	3GPP TS 23.401: "GPRS enhancements for E-UTRAN access".

[x]	3GPP TS 23.272: "Circuit Switched Fallback in Evolved Packet System; Stage 2".

[y]	3GPP TS 24.390  "Unstructured Supplementary Service Data (USSD) using IP  Multimedia (IM) Core Network (CN) subsystem IMS; Stage 3"

[z]	3GPP TS 22.030: "Man-Machine Interface (MMI) of the User Equipment (UE)".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next change * * * *

## 6.x	Solution #X: Key issue 1 UE reports its 3GPP PS Data Off status to HSS using USSD and USSI

### 6.x.1	Description

### The solution below is based upon using USSD and shows the case when USSD over CS has been chosen.  However the functionality is equally applicable to using USSI as specified in TS 24.390 [y].

Figure 6.X.1-1: USSD Activation / Deactivation of 3GPP PS Data Off

Steps 1-6 are optional and are only performed by the UE when UICC is inserted into ME for the first time

1)	UE performs an Attach procedure as specified in 3GPP TS 23.401 [w]

2)	The UE sends a USSD message containing a well known *# sequence that deactivates 3GPP PS Data off.  The domain used for USSD will be selected according to TS 23.221 [k].clause 7.2e. The *# sequence is provisioned in the UE by the HPLMN using device management or on the USIM.

NOTE 1:	The well known 3GPP PS Data Off deactivation *# sequence is chosen from the HPLMN range that can be found in 3GPP TS 22.030 [z].

NOTE 2	The *# sequence could be specified by the GSMA or chosen by the HPLMN.

3)	The MSC analyses the *# sequence received in the USSD message from the UE and determines from the *# sequence that the USSD message is to be routed to the HPLMN.  The MSC includes the IMSI in the message.

4)	The HSS marks 3GPP PS Data Off as being deactive for the IMSI received.

5)	The HSS sends back an acknowledgement.

6)	The UE receives the USSD acknowledge.

Activation

7)	3GPP PS Data Off is activated.

8)	The UE sends a USSD message containing a well known *# sequence that activates 3GPP PS Data off.  The domain used for USSD will be selected according to TS 23.221 [k] clause 7.2e. The *# sequence is provisioned in the UE by the HPLMN using device management or on the USIM.

NOTE 3:	The well known 3GPP PS Data Off activation *# sequence is chosen from the HPLMN range that can be found in 3GPP TS 22.030 [z].

NOTE 4	The *# sequence could be specified by the GSMA or chosen by the HPLMN.

9)	The MSC analyses the *# sequence received in the USSD message from the UE and determines from the *# sequence that the USSD message is to be routed to the HPLMN.  The MSC includes the IMSI in the message.

10)	The HSS marks 3GPP PS Data Off as being active for the IMSI received.

11)	The HSS sends back an acknowledgement.

12)	The UE receives the USSD acknowledge.

Deactivation

See steps 2)-6).

### 6.x.2	Impacts on existing nodes and functionality

The HSS needs to be provisioned with which services are 3GPP PS Data Off Exempt Services and

- be able to receive a USSD message and associate that with activating or de-activing 3GPP PS Data Off Exempt Services; and

UE needs to be able to send indications if 3GPP PS Data Off service has been activated or deactivated.

### 6.x.3	Solution Evaluation

The solution requires no new functionality in any VPLMN nodes.  The solution does require UTRAN or GERAN to be available to support the USSD operation otherwise the UE needs to use USSI as specified in TS 24.390 [y]. The solution with USSI works only when USSI is an 3GPP PS Data Off Exempt Service.