SA WG2 Meeting #116	S2-163606

11 – 15 July 2016, Vienna, Austria	(was S2-163606)

Source:	Nokia , Alcatel-Shanghai Bell

Title:	Role of the common UP-GW in case of multi-homed (IPv6) PDU Session

Document for:	Approval

Agenda Item:	6.10.4

Work Item / Release:	NextGen / Rel14

Abstract of the contribution: Updates solution 6.4.4 .

# 1	Discussion

Void.

If it would not be possible to include the proposed modifications in solution 6.4.4, then this Tdoc would propose to create another solution.

Figure 6.4.4.1-2 and Figure 6.4.4.1-3 have been modified

# 2	Proposal

It is proposed to modify TR 23.799 as follows…

### 6.4.4	Solution 4.4: Session Management model for multiple parallel PDU Sessions and for multi-homed PDU Session

This solution addresses WT#1 (SM model) and WT#2 (relation between SM and MM).

#### 6.4.4.1	Architecture description

This clause describes a session management model focusing on the following two cases:

-	Multiple parallel PDU Sessions of IPv4, IPv6, IPv4/IPv6 (if this type is supported in NextGen) or non-IP type;

-	Multi-homed PDU Session using IPv6.

Figure 6.4.4.1-1: Session management model with multiple parallel PDU Sessions

The proposed solution assumes the following session management model principles:

-	A PDU Session establishment and release is performed using NG1 signalling as in solution 4.2 and 4.3.

-	A PDU Session is identified with a Data Network Name (DNN).

Editor's note: It is FFS whether the DNN is identical to the APN.

-	CP functions in the NextGen core configure the user plane path for the PDU Session.

-	The user plane path in the NextGen core consists of user plane gateways (UP-GWs). The number of UP-GWs for a PDU Session is not imposed by the specification.

-	For UE with multiple PDU sessions towards different DN there is no need for mandatory “convergence point” similar to the SGW. In other words, going out of the RAN the user plane paths of PDU Sessions belonging to the same UE may be completely disjoint. This also implies that for idle mode UEs (if NextGen_Idle state is supported) there can be a distinct buffering node per PDU Session.

-	For UE with multiple PDU sessions towards the same DN there is the need of a Core Network User plane “convergence point” to support APN AMBR and charging:

NOTE1: 	DL APN AMBR enforcement has to be run in a unique entity that handles all the DL traffic from the same DN. DL traffic Charging is to be performed  after potential packet discard by DL APN-AMBR enforcement.

-	A user plane path is materialised as a tunnel. There is one tunnel per PDU Session on each reference point. The tunnel carries all traffic of a PDU Session, regardless of the QoS requirements of individual traffic flows. The tunnel encapsulation header needs to be able to carry per-packet QoS markings and possibly other information.

-	The network may decide to reconfigure the user plane path of a PDU Session outside of any UE mobility event.

-	Multiple PDU Sessions to the same Data Network are supported as described in Solution 4.3 (clause 6.4.3) or by using a multi-homed PDU Session described below.

-	When IPv6 is used a PDU Session may be associated with one or multiple IPv6 prefixes. The latter case is referred to as multi-homed PDU Session and is described in Figure 6.4.4.1-2 and Figure 6.4.4.1-3. In this case the PDU Session provides access to the Data Network via two separate IP anchors. The two user plane paths leading to the IP anchors branch out of a “common” UP-GW. This “common” UP-GW is a Core function as it has to enforce APN AMBR and charging. It enforces split of UL traffic from the UE (forwarding the traffic towards the different IP anchors) and merge of DL traffic to the UE (merging the traffic from the different IP anchors towards the link towards the UE)

NOTE 2:	Note 1 applies also in this case.

-	In an alternative 1, the “Common UP-GW” is configured as a mobility anchor that spreads the UL traffic between the IP anchors based on the Source Prefix of the PDU (selected by the UE based on policies received from the network).  This corresponds to Scenario 1 defined in IETF RFC 7157 “IPv6 Multihoming without Network Address Translation”. This allows to make the “Common UP-GW” unaware of the routing tables in the Data Network and to keep the first hop router function in the IP anchors.

-	In an alternative 2, the “Common UP-GW” is configured as a common “default IP router” functionality as defined in IETF RFC 7157 “IPv6 Multihoming without Network Address Translation” scenario 2.

The multi-homed PDU Session may be used to support make-before-break service continuity as described in Solution 6.1 (SSC mode 3 in clause 6.6.1) or cases where UE needs to access both a local service (e.g. Mobile Edge Computing server) and the Internet. These two cases are illustrated in Figure 6.4.4.1-2 and Figure 6.4.4.1-3, respectively.

NOTE 3:	The multi-homed PDU Session applies only to PDU Sessions of IPv6 type.

Figure 6.4.4.1-2: Multi-homed PDU Session using IPv6: service continuity case

Figure 6.4.4.1-3: Multi-homed PDU Session using IPv6: access to local DN

#### 6.4.4.2	Function description

#### 6.4.4.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * * Next Change * * * *