SA WG2 Meeting #116	S2-163936

11 – 15 July 2016, Vienna, Austria	(was S2-163606 + 3315)

Source:	Nokia , Alcatel-Shanghai Bell, Ericsson?

Title:	Role of the common UP-GW in case of multi-homed (IPv6) PDU Session

Document for:	Approval

Agenda Item:	6.10.4

Work Item / Release:	NextGen / Rel14

Abstract of the contribution: Updates solution 6.4.4 .

# 1	Discussion

Void.

Figure 6.4.4.1-2 and Figure 6.4.4.1-3 have been modified. Text from initial 3606

Text based on comments while presenting 3606

# 2	Proposal

It is proposed to modify TR 23.799 as follows…

### 6.4.4	Solution 4.4: Session Management model for multiple parallel PDU Sessions and for multi-homed PDU Session

This solution addresses WT#1 (SM model) and WT#2 (relation between SM and MM).

#### 6.4.4.1	Architecture description

This clause describes a session management model focusing on the following two cases:

-	Multiple parallel PDU Sessions of IPv4, IPv6, IPv4/IPv6 (if this type is supported in NextGen) or non-IP type;

-	Multi-homed PDU Session.

Figure 6.4.4.1-1: Session management model with multiple parallel PDU Sessions

The proposed solution assumes the following session management model principles:

-	A PDU Session establishment and release is performed using NG1 signalling as in solution 4.2 and 4.3.

-	A PDU Session is identified with a Data Network Name (DNN).

Editor's note: It is FFS whether the DNN is identical to the APN.

-	CP functions in the NextGen core configure the user plane path for the PDU Session.

-	The user plane path in the NextGen core consists of user plane gateways (UP-GWs). The number of UP-GWs for a PDU Session is not imposed by the specification.

-	For UE with multiple PDU sessions towards different DN there is no need for mandatory “convergence point” similar to the SGW. In other words, going out of the RAN the user plane paths of PDU Sessions belonging to the same UE may be completely disjoint. This also implies that for idle mode UEs (if NextGen_Idle state is supported) there can be a distinct buffering node per PDU Session.

-	(When APN-AMBR enforcement is required in 5G Core) For UE with multiple PDU sessions towards the same DN there is the need of a Core Network User plane “convergence point” to support APN AMBR and charging:

NOTE1: 	DL APN AMBR enforcement has to be run in a unique entity that handles all the DL traffic from the same DN. DL traffic Charging is to be performed  after potential packet discard by DL APN-AMBR enforcement.

-	A user plane path is materialised as a tunnel. There is one tunnel per PDU Session on between two entities . The tunnel carries all traffic of a PDU Session, regardless of the QoS requirements of individual traffic flows. The tunnel encapsulation header needs to be able to carry per-packet QoS markings and possibly other information.

-	The network may decide to reconfigure the user plane path of a PDU Session outside of any UE mobility event.

-	Multiple PDU Sessions to the same Data Network are supported as described in Solution 4.3 (clause 6.4.3) or by using a multi-homed PDU Session described below.

-	a PDU Session may be associated with one or multiple IPv6 prefixes or IPV4 addresses. The latter case is referred to as multi-homed PDU Session and is described in Figure 6.4.4.1-2 and Figure 6.4.4.1-3. In this case the PDU Session provides access to the Data Network via two separate IP anchors. The two user plane paths leading to the IP anchors branch out of a “common” UP-GW referred to as “branching point”. The branching point is a logical functionality which may be co-located with other entities (e.g., a UP-GW for one of the IP anchors or the RAN). The branching point functionality ensures that uplink packets take the appropriate path based on the UE’s source address or other header fields The branching point is a Core functionality as it has to enforce APN AMBR and charging. It enforces split of UL traffic from the UE (forwarding the traffic towards the different IP anchors) and merge of DL traffic to the UE (merging the traffic from the different IP anchors towards the link towards the UE)

NOTE 2:	(When APN-AMBR enforcement is required in 5G Core) Note 1 applies also in this case.

-	The use of multiple IPv6 prefixes in a PDU session could be based on following alternatives

-	In an alternative 1, the “branching point”is configured as a mobility anchor that spreads the UL traffic between the IP anchors based on the Source Prefix of the PDU (selected by the UE based on policies received from the network).  This corresponds to Scenario 1 defined in IETF RFC 7157 “IPv6 Multihoming without Network Address Translation”. This allows to make the “Common UP-GW” unaware of the routing tables in the Data Network and to keep the first hop router function in the IP anchors.

-	In an alternative 2, the “branching point”is configured as a common “default IP router” functionality as defined in IETF RFC 7157 “IPv6 Multihoming without Network Address Translation” scenario 2.

Editor’s Note:	It is FFS which of the 2 alternatives above will be defined in 5G core specifications

Editor’s Note:	In  case of IPv4 multi-homed PDU session, it is FFS how the network can influence UE decisions on which source address to use

- The multi-homed PDU Session may be used to support make-before-break service continuity as described in Solution 6.1 (SSC mode 3 in clause 6.6.1) illustrated in Figure 6.4.4.1-2. The multi-homed PDU session may also be used to support  cases where UE needs to access both a local service (e.g. Mobile Edge Computing server) and the Internet, illustrated in Figure 6.4.4.1-3. Access to local services may also be realized using the same address/prefix as for other services, as described in Solution 5.2 in more detail. In this case, the branching point may use filtering criteria other than the source IP address for uplink traffic.

.

- A branching point for a given session may be inserted or removed by the control plane on demand. For example, when a session is first created initially with a single address/prefix, no branching point is needed. When a new address/prefix is added to the session, a branching point may be inserted. After the release of the additional addresses/prefixed, the branching point may be removed by the control plane when there is only a single address/prefix for the session.

Figure 6.4.4.1-2: Multi-homed PDU Session: service continuity case

Figure 6.4.4.1-3: Multi-homed PDU Session: access to local DN

#### 6.4.4.2	Function description

#### 6.4.4.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * * Next Change * * * *