SA WG2 Meeting #S2-117	S2-165800

17 - 21 October 2016, Kaohsiung City, Taiwan	(revision of S2-16XXXX)

Source:	LG Electronics

Title:	Interim agreement on reflective QoS

Document for:	Discussion & Approval

Agenda Item:	6.10.2

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This contribution proposes an interim agreement on key issue 2 – QoS Framework especially on reflective QoS.

Discussion

There is a following interim agreement about reflective QoS in TR 23.799.

1	Support Reflective QoS over RAN under control of the network. The network decides on the QoS to apply on the DL traffic and the UE reflects the DL QoS to the associated UL traffic. When the UE receives a DL packet for which reflective QoS should be applied, the UE creates a new implicit QoS rule. The packet filter in the implicit QoS rule is derived from the header of the DL packet.

Editor's note:	It is FFS whether Reflective QoS indication is signalled via C-plane, or inband, or not signalled at all.

This contribution tries to solve above Editor's note.

There are three different signalling methods.

- If control signalling is used, reflective QoS may increases signalling which is not aligned with principle that signalling should be minimized.

- The inband signalling can be a good solution for reflective QoS because it does not introduces new signalling. However, inband signalling cannot be used in non-3GPP access.

- For the last option, i.e., signalling is not used, we need to agree that all uplink flows use reflective QoS if there are corresponding downlink flows. In this option, uplink QoS and downlink QoS is always same if explicit QoS rule is not provided.

It is proposed to use both inband signalling and no signalling to indicate reflective QoS. How to indicate reflective QoS to the UE is decided by the network during the PDU session establishment procedure.

For example, when the UE is attached through 3GPP access the network may use inband signalling for reflective QoS. The UE applies reflective QoS when inband signalling is received. When the UE is attached through non-3GPP access, the network may use no signalling for reflective QoS. In this case, the UE always applies reflective QoS for uplink traffic if there is corresponding downlink flow.

Proposal

It is proposed to add the following solution to the TR 23.799 for Study on Architecture for Next Generation System.

* * * * Start of 1st Change * * * *

8.2	Interim Agreements on Key Issue #2: QoS framework

Interim agreements for Key issue #2 QoS framework are as follows:

1	Support Reflective QoS over RAN under control of the network. The network decides on the QoS to apply on the DL traffic and the UE reflects the DL QoS to the associated UL traffic. When the UE receives a DL packet for which reflective QoS should be applied, the UE creates a new implicit QoS rule. The packet filter in the implicit QoS rule is derived from the header of the DL packet.

1a. Reflective QoS indication is signalled via inband or not signalled at all by the network. How to indicate reflective QoS to the UE is decided by the network during the PDU session establishment procedure. When reflective QoS is not signalled, all uplink flows use reflective QoS if there are corresponding downlink flows.

Editor's note:	It is FFS whether implicit rules (derived via Reflective QoS) have higher or lower precedence order compared to explicitly signalled QoS rules.

2.	U-plane marking for QoS is carried in encapsulation header on NG3 i.e. without any changes to the e2e packet header.

3a.	A default QoS rule shall and pre-authorised QoS rules may be provided at PDU Session establishment to UE.

Editor's note:	The content of the QoS rule is FFS, including a possible change of the term to avoid confusion with PCC/QoS rules. It is FFS whether the QoS rule signalling to UE involves NAS or AS-level signalling.

Editor's note:	QoS related signalling to the UE for non-3GPP access is FFS.

3b. QoS rules can be (e.g. depending on access capabilities) provided at PDU Session establishment to the RAN using NG2 signalling.

4.	QoS Flow-specific QoS signalling via the C-plane is needed for GBR SDF.

5.	NG2 signalling related to QoS, outside of PDU Session establishment, corresponding to a pre-authorised QoS rule should be minimised for initiation, modification or termination of SDFs with no GBR requirements.

Editor's note:	This is target for SA2, but the feasibility needs to be confirmed by RAN.

Editor's note:	NG2 QoS related signalling for non-3GPP access is FFS.

6.	NG1 signalling related to QoS, outside of PDU Session establishment, corresponding to a pre-authorised QoS rule should be minimised for initiation, modification or termination of SDFs with no GBR requirements.

Editor's note:	NG1 QoS related signalling for non-3GPP access is FFS.

7.	For the purpose of subscription and service differentiation, enforcement of UL rate limit per Service Data Flow and per PDU Session shall be done in a CN_UP, being a trusted point of enforcement in the network, handling all traffic of the PDU session.

Editor's note:	It is FFS which type of flows the CN_UP applies “per SDF”, “per PDU session” rate limitation on.

8.  The AN shall enforce a rate limit in UL per UE.

Editor's note:	It is FFS which type of flows the AN applies rate limitation on.

Editor's note:	How to handle UL rate limitation per UE when the UE has access over non-3GPP AN and when the UE has access over multiple ANs including 3GPP and non-3GPP ANs is FFS

Editor's note:	UL Rate limitation requirements for the UE is FFS.

9.	QoS Flow is the finest granularity for QoS treatment in the NG System.

10.1. In the downlink the (R)AN binds QoS Flows onto access-specific  resources based on the NG3 marking and the corresponding QoS characteristics provided via NG2 signalling. Packet filters are not used for binding of QoS Flows onto access-specific  resources in (R)AN.

10.2. UE binds uplink packets onto access-specific  resources based on information for binding uplink packets onto access-specific  resources provided explicitly by the access network and/or based on QoS rules (explicitly signaled or implicitly derived via reflective QoS).

Editor's note:	It is FFS whether UE is aware of the QoS level / QoS profile associated with a QoS flow.

11. Some User plane markings are scalar values that have standardized QoS characteristics.

12. Some User plane markings are scalar values that point to dynamic QoS parameters signalled over NG2.

13. Dynamic QoS parameters may include the following:

a. Maximum Flow Bit Rate

b. Guaranteed Flow Bit Rate.

c. Priority level

d. Packet Delay Budget

e. Packet Error rate.

f. Admission control.

Editor's note:	Whether a certain parameter in bullet 13) applies to both bullets (#11 and #12) or only one of them (either bullet #11 or bullet #12) is FFS.

Editor's note:	Whether Priority Level is used for more than scheduling purpose is FFS.

* * * * End of Changes * * * *