SA WG2 Meeting #117	S2-165810

17-21 October 2016, Kaohsiung city, Taiwan(R.O.C)	(was S2-16)

Source:	ZTE Corporation

Title:	Clarification on the direct interaction

Document for:	Approval

Agenda Item:	6.10.7

Work Item / Release:	FS_NextGen / Rel14

Abstract of the contribution: It is clarified that the source NF may use name or IP address or logic identity to identify the destination NF for direct interaction

1	Proposal

It is proposed to modify TR 23.799 as follows…

* * * * Start of Change * * * *

8.7	Interim Agreements on Key Issue #7: Network function granularity and interactions between them

Interim agreements for Key issue #7 “Function Granularity and Interconnection of them” are as follows:

1.	Any two NFs interacts with each other directly while avoiding the functional and signalling impact on unrelated NF.

NOTE: 	This does not preclude to pass information via a third NF if two NFs do not interact directly, e.g. if MM received subscription information from SDB then it can pass it to SM if there is an interaction between MM and SM (e.g. during PDU connection establishment procedure).

NOTE:	The source NF may use name or IP address or logic identity to identify the destination NF for direct interaction.

2.	Procedures (i.e. set of interactions between two NFs) are defined as a service, wherever applicable, (e.g. by following the guidelines defined in Annex E) so that its re-use is possible. This will be evaluated on a case by case basis when specifying procedure.

3.	The NF selection and discovery shall be supported to enable NF selection and discovery;

NOTE:	 whether it utilizes the NF Repository function or an enhancement of the DNS server to reach this functionality is left for CT WG to determine.

Editor's note:	It is FFS whether the NF Repository Function returns the logic identities or names/addresses of the Destination NFs.

* * * * End of Change * * * *