SA WG2 Meeting #117	S2-165997

17-21 October 2016, Kaohsiung city, Taiwan	(was S2-165764)

Source:	Qualcomm Inc.

Title:	Additional interim agreements for QoS establishment

Document for:	Approval

Agenda Item:	6.10.2

Work Item / Release:	NextGen / Rel14

Abstract of the contribution: the paper proposes a way forward and interim agreements on QoS establishment mechanisms.

# 1	Discussion

Interim agreements have been achieved regarding the use of Pre-Authorized QoS rules defined by the NextGen CN upon PDU session establishment and on the provisioning by the CN control plane of Preauthorized QoS rules to the UE, the AN and the User Plane Functions in the CN.

However, the interim agreements do not have conclusions on the mechanisms required for the actual establishment of QoS, included dedicated resources in the AN when necessary depending on the type of AN.

# 2	Proposal

The proposal is to accept the following changes.

START OF CHANGES

## 8.2	Interim Agreements on Key Issue #2: QoS framework

Interim agreements for Key issue #2 QoS framework are as follows:

1	Support Reflective QoS over RAN under control of the network. The network decides on the QoS to apply on the DL traffic and the UE reflects the DL QoS to the associated UL traffic. When the UE receives a DL packet for which reflective QoS should be applied, the UE creates a new implicit QoS rule. The packet filter in the implicit QoS rule is derived from the header of the DL packet.

Editor's note:	It is FFS whether Reflective QoS indication is signalled via C-plane, or inband, or not signalled at all.

Editor's note:	It is FFS whether implicit rules (derived via Reflective QoS) have higher or lower precedence order compared to explicitly signalled QoS rules.

2.	U-plane marking for QoS is carried in encapsulation header on NG3 i.e. without any changes to the e2e packet header.

3a.	A default QoS rule shall and pre-authorised QoS rules may be provided at PDU Session establishment to UE.

Editor's note:	The content of the QoS rule is FFS, including a possible change of the term to avoid confusion with PCC/QoS rules. It is FFS whether the QoS rule signalling to UE involves NAS or AS-level signalling.

Editor's note:	QoS related signalling to the UE for non-3GPP access is FFS.

3b. QoS rules can be (e.g. depending on access capabilities) provided at PDU Session establishment to the RAN using NG2 signalling.

4.	QoS Flow-specific QoS signalling via the C-plane is needed for GBR SDF.

5.	NG2 signalling related to QoS, outside of PDU Session establishment, corresponding to a pre-authorised QoS rule should be minimised for initiation, modification or termination of SDFs with no GBR requirements.

Editor's note:	This is target for SA2, but the feasibility needs to be confirmed by RAN.

Editor's note:	NG2 QoS related signalling for non-3GPP access is FFS.

6.	NG1 signalling related to QoS, outside of PDU Session establishment, corresponding to a pre-authorised QoS rule should be minimised for initiation, modification or termination of SDFs with no GBR requirements.

Editor's note:	NG1 QoS related signalling for non-3GPP access is FFS.

7.	For the purpose of subscription and service differentiation, enforcement of UL rate limit per Service Data Flow and per PDU Session shall be done in a CN_UP, being a trusted point of enforcement in the network, handling all traffic of the PDU session.

Editor's note:	It is FFS which type of flows the CN_UP applies “per SDF”, “per PDU session” rate limitation on.

8.  The AN shall enforce a rate limit in UL per UE.

Editor's note:	It is FFS which type of flows the AN applies rate limitation on.

Editor's note:	How to handle UL rate limitation per UE when the UE has access over non-3GPP AN and when the UE has access over multiple ANs including 3GPP and non-3GPP ANs is FFS

Editor's note:	UL Rate limitation requirements for the UE is FFS.

9.	QoS Flow is the finest granularity for QoS treatment in the NG System.

10.1. In the downlink the (R)AN binds QoS Flows onto access-specific  resources based on the NG3 marking and the corresponding QoS characteristics provided via NG2 signalling. Packet filters are not used for binding of QoS Flows onto access-specific  resources in (R)AN.

10.2. UE binds uplink packets onto access-specific  resources based on information for binding uplink packets onto access-specific  resources provided explicitly by the access network and/or based on QoS rules (explicitly signaled or implicitly derived via reflective QoS).

Editor's note:	It is FFS whether UE is aware of the QoS level / QoS profile associated with a QoS flow.

11. Some User plane markings are scalar values that have standardized QoS characteristics.

12. Some User plane markings are scalar values that point to dynamic QoS parameters signalled over NG2.

13. Dynamic QoS parameters may include the following:

a. Maximum Flow Bit Rate

b. Guaranteed Flow Bit Rate.

c. Priority level

d. Packet Delay Budget

e. Packet Error rate.

f. Admission control.

Editor's note:	Whether a certain parameter in bullet 13) applies to both bullets (#11 and #12) or only one of them (either bullet #11 or bullet #12) is FFS.

Editor's note:	Whether Priority Level is used for more than scheduling purpose is FFS.

14.	UE triggered QoS establishment for non-guaranteed bit rate QoS flows corresponding to pre-authorized QoS rules can be based on

explicit user plane marking determined based on the pre-authorized QoS rules.

15.	UE triggered QoS establishment for guaranteed bit rate QoS flows is based on explicit UE-requested QoS over NG1.

END OF CHANGES