SA WG2 Meeting #117	S2-166245

17-21 October 2016, Kaohsiung city, Taiwan	(was S2-165766)

Source:	Qualcomm Inc.

Title:	Way forward on Key Issue 4 on Session Management for WT1 and WT2

Document for:	Approval

Agenda Item:	6.10.4

Work Item / Release:	NextGen / Rel14

Abstract of the contribution: the paper addresses Key Issue #4, and specifically work task 1 on “Baseline session set up / baseline model” and work task 2 on “MM/SM).” The contribution proposes a way forward.

# 1	Introduction

The paper addresses Key Issue #4, and specifically work task 1 on “Baseline session set up / baseline model” and work task 2 on “Relation between MM and SM.”

The contribution proposes a way forward by:

-	updating solution 6.4.3

-	endorsing concepts in solutions 6.4.1, 6.4.2, and the updated solution 6.4.3

-	proposing a set of interim agreements

# 2	Proposal

START OF CHANGES

## 8.4	Interim Agreements on Key Issue #4: Session management

Interim agreements for Key issue #4 Session Management are as follows:

1.	The NextGen system should support multiple PDU sessions via multiple accesses to the same data network or different data networks in the following case

-	One access network is NG RAN and another access network is non-3GPP access

2.	The NextGen system should support PDU sessions whose traffic is simultaneously carried over multiple access where one access is a 3GPP access and the other is a non-3GPP access

NOTE: The bullet 2 will be handled in Phase 2.

3.	The NextGen system should support the ability to have multiple PDU sessions to the same Data Network and served by different UP functions terminating NG6.

4.	The User Plane format in NextGen on NG3 shall at least support per PDU Session tunnelling, as described in clause 6.4.10.

Editor's note:	User Plane format within the CN is FFS.

Editor's note:	The granularity of the tunnelling for non-3GPP accesses is FFS.

Editor's note:	Whether an additional tunnelling granularity variant will be supported for stationary UEs is FFS.

5.	For home routed traffic, a UPF in the VPLMN is allocated to support the PDU session. As an example, this is to enable routing of the traffic of a PDU session between the HPLMN and the VPLMN,  to minimize the impact on the HPLMN of the UE mobility within the VPLMN (for scenarios where SSC1 is applied), and to avoid requiring for idle mode UEs (if NextGen IDLE state is supported) that the UPF in the HPLMN acts as buffering node for the PDU Session.

6.	The establishment of a PDU Session may be authorized/authenticated by an external DN via the SMF..

Editor’s Note: the extent of specification work in 3GPP to enable such authorization/authentication is FFS and depends on SA3 work.

Editor’s Note: The interaction between the NGC and external Data Networks needs to be specified by 3GPP to provide transport of signalling for PDU session authorization/authentication by the external Data Network.

Interim agreements for MM and SM interaction are as follows:

1.	A single NG1 NAS connection is used for both MM and SM-related messages and procedures for a UE. The single NG1 termination point is located in MM.

Editor notes: This is applied for UE only registered via 3GPP access. The case of UE registered via non-3GPP is FFS.

2.	The MM selects the SM functions for the PDU sessions. MM may select different SM functions for different PDU sessions.

3.	MM forwards SM related NAS information to the SM function.

4.	MM stores the identification of serving SM function(s) of UE and SM stores the identification of serving MM function of UE.

END OF CHANGES