SA WG2 Meeting #118Bis	S2-170154

16- 20 January 2017, Spokane, US	(revision of S2-170154)

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	User plane protocol stack between the UE and the DN

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_Ph1  / Rel-15

Abstract of the contribution: This contribution proposes the protocol stack between the UE and the DN.

# Introduction

# Proposal

It is proposed to add the following descriptions into TS 23.501 “System Architecture for 5G System”.

* * * Start of changes* * * * (all text is new)

## 8.3	User Plane Protocol stacks

.

#### 8.3.1	User Plane Protocol stack for a PDU session

This clause illustrates the protocol stack for the User plane transport related with a PDU session.

5GC      Encapsu-lation

UPF

(PDU session anchor)

L1

PDU layer

Application

N6

5G AN

UDP/IP

L2

Relay

L1

UE

N3

AN protocol stack

AN protocol stack

UDP/IP

L2

L1

PDU layer

5GC Encapsu-lation

UDP/IP

L2

L1

5GC Encapsu-lation

UDP/IP

L2

L1

5GC Encapsu-lation

UPF

(Non PDU session anchor)

Legend:

-	PDU layer: This layer corresponds to the PDU carried between the UE and the DN over the PDU session. When the PDU session Type is IPV6, it corresponds to IPv6 packets ; When the PDU session Type is Ethernet, it corresponds to Ethernet frames ; etc….

-	5GC Encapsulation: This layer supports multiplexing traffic of different PDU sessions (possibly corresponding to different PDU session Types) on the same N9 interface between 2 User plane entities of the 5G Core or on N3. This multiplexing is done on a per PDU session level. This layer carries also the marking associated with a QoS Flow defined in clause 5.8.1

Editor’s notes: The details of encapsulation header should be updated according to the protocol chose/defined in stage 3.

-	AN protocol stack: This set of protocols/layers depends on the AN

-	When the AN is a 3GPP RAN, it is defined by the 3GPP RAN

Editor’s notes: The details of RAN protocol stacks depend on the decision of RAN WG and will be updated later.

- When the AN is an Untrusted non 3GPP access to 5GC the 5G AN interfaces with the 5GC at a N3IWF defined in clause 4.3.2 and the AN protocol stack is defined in clause 8.3.3

-	UDP/IP: These are the backbone network protocols used for routing user data and control signalling.

Figure 8.3.1-1: User Plane

NOTE1:  How to support the scenarios with an UPF supporting an ULCL or a branch point functionality (as defined in clause 5.3)  is described in clause 8.3.2.

NOTE2:  The number of UPF in the data path is not constrained by 3GPP specifications: there may be in the data path 0, 1 or multiple UPF that do not support a PDU session anchor functionality. In case of Home Routed deployment there is in the data path at last 1 UPF in the VPLMN that does not support a PDU session anchor functionality. For the IP type PDU session, the UPF acting as PDU session anchor is the IP anchor point of the IP address/prefix allocated to the UE.

NOTE3:  The N9 interface may be intra-PLMN or inter PLMN (in case of Home Routed deployment).

#### 8.3.2	Protocol stack for a PDU session in case of UL CL or IPv6 Multi-homing

Editor’s notes: FFS.

#### 8.3.3	AN Protocol stack for a PDU session in case of Untrusted access to 5GC

Editor’s notes: FFS.

* * * End of Changes * * *