SA WG2 Meeting #118-BIS	S2-170244

16 - 20 Jan 2017 Spokane, WA, USA	(revision of S2-17xxxx)

Source:	LG Electronics, Nokia, Alcatel-Lucent Shanghai Bell

Title:	23.502: Deregistration procedure for non-3GPP access

Document for:	Approval

Agenda Item:	6.5.10 Non-3GPP access specific functionality and flows

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution:This contribution proposes deregistration procedure for Non-3GPP access.

Discussion

Proposal

Add the following to the TS 23.502.

* * * * Start of 1st Change * * * *

4.11	Procedures for non-3GPP access

Editor’s Note:	Including interworking procedures with untrusted non-3GPP access, i.e. the differences south of N2 compared to what is described in other procedures e.g. Registration procedures.

4.11.X	Deregistration procedure

Figure 4.11.X-1: Deregistration procedure for untrusted non-3gpp access

1.	The Deregistration procedure is triggered by one of the events described in sub-clause 4.1.1.2

Editor’s Note: It is FFS whether the N3IWF can initiate the deregistration procedure by sending UE Release (UE identity) message to the AMF via N2 signalling when the IKEv2 tunnel is released.

2.	As in step X of Figures 4.1.1.2-X.

3.	As in step X of Figures 4.1.1.2-X.

4a.	As in step X of Figures 4.1.1.2-X.

4b.	As in step X of Figures 4.1.1.2-X.

5.	The AMF sends a CN2 UE release message to the N3IWF.

6.	Non-3GPP access specific resources are released including the IKEv2 tunnel (and the associated IPSec resources) and the local UE contexts in N3IWK (N3 tunnel Id).

7.	The N3IWF acknowledges the N2 UE release

* * * * End of Changes * * * *