SA WG2 Meeting #118bis	S2-170298

Jan 16 – 20, 2017, Spokane, Washington, USA	(revision of S2-17xxxx)

Source:	ITRI

Title:	Identities used for Network Slicing in Section 5.7 Identities

Document for:	Approval

Agenda Item:	6.5.1 General aspects, concept and reference models

Work Item / Release:	NexGen/Rel-15

Abstract of the contribution:

This contribution proposed the content to TS 23.501.

- Discussion

Figure 1: an example of Network Slicing of Group B Architecture

In Sec. 8.1 Agreement on Key Issue #1: network slicing of NextGen SID TR, the Agreement 6: “with reference to Annex D: move forward with Group B type of solution in rel-15 (Group C is subsumed under Group B). Group A is not pursued in R15.” Therefore, in the following discussion, it will focus on Group B architecture (shown in Figure 1).

As shown in Figure 1, it can be seen that a CCNF is shared among multiple CNIs which have same slice/service type and each CNI is owned by a tenant according to Agreement 1 (c): “Network may deploy multiple Network Slice Instances delivering exactly the same optimizations and features as per but dedicated to different groups of UEs, e.g., as they deliver a different committed service and/or because they may be dedicated to a customer.”. Furthermore, a CCNF-ID is used and allocated by a network operator. In Agreement 2 (a): “The NSSAI can have standard values or PLMN specific values. The NSSAI (which is used to select the CCNF) is a collection of SM-NSSAIs, each allowing the network to select a particular slice”, it implies that a CCNF-ID could be standard value or PLMN specific value. In order to distinguish different CCNFs and CNIs, the following identities are required and the definition for each ID is described as

- CCNF-ID: to identify a CCNF in a PLMN.

- Tenant-ID: to identify a CNI provided by a 3rd party.

- Slice/Service type (SST) ID: to identify a specific type of a CNI.

CCNF-ID and Tenant-ID must be unique in a PLMN and be defined by a network operator who orchestrates the resource and be able to recognize each CNI in the network. For the Slice/Service type ID can be defined by a third party and informs to the network operator.

2. Proposal

It is proposed to add the following content to the TS 23.501.

* * * Start of Changes * * *

5.7	Identities

5.7.1 Identities required for Network Slice Instance (NSI) in Core Network

Below identities related to Network Slice Instance are required

- CCNF-ID: to identify a CCNF in a PLMN.

- Tenant-ID: to identify a CNI provided by a 3rd party.

- Slice/Service type (SST) ID: to identify a specific type of a CNI.

Note: CCNF-ID and Tenant-ID must be unique in a PLMN and be defined by a network operator who orchestrates the resource and be able to recognize each CNI in the network. For the Slice/Service type ID can be defined by a third party and informs to the network operator.

* * * End of Changes * * *