SA WG2 Meeting #117	S2-166637

Oct 19 – 23, 2016, Kaohsiung City, Taiwan	(revision of S2-16xxxx)

Source:	China Mobile

Title:	Clarification on the UL UP function for traffic branching

Document for:	Approval

Agenda Item:	6.10.6

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: this proposal is to clarify how the UL UP function is used for traffic branching while avoid always adopt two hops in the traffic path to achieve efficient traffic forwarding

1	Introduction

For the existing solutions in TR 23.799, the solution 6.4.3,6.5.3, 6.5.4 are about one UP function(IP anchor) in the user plane path.The solution 6.5.2,6.5.5 have the description about  two UP functions in the user plane path, and the traffic to the UP function(IP anchor) is processed by two UP functions(UL-CL UP and IP anchor UP).

These two types of user plane architecture are not conflict and they can coexist in the NextGen System which makes the network architecture more efficient.

2	Proposal

The following modification could be added in the TR 23.799.

.

* * * First changes * * * *

### 6.5.2	Solution 5.2: Solution to support Session continuity to enable (re)selection of efficient user plane paths

The Solution applies to Key issue 5: Enabling (re)selection of efficient user plane paths, especially the following issues:

-	Reselection of user-plane path between a UE attached to the mobile network and communication peers outside of the mobile network (e.g. Internet hosts) when the previous path becomes inefficient.

-	Minimising impact to the user experience (e.g. minimisation of interruption time and loss of packets) when changing the anchoring point for some or all packet data connections of a UE.

The Solution applies to Key issue 6: Support for session and service continuity

-	The types of sessions to be considered in the context of session continuity;

-	The level of session continuity to be supported depending on e.g., the type of service such as broadband, group communications, mission critical communications, etc.;

Editor's note:	This solution assumes support for IP services. Extending this for supporting non-IP (e.g. Ethernet) services is FFS.

#### 6.5.2.1	Architecture description

This solution assumes that the Next Generation network user plane will include two functions.

-	One UP function providing the following functionality: uplink classifier functionality (UL CL) that allows steering local traffic to local services and/or central UP function towards central services, i.e. the ability to apply filtering rules (e.g. to examine the destination IP address of IP packets sent by the UE) and determine how the packet should be routed. This is combined with connectivity to a local network packet routing, packet forwarding, traffic offloading to the local network as well as charging, LI and bitrate enforcement for locally offloaded traffic.

-	Another UP function providing IP anchoring functionality.

Figure 6.5.2.1-1 User plane Architecture for the solution

The operator uses configuration or policies to determine which flows are to be routed to/from the local network. Such configuration or policies are applied in the "UP function UL-CL" using uplink classification based on IP-5-tuple filtering rules. For downlink, the operator configures the network routing such that only legitimate traffic from the local server may pass via the UP function local IP point of presence.

Editor's note:	Whether additional filtering rule types need to be supported is FFS.

The operator may use a dedicated IP address range for the set of UEs that need to be reachable locally. The local service network is configured such that this address range can be used for local downlink traffic.

In the case of mobility, session continuity is provided using some form of tunnelling, both for the central IP anchor point, and for the local IP point of presence.

There are two types of user plane architecture, i.e., architecture described in this Section (Figure 6.5.2.1-1) and the architecture with only one UP function for IP anchor.  The solution 6.5 does not imply that there always two UP functions in the UE data path. When there is no local network, the data packet is only processed by UP function (IP anchor), which is more efficient. The UE’s PDU session shall be able to update between these two type of architecture based on e.g., the UE’s mobility or the network identified a more efficient data path should be setup.

A subscription record "Local Network Allowed", applicable in both non-roaming and roaming cases, is used to allow direct access to the local service network. In case of roaming, the visited operator is only allowed to use direct access to a local service network in case the home operator has explicitly allowed it.

Editor's note:	Further detailing of the subscription data is FFS, e.g. whether more information is needed than a binary indication of "Local Network Allowed".

#### 6.5.2.2	Function description

##### 6.5.2.2.1	General

UE has access to centralized services (access to internet) via the IP anchoring UP function, distributed services (e.g. cached content of a CDN, content distribution network) via the UL-CL UP function that provides connectivity to the local network. The IP anchoring UP function providing access to the centralized services is assumed to be located in a more centralized location than the UL-CL UP function providing access to the distributed services. Note, that in this scenario only one host IP address that is centrally anchored has been assigned to the UE.

So, the UL-CL UP function with the local IP point of presence examines the IP packets (i.e. destination IP address) sent by the UE and decides whether to transmit the PDU within a tunnel to the central IP/PDU session anchor or it should be transmitted to the local application server (e.g. CDN) (e.g. within a tunnel).

NOTE 1:	Details of the mechanism for transmitting PDUs from the UL-CL UP function to/from the local data network/local application server are beyond the scope of 3GPP.

Another option is that multiple IP addresses are assigned to the UE, one IP address for the local services and one IP address for the more central services.

In this case, i.e. when different IP addresses are assigned to the UE for local and central services rules configured into the UE determine whether the traffic is intended for the central IP anchor or the local IP anchor by choosing the appropriate source IP address.

For IPv6, RFC 4191 is used to configure rules into the UE to influence the selection of the source address. For IPv4 the same is achieved by means of RFC 3442. As an alternative for IPv4 (e.g. if DHCP is not supported), routing information is passed as a control-plane message to the UE.

Editor's note:	The DHCPv4 option does not allow for subsequent updates of the routing information. Whether this limitation is an issue is FFS.

NOTE:	Multiple IP addresses (e.g. one for the local anchor and one for the central anchor) could be assigned within one PDU session (a multi-homed PDU session) or via different PDU sessions. Whether only one or both approaches are supported will be concluded as part of key issue 4.

Following are the main characteristics of the single IP address variant of the solution:

-	The possibility of having both a local IP point of presence (via the UL-CL UP function) and a central IP point of presence for a given session using the IP address allocated for by the central IP anchoring UP function. The local IP point of presence is applicable only for a well-defined set of traffic flows under operator control, while the central IP point of presence is used by default. The operator configures the local networks such that only the well-defined set of traffic flows can access the local IP point of presence.

-	An operator controlled uplink classification functionality within the UL-CL UP function determines which uplink flows to direct towards the local IP point of presence when local IP point of presence is applied by the operator.

-	The network decides how the traffic is routed i.e. whether it is routed towards the local network or towards the more centrally located UP function and Data Network. Thus, the UE is not aware of the traffic redirection in this scenario.

-	The core network control plane is responsible for the setup, configuration and release of the user plane functions mentioned above, based on operator policies, UE preferences and subscription information, on a per session basis.

-	The CP verifies UE's permission for local network traffic offloading by obtaining "Local Network Allowed" data from user subscription repository. If there is only one UP function(IP anchor) existing in the data traffic path,then, the SM allocates UL-CL UP function and updates the UP path between AN and UP Function (IP anchor) by sets up the user plane tunnel between AN and UL-CL UP,and between UL-CL UP function and UP function(IP anchor). Also, the SM installs the filtering rules in the UL-CL UP function.

Editor's note:	The main characteristics of the multiple IP address variant of the solution need to be added.