SA WG2 Meeting #S2-118	S2-166722

14 - 18 November 2016, Reno, Nevada, USA 	(revision of S2-16xxxx)

Source:	Cisco Systems, Inc., NTT DoCoMo, Ericsson, ZTE

Title:	Agreements for KI 7

Document for:	Approval

Agenda Item:	6.10.7

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: Proposes Agreements for key issue 7.

# Proposal

In line with the analysis presented in S2-16xxxx it is proposed to agree to the following changes to TR 23.799.

* * * 1st Change * * * *

## 8.8	Interim Agreements on Key Issue #7

Interim agreements for Key issue #7 "Function Granularity and Interconnection of them" are as follows:

1.	Any two NFs interacts with each other directly while avoiding the functional and signalling impact on unrelated NF.

NOTE 1:	This does not preclude to pass information via a third NF if two NFs do not interact directly, e.g. if MM received subscription information from SDB then it can pass it to SM if there is an interaction between MM and SM (e.g. during PDU connection establishment procedure).

2.	In order to facilitate utilization of the capability (s) of one NF the capability (s) of NFs are exposed as a service to other NF, wherever applicable, (e.g. by following the guidelines defined in Annex E). As such the NF provides a service based interface to other NFs.

NOTE 2:	It is expected that SA2 will specify the services and functionalities that one NF supports, and CT WGs define the data model of service interface, i.e. information elements included in service interface.

NOTE 3:	To support different variants of a service and to enable the invoking NF to discover the expected service, the service need be uniquely identified.

3.	The feasibility to expose NF capabilities as service will be evaluated on a case by case basis when specifying each procedure. The service based interfaces should be considered for the interconnection between CN CP NFs. The NG1, NG2, NG4 interface are not considered to support the service based interface.

4.	The NF selection and discovery shall be supported to enable NF selection and discovery, including:

-	The NF selection and discovery function maintains the function profile of the deployed NF instances, e.g. the type of the NF.

-	When deploying/removing one NF instance, the information of the NF instance is updated.

-	One NF shall be able to utilize NF type and other service parameters to discover the expected NF instance (s), and the NF selection and discovery function provides the IP address or the FQDN of NF instance(s) to the NF.

NOTE 4:	whether it utilizes the NF Repository function or an enhancement of the DNS server to reach this functionality is left for CT WG to determine.

X.	3GPP should recommend or specify optional interfaces from network functions to storage services that network functions may use to store selected state as opaque data.

NOTE Y: 	Recommendation may for instance be by reference to an existing industry standard from an SDO other than 3GPP.  The details of this, including whether a single storage service (interface) is sufficient or if multiple storage service types need to be supported is up to Stage 3.

* * * End of changes * * * *