SA WG2 Meeting #119	S2-170850

13 - 17 February 2017, Dubrovnik, Croatia

Source:	LG Electronics Inc.

Title:	TS 23.501: Update to Selective activation of UP connection of existing PDU session

Document for:	Discussion / Approval

Agenda Item:	6.5.7

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to use SM PDU session activation procedure for selective PDU session activation when UE in connected mode.

1.	Discussion

In SA2#118bis meeting, basic functional description for selective PDU session activation and service request procedure with this concept are agreed for TS 23.501 and TS 23.501 accordingly. But some issues still remain with the editor’s note. One of the remaining issue is what procedure should be used for selective PDU session activation when the UE is in CM-CONNECTED state.

In this contribution, it is proposed to use “PDU session activation procedure”, a new SM procedure for this purpose, instead of using service request procedure. Detailed analysis and discussion are described in S2-170849, and the actual procedure is proposed in S2-170851.

Also considering the discussion in the study phase, selective PDU session deactivation seems not needed for the current 5GS normative work. Remaining editor’s note on “Whether CN-initiated PDU session deactivation is necessary or not is FFS.” seems to assume the per PDU session deactivation, since N2 release procedure is already including the CN-initiated release (PDU session deactivation). So we would like to remove the remaining editor’s note in clause 5.6.8.

2.	Proposal

According to the discussion above, we would like to propose to add the following texts in TS 23.501

* * * * Start of 1st Change * * * *

5.6.8	Selective activation of UP connection of existing PDU session

This clause applies to the case when a UE has multiple established PDU sessions. The activation of UP connection of existing PDU session causes UE-CN User Plane connection (i.e. data radio bearer and N3 tunnel) for the PDU session to be activated. The UP connection of different PDU sessions can be activated independently.

For the UE in the CM-IDLE state, either UE or network-triggered Service Request procedure may support independent activation of UP connection of existing PDU session.

For the UE in the CM-CONNECTED state, 5GS supports independent activation of UP connection of existing PDU session. In this case, the UE performs PDU Session Activation procedure as described in clause 4.2.3.4 of TS 23.502 [3].

* * * * End of Changes * * * *