# SA WG2 Meeting #S2-119	S2-170733

# 13 - 17 February 2017, Dubrovnik, Croatia

# 3GPP TSG CT WG 1 Meeting 101bis	TDoc C1-170533

# Spokane (WS), USA, 16-20 January 2017

Title:	LS on questions on congestion control for transport of user data via the control plane

Response to:	-

Release:	Rel-14

Work Item:	CIoT-ext

Source:	3GPP CT WG1

To:	3GPP SA WG2

Cc:	-

Contact person:	Christian Herrero-Veron

Christian dot Herrero at Huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 have discussed contributions related to congestion control for transport of user data via the control plane with a corresponding back-off timer due to new requirements added to TS 23.401.

As a result of the discussions, CT1 would like to provide the following information:

- The requirements added in stage 2 result in breaking the principle of the NAS layered architecture. Note that the control plane (CP) backoff timer is actually signalled to the UE in EMM messages when the messages, which are subject to be backed off, belong to ESM. Also, the reception in the UE of an ESM DATA TRANSPORT message (ESM message) results in stopping the CP backoff timer;

- There is no network enforcement on the network side for UEs which have received the CP backoff timer value, i.e., the network won’t reject requests from UEs which are actually backed off. Note that this is different from the congestion control mechanism introduced in Rel-10 under MTC work item where enforcement does exist; and

- The CP backoff timer value is provided to the UE as result of an initial NAS message which procedure is completed successfully. Later on the CP backoff timer cannot be provided while the UE is in CONNECTED mode due to lack of NAS message to signalled it to the UE. This means that once the UE have entered CONNECTED mode, there is no means for the network to apply CP congestion control by providing a CP backoff timer. This could be an issue overall for UEs that are kept in CONNECTED mode (e.g., due to light connection). Note also that the UE has means to override the backoff by, e.g, sending a CONTROL PLANE SERVICE REQUEST message which includes NAS message container (SMS), sending a CONTROL PLANE SERVICE REQUEST message without any message container, performing tracking area updating procedure and moving to CONNECTED mode.

CT1 would like to ask the following questions:

- Can SA2 consider defining signalling the CP backoff timer in ESM? CT1 would like to note that recently for rate control SA2 initially modelled it in EMM but this was finally decided to be done in ESM which does not break the NAS layered architecture.

- Have SA2 considered the case of UEs being in CONNECTED mode (e.g., due to light connection) when the network needs to restrict transport of user data via the control plane? CT1 believe that SA2 should investigate means to backoff UEs in CONNECTED mode.

# 2	Actions

To 3GPP SA WG2

ACTION: 	CT1 kindly ask SA2 to provide answers to the questions above.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 102	13 - 17 February 2017	Dubrovnik, Croatia, EU

TSG CT WG1 Meeting 103	3- 7 April 2017		Spokane (WS), USA